3GPP TSG-RAN WG3 Meeting #121bis                                                                 R3-235724

Xiamen, China, October 9 – 13, 2023

Agenda item: 		13.1

Source: 		Qualcomm Incorporated (WI Rapporteur)

Title: 	Offline discussion on mIAB

Document for:	Discussion

# 1	Introduction

This document includes a summary of a Rapporteur-initiated offline discussion on mobile IAB.

# 2	Proposals

The following is proposed:

Issue 0: Stage 2: DU migration, NG-based MT migration and network integration

Proposal 0: Allocate CB to include stage-2 of DU migration, network integration, and NG-based MT migration to BL CR including agreements from this meeting.

Issue 1: Issues related to mIAB-node authorization.

Proposal 1a: RAN3 assumes that Xn is always available to pass mIAB-node’s authorization status indicator from MT’s CU to DU’s CU. Stage-2 to capture that the network’ behavior in absence of Xn between the mIAB-MT’s CU and mIAB-DU’s CU is left up to implementation.

Proposal 1b: The authorization status update is passed from the MT’s CU to the DU’s CU via Transport Management Modification Request message.

If agreed, allocate CB for TP to TS 38.423 using R3-235632 (Xiaomi) as baseline.

Proposal 1c: After the MT’s CU has receives the mIAB authorization status set to “non authorized” and all traffic has been released from the MT’s backhaul link, the MT’s CU will release all backhaul resources (including BAP address, TNL address and default BAP reconfiguration).

Proposal 1d: For mIAB-node integration, in case the MT’s CU receives the mIAB authorization status set to “non authorized”, it will not establish backhaul resources ( including BAP address, TNL address and default BAP configuration) for this mIAB-node.

Proposal 1d2: RAN3 to capture mIAB-node authorization procedure as separate section in Stage-2.

Proposal 1e: RAN3 to discuss whether mIAB authorization can be sent by AMF in DL NAS Transport message to align with SA2’s assumption.

Issue 2: Behaviour of DU’s CU in absence of Xn to MT’s CU for TMM and IAB-Resource Coordination

Proposal 2a: RAN3 assumes Xn is always available  so that IAB-related Xn procedures can still be performed between MT’s CU and DU’s CU. Stage-2 to capture that the network’s behavior in absence of Xn is left up to implementation.

Proposal 2b: In case NG-based MT handover is applied, RAN3 assumes that IAB-related Xn procedures can still be performed between MT’s target CU and DU’s CU via Xn.

Issue 3: Passing of information to initiate the TMM- and IAB-Resource Coordination procedures.

Proposal 3a: For the integration of mIAB-MT and mIAB-DU to different CUs and for mIAB-DU migration, RAN3 to support Option B, i.e., the gNB-ID of the MT’s CU and the MT-ID are passed via F1AP to DU’s CU.

Proposal 3b: For MT migration, RAN3 to select between Option A and Option B.

Proposal 3c: For Option B, RAN3 to decide whether to use the XnAP UE ID or the BAP address as the identifier for the MT in the initial TMM message sent by the DU’s CU to the MT’s CU.

Proposal 3d: In case the BAP address is used, the destination XnAP UE ID field in the initial TMM message is set to a [specified] dummy value.

Proposal 3e: The parent cell supporting mobile IAB access should always broadcast gNB-ID-Length in SIB1, and the mIAB-MT should be able to decode this information.

Issue 4: Passing of mIAB-MT’s ULI to mIAB-DU’s CU

Proposal 4a: The MT’s ULI (cell ID + TAC) is passed to the DU’s CU via F1 signaling.

If agreed, allocate CB for TP to BL CR 38.473 using R3-235204 (ZTE) as a baseline.

Issue 5: Selection of mIAB-DU’s CU during mobile IAB integration

Proposal 5a: Turn the following WA to an agreement: “The mIAB-DU and mIAB-MT can integrate at different CUs. For this purpose, OAM can be used to configure the mIAB-DU with: a) the donor CU to connect to, and b) the parameters used by the mIAB-DU to establish TNL associations, IPSec tunnels and F1 connectivity to this donor CU.”

Proposal 5b: RAN3 to decide whether the MT’s CU can also configure the mIAB-DU with: a) the donor CU to connect to, and b) the parameters used by the mIAB-DU to establish TNL associations, IPSec tunnels and F1 connectivity to this donor CU.

Proposal 5c: In case RAN3 decides that the MT’s CU can configure this information, RAN3 to send an LS to RAN2 to support this configuration via RRC.

Issue 6: OAM connectivity issues

Proposal 6: Add statement to stage-2 that continuity of OAM connectivity of mobile IAB-node needs to be ensured and liaise SA5 accordingly.

If agreed, allocate CB for TP to 38.401 using R3-235480 as the baseline.

Issue 7: Overlapping MT migration and DU migration

Proposal 7: When MT migration is conducted during DU migration, the source-DU’s CU and the target-DU’s CU need to be informed about the gNB-ID of the MT’s target CU and the MT-ID selected by the MT’s target CU.

Issue 8: Passing of mIAB indicator or mIAB authorization status in HO Request?

Proposal 8a: RAN3 to decide whether the mIAB authorization status is passed in XnAP HO Request.

Proposal 8b: HO request for an mIAB-node should contain an indication related to mobile IAB [so that the target CU can reject the HO request in case it does not support mIAB, e.g., it does not recognize this indication].

Proposal 8c: In case the mIAB authorization status is passed in XnAP HO request, there is no need for an additional mIAB indication in this XnAP HO Request.

Issue 9: NCI reconfiguration by DU’s CU

Proposal 9a: Add statement to stage-2 of DU migration that OAM is updated with cell-local ID part of NCI when reconfigured by DU’s CU, and that the DU’s CU obtains a list of NCI  from OAM in advance.

If agreed, allocate CB for TP to BL CR to 38.473 for NCGI reconfiguration using R3-235417 as baseline.

Proposal 9b: Send a reply LS to SA5 to inform that CU-based NCI reconfiguration and the corresponding statements has been added to the RAN3 specification as requested by SA5.

If agreed, allocate CB for reply LS.

Issue 10: RAN2 LS on RACH-less handover

Proposal 10: RAN3 to discuss if there are any issues with RACH-less handover from RAN3 perspective.

Issue 11: SA2 LS on reestablishment of F1 after mIAB de-authorization.

Proposal 11: RAN3 to reply to SA2 on Q1 and Q2 according to R3-235301, Annex 3: “

On Q1: RAN3’s confirms SA2’s understanding.

On Q2: RAN3 does not support a RAN-based mechanism that prevents the mIAB-DU from re-attempting F1 Setup after F1-release due to the mobile IAB node’s authorization status change to “non-authorized”. Nevertheless, RAN3 agrees that the IAB-donor will not allocate backhaul resources to the mobile IAB node if it is “not-authorized”. In that respect, RAN3 assumes that not providing backhaul resources to the mobile IAB node, or the NAS-based signaling to the mobile IAB node provided in TS 23.501 section 5.35A.4, v.18.3.0 is sufficient to prevent the mobile IAB-DU attempting to reestablish F1 in case it is “non-authorized”.”

Issue 12: SA2 Reply LS on FS_VMR solutions review.

Proposal 12a: Agree R3-235418 and R3-235419 for SA2’s requirement on support UE positioning when the UE is served by MBSR.

Proposal 12b: If P12a is agreed, RAN3 to reply to SA2 according to R3-235301, Annex 1: “

As indicated in RAN3’s reply LS in R3-231011, RAN3 has discussed point#6 raised by SA2 and converged on a solution.

SA2 question on point#6:

- For point#6 (regarding KI#5), based on the SA2 study, NRPPa triggered procedure for the LMF to obtain MBSR location information i.e., location and velocity at a specific scheduled time could be a good alternative to the GMLC based MT-LR solution. Additionally, SA2 would also like to allow the LMF to obtain the UE ID of the MBSR via NRPPa from the donor gNB. SA2 would like to ask RAN3 to consider supporting such solution within Rel-18 timeframe.

RAN3 answer on point#6:

- For point#6 (regarding KI#5): RAN3 has agreed the solution to support UE positioning when the UE is served by MBSR and agreed on the corresponding CRs to TS 38.305/38.455/38.473 (R3-233517/R3-233518/R3-233519), and the TPs to TS 38.455/38.473 (R3-235418 and R3-235419).”

Issue 13: CU- vs. OAM-based selection of DU’s target CU

Proposal 13: RAN3 to discuss how source DU’s CU and OAM coordinate, which of them decides on the information of target DU’s CU for DU migration.

# 2	Discussion

## Issue 0: Stage 2: DU migration, NG-based MT migration and network integration

Proposal 0-1: Allocate CB to include stage-2 of DU migration, network integration, and to BL CR including agreements from this meeting.

[MOD]:

We do not need to capture NG handover. We need to capture how F1 is migrated during NG-based F1 migration. This is in RAN3 scope. Therefore, we will include this into RAN3 Stage.2

Proposal 0: Allocate CB to include stage-2 of DU migration, network integration, and NG-based MT migration to BL CR including agreements from this meeting.

## Issue 1: Issues related to mIAB-node authorization.

Proposal 1a: RAN3 assumes that Xn is always available to pass the mIAB-MT’s authorization status indicator from MT’s CU to DU’s CU. Stage-2 to capture that the network’ behavior in absence of Xn between the mIAB-MT’s CU and mIAB-DU’s CU is left up to implementation.

[MOD]: Agree with second change. First change needs to be reworded.

Proposal 1a: RAN3 assumes that Xn is always available to pass mIAB-node’s authorization status indicator from MT’s CU to DU’s CU. Stage-2 to capture that the network’ behavior in absence of Xn between the mIAB-MT’s CU and mIAB-DU’s CU is left up to implementation.

Proposal 1b: The “non-authorized” status update is passed from the MT’s CU to the DU’s CU via Transport Management Modification Request message.

[Xiaomi] we have a TP for this in R3-235632, can we work on the TP this meeting?

CATT: First, there is no need to indicate the “authorized” status to Du’s CU for the Du’s Cu to admit a F1 setup request from mIAB-node, since the mIAB-node will not initiate F1 setup unless it is authorized by NAS.

Second, when the mIAB node is not authorized there is no Du’s CU because the F1 does not exist at all. If the non-authorized mIAB-MT has migrated across multiple CUs, the current MT’s CU has no idea where the DU’s CU is.

[Ericsson]: The original wording “authorization status” is correct. We understand CATT’s reasoning, but, F1 release and handover of served UEs takes time. During this time, MT’s CU shall note remove the BH resources yet. During this time it may happen that the node becomes authorized again, and the MT’s CU should be able to indicate the “authorized” status to the DU’s CU, so that the DU’s CU can take appropriate action (e.g. stop with the process of “kicking out the mIAB-DU out of the network”).

[MITRE]: Agree with Ericsson.

[MOD]: Let’s keep original P1b.

Proposal 1b: The authorization status update is passed from the MT’s CU to the DU’s CU via Transport Management Modification Request message.

If agreed, allocate CB for TP to TS 38.423 using R3-235632 (Xiaomi) as baseline.

Proposal 1c: In case the MT’s CU receives the mIAB authorization status set to “non authorized”, it will release all backhaul resources after the DU’s CU has released all traffic as requested by the MT’s CU.

[ZTE] We would like to clarify that all backhaul resources includes BAP address, TNL address and default BAP configuration to make it clear.

And we think the MT’s CU can send Transport Management Modification Request message including authorization status and Traffic To Be Released Information IE. And then DU’s CU can confirm that the traffic has been released by sending RESPONSE message including Traffic Released List IE. So it’s not appropriate to use “DU’CU has requested the release”. So we suggest the following rewording:

Proposal 1c: In case the MT’s CU receives the mIAB authorization status set to “non authorized”, it will release all backhaul resources ( including BAP address, TNL address and default BAP configuration)  after all traffic has been released.

[Ericsson]: Additional intervention on top of ZTE’s version:

Proposal 1c: In case the MT’s CU receives the mIAB authorization status set to “non authorized”, it will release all backhaul resources ( including BAP address, TNL address and default BAP configuration)  after all the UEs served by the mIAB-DU have been handed over elsewhere, the F1 interface between mIAB-DU and its CU has been removed and all traffic has been released.

[MOD]: The MT’s CU does not know if UEs have been handed over or F1 has been released. It only knows that the traffic has been released:

Proposal 1c: After the MT’s CU has receives the mIAB authorization status set to “non authorized” and all traffic has been released from the MT’s backhaul link, the MT’s CU will release all backhaul resources (including BAP address, TNL address and default BAP reconfiguration).

Proposal 1d: For mIAB-node integration, in case the MT’s CU receives the mIAB authorization status set to “non authorized”, it will not establish backhaul resources for this mIAB-node.

[ZTE] We would like to clarify that all backhaul resources includes BAP address, TNL address and default BAP configuration to make it clear. So we suggest the following rewording:

Proposal 1d: For mIAB-node integration, in case the MT’s CU receives the mIAB authorization status set to “non authorized”, it will not establish backhaul resources ( including BAP address, TNL address and default BAP configuration) for this mIAB-node.

[MOD]: Fine with me:

Proposal 1d: For mIAB-node integration, in case the MT’s CU receives the mIAB authorization status set to “non authorized”, it will not establish backhaul resources ( including BAP address, TNL address and default BAP configuration) for this mIAB-node.

We should also capture all of this as a separate section in Stage-2:

Proposal 1d2: RAN3 to capture mIAB-node authorization procedure as separate section in Stage-2.

Proposal 1e: RAN3 to discuss whether mIAB authorization can be sent by AMF in DL NAS Transport message to align with SA2’s assumption.

[ZTE] We prefer to try if we could achieve agreement this meeting considering that there is no technical concern on this issue so far as I know. So we suggest the following:

Proposal 1e: RAN3 to decide whether mIAB authorization can be sent by AMF in DL NAS Transport message to align with SA2’s assumption.

[MOD]: No time for this. Let’s keep it for next meeting.

Proposal 1e: RAN3 to discuss whether mIAB authorization can be sent by AMF in DL NAS Transport message to align with SA2’s assumption.

## Issue 2: Behaviour of DU’s CU in absence of Xn to MT’s CU for TMM and IAB-Resource Coordination

Proposal 2a: RAN3 assumes Xn is always available  so that IAB-related Xn procedures can still be performed between MT’s CU and DU’s CU. Stage-2 to capture that the network’s behavior in absence of Xn is left up to implementation.

[MOD]: Fine

Proposal 2a: RAN3 assumes Xn is always available  so that IAB-related Xn procedures can still be performed between MT’s CU and DU’s CU. Stage-2 to capture that the network’s behavior in absence of Xn is left up to implementation.

Proposal 2b: In case NG-based MT handover is applied, RAN3 assumes that IAB-related Xn procedures can still be performed between MT’s target CU and DU’s CU via Xn.

[MITRE]: Agree with Ericsson changes.

[MOD]: Fine

Proposal 2b: In case NG-based MT handover is applied, RAN3 assumes that IAB-related Xn procedures can still be performed between MT’s target CU and DU’s CU via Xn.

## Issue 3: Passing of information to initiate the TMM- and IAB-Resource Coordination procedures.

Proposal 3a: For the integration of mIAB-MT and mIAB-DU to different CUs and for mIAB-DU migration, RAN3 to support Option B, i.e., the gNB-ID of the MT’s CU and the MT-ID are passed via F1AP to DU’s CU.

[MOD]: Fine

Proposal 3a: For the integration of mIAB-MT and mIAB-DU to different CUs and for mIAB-DU migration, RAN3 to support Option B, i.e., the gNB-ID of the MT’s CU and the MT-ID are passed via F1AP to DU’s CU.

Proposal 3b: For MT migration, RAN3 to decides whether to revert previous agreement on option A.

CATT: Prefer original proposal.

Canon: The previous agreement is:  The mIAB-MT’s source donor CU can send the info on the mIAB-MT’s target donor CU to the mIAB-DU’s donor CU after the completion of IAB-MT HO. Actually, Option B is compatible with this agreement as with option B the mIAB-MT’s source donor CU would pass the information to the mIAB-DU’s donor CU, not directly, but through the mobile IAB-node.

As a limitation to option A, we may consider the case where the mIAB-node does not serve any UE. In such case, there would be no TMM procedure so that the mIAB-MT’s source donor CU knows the mIAB-DU’s donor CU. Option B will also work in that case, while option A would require some additional signalling.

[MITRE]: Several contributions were presented based on the previous agreement, To make them moot, we prefer Huawei’s suggestion to clearly capture this as a change of agreement in the meeting notes (if agreed).

[MOD]: We have never agreed to use Xn. We keep the original proposal. We need to SoH anyway.

Proposal 3b: For MT migration, RAN3 to select between Option A and Option B.

Proposal 3c: For Option B, RAN3 to decide whether to use the XnAP UE ID or the BAP address as the identifier for the MT in the initial TMM message sent by the DU’s CU to the MT’s CU.

[MOD]: Keep this one.

Proposal 3c: For Option B, RAN3 to decide whether to use the XnAP UE ID or the BAP address as the identifier for the MT in the initial TMM message sent by the DU’s CU to the MT’s CU.

Proposal 3d: In case the BAP address is used, the destination XnAP UE ID field in the initial TMM message is set to a specified dummy value.

[ZTE] Agree, but prefer to use “Non-F1-Terminating IAB-donor UE XnAP ID” instead of “destination XnAP UE ID” to avoid confusion. So we suggest the following rewording:

Proposal 3d: In case the BAP address is used, the Non-F1-Terminating IAB-donor UE XnAP ID field in the initial TMM message is set to a dummy value.

[Ericsson]: we prefer the original wording. If we go for BAP address, then another problem arises. Namely, in the first TMM run between the two donors, one dummy XnAP UE ID will be used, and in all subsequent run, the correct one will be used (since both donors will learn both XnAP UE IDs). From the MT’s CU’s point of view, it doesn’t know how to distinguish dummy value vs correct value. If there are link breaks, software upgrades or disturbances in the network, it is not easy to know whether a TMM run was the first run vs the subsequent run. If 3GPP can define a fixed dummy value (which should never be used by any real node), that would simplify the issue and it becomes easier to specify too.

[MOD]: Ericsson raises an important point:

Proposal 3d: In case the BAP address is used, the destination XnAP UE ID field in the initial TMM message is set to a specified dummy value.

Proposal 3e: RAN3 to check if there is a UE capability for decoding gNB-ID length contained in SIB1, and/or whether such a UE capability would be needed by mIAB-MT to obtain gNB-ID of MT’s CU.

Proposal 3f: The cell support mobile IAB access should always broadcast gNB-ID-Length in SIB1.

CATT: As there is no specific capability for decoding the gNB-ID length field (it belongs to R17 capability), should the mIAB-MT at least support to R17? Or, the behaviour of reading the gNB-ID length of mIAB-MT implicitly means it should have this capability (nothing to specified)?

[MOD] It seems CATT has checked and verified that there is no specific capability for reading gNB-ID length. This was introduced in Rel-17. The mobile IAB-MT is a Rel-18 device, so it should be able to read this information. We agree with Huawei that P3f is needed. Propose on a new P3e:

Proposal 3e (new): The parent cell supporting mobile IAB access should always broadcast gNB-ID-Length in SIB1, and the mIAB-MT should be able to decode this information.

## Issue 4: Passing of mIAB-MT’s ULI to mIAB-DU’s CU

Proposal 4a: The MT’s ULI (cell ID + TAC) is passed to the DU’s CU via F1 signaling.

[ZTE] Can we work on the stage 3 TP to 38.473 based on the contribution R3-235404? Suggest to reword:

Proposal 4a: The MT’s ULI (cell ID + TAC) is passed to the DU’s CU via F1 signaling. Work on the TP to TS 38.473 based on R3-235404.

[MOD] P4a holds. Let’s include note on TP.

Proposal 4a: The MT’s ULI (cell ID + TAC) is passed to the DU’s CU via F1 signaling.

If agreed, allocate CB for TP to BL CR 38.473 using R3-235204 (ZTE) as a baseline.

## Issue 5: Selection of mIAB-DU’s CU during mobile IAB integration

Proposal 5a: Turn the following WA to an agreement: “The mIAB-DU and mIAB-MT can integrate at different CUs. For this purpose, OAM can be used to configure the mIAB-DU with: a) the donor CU to connect to, and b) the parameters used by the mIAB-DU to establish TNL associations, IPSec tunnels and F1 connectivity to this donor CU.”

[MOD] P5a holds.

Proposal 5b: RAN3 to decide whether the MT’s CU can also configure the mIAB-DU with: a) the donor CU to connect to, and b) the parameters used by the mIAB-DU to establish TNL associations, IPSec tunnels and F1 connectivity to this donor CU.

[MOD] P5b holds.

Proposal 5c: In case RAN3 decides that the MT’s CU can configure this information, RAN3 to send an LS to RAN2 to support this configuration via RRC.

CATT: We think should avoid impacting RRC as much as possible. It’s not good idea to impact RRC spec given the configuration issue can be settled by NW internally.

[Ericsson]: we support these proposals.

[MITRE]: Agree with the proposals.

[MOD] P5c holds.

## Issue 6: OAM connectivity issues

Proposal 6: Add statement to stage-2 that continuity of OAM connectivity of mobile IAB-node needs to be ensured and liaise SA5 accordingly. Use TP for 38.401 in R3-235480 as the baseline.

[MOD] Propose slight rewording of reworded P6:

Proposal 6: Add statement to stage-2 that continuity of OAM connectivity of mobile IAB-node needs to be ensured and liaise SA5 accordingly.

If agreed, allocate CB for TP to 38.401 using R3-235480 as the baseline.

## Issue 7: Overlapping MT migration and DU migration

Proposal 7a: When MT migration is conducted during DU migration, the source-DU’s CU and the target-DU’s CU need to be informed about the gNB-ID of the MT’s target CU and the MT-ID selected by the MT’s target CU.

CATT: nothing violated? Can be an observation?

[MITRE]: This issue needs to be explored further. How do the relevant nodes know there is an overlap happening? The answer is contingent on the option selected for MT migration in Issue 3. If initial DU migration was triggered by OAM based on the previous conditions, does OAM need to be made aware of this overlap?

[MOD] P7a simply captures a condition that needs to be met for overlapping MT migration and DU migration. This always holds.

Proposal 7a: When MT migration is conducted during DU migration, the source-DU’s CU and the target-DU’s CU need to be informed about the gNB-ID of the MT’s target CU and the MT-ID selected by the MT’s target CU.

## Issue 8: Passing of mIAB indicator or mIAB authorization status in HO Request?

Proposal 8a: RAN3 to decide whether the mIAB authorization status is passed in HO Request.

[ZTE] suggest to add “XnAP” before “HO Request” since we already agreed to include mIAB authorization status in NGAP HO Request. So we suggest the following rewording:

Proposal 8a: RAN3 to decide whether the mIAB authorization status is passed in XnAP HO Request.

[MOD] Fine:

Proposal 8a: RAN3 to decide whether the mIAB authorization status is passed in XnAP HO Request.

Proposal 8b: HO request for an mIAB-node should contain an indication related to mobile IAB [so that the target CU can reject the HO request in case it does not support mIAB, e.g., it does not recognize this indication].

[ZTE] suggest to remove the latter part since RAN2 agreed that mobile IAB node can access or connect to a R16/17 IAB donor which doesn’t support mIAB. Similarly it needs to be discussed whether mobile IAB node can be handed over to R16/17 IAB donor. So we suggest the following rewording:

Proposal 8b: HO request for an mIAB-node should contain an indication related to mobile IAB .

CATT: The last sentence is needed. If the gNB does not recognize this indication, it’s definite that this gNB does not support mIAB, but not sure vice versa.

Update the proposal as above.

[MOD] Yesterday, RAN2 agreed:

From R2 perspective It is not supported that Rel-18 mobile IAB-node concurrently operate as a Rel-16/17 IAB-node, as e.g. mobile-IAB doesn’t support child IAB nodes.

This means that there are restrictions for the network in configuring concurrent use of R-18 mIAB feature(s) and rel-16/17 IAB features (details FFS).

FFS if an IAB-node may send both MSG5 indications to the network, and the network decides (or if the IAB-node should decide).

The reworded version by CATT is fine.

Proposal 8b: HO request for an mIAB-node should contain an indication related to mobile IAB [so that the target CU can reject the HO request in case it does not support mIAB, e.g., it does not recognize this indication].

Proposal 8c: In case the mIAB authorization status is passed in XnAP HO request, there is no need for an additional mIAB indication in this XnAP HO Request.

[MOD] P8c holds

## Issue 9: NCI reconfiguration by DU’s CU

Proposal 9a: Add statement to stage-2 of DU migration that OAM is updated with cell-local ID part of NCI when reconfigured by DU’s CU, and that the DU’s CU obtains the cell-local ID from OAM.

Proposal 9b: RAN3 work on F1AP TP for the NCGI reconfiguration, R3-235417 can be taken as baseline.

[ZTE] agree, suggest the following rewording to be aligned with SA5’s reply LS. And we suggest to work on the stage 2 TP based on the annex B in R3-235402.

Proposal 9a: Add statement to stage-2 of DU migration that OAM is updated with cell-local ID part of NCI when reconfigured by DU’s CU, and that the DU’s CU obtains a list of NCI  from OAM in advance. Annex in R3-235481 can be taken as baseline.

Besides, we need to send a reply LS to SA5 to inform that CU-based NCI reconfiguration has been added to the RAN3 specification as requested by SA5 in the reply LS as copied in the below. Annex C in the contribution R3-235402 can be used as a baseline. So we suggest the add the following proposal:

[MOD] Fine with ZTE’s rewording. Companies are welcome to fight about TPs during online session.

Proposal 9a: Add statement to stage-2 of DU migration that OAM is updated with cell-local ID part of NCI when reconfigured by DU’s CU, and that the DU’s CU obtains a list of NCI  from OAM in advance.

If agreed, allocate CB for TP to BL CR to 38.473 for NCGI reconfiguration using R3-235417 as baseline.

Proposal 9b: Send a reply LS to SA5 to inform that CU-based NCI reconfiguration and the corresponding statements has been added to the RAN3 specification as requested by SA5.

R3-235034 LS reply to LS on Cell ID (re)configuration for mobile IAB cell SA5 would like RAN3 to also add the following statements in the RAN3 relevant specification: 1) In order to be able to perform CU-based NCI reconfiguration, the donor CU obtains a list of NCI from its OAM in advance. 2) The value change of cellLocalId shall be notified to the OAM system following the NCI reconfiguration. SA5 would like RAN3 to inform SA5 when CU-based NCI reconfiguration is added to the RAN3 specification for SA5 to plan the work.

[MOD] We keep P9b and add CB for reply LS.

Proposal 9b: Send a reply LS to SA5 to inform that CU-based NCI reconfiguration and the corresponding statements has been added to the RAN3 specification as requested by SA5.

If agreed, allocate CB for reply LS.

## Issue 10: RAN2 LS on RACH-less handover

Proposal 10: RAN3 to reply to RAN2: “RAN3 presently does not see any issues with RACH-less handover during DU migration. RAN3 will wait for further progress by RAN2.”

Proposal 10a: RAN3 to discuss the following issues for RACH-less handover of UE during mIAB-DU migration:

- Which node (among source CU, target CU, target IAB-DU) decides the RACH-less HO for the UE connected to mobile IAB?

- Which node (among source CU, target CU, target IAB-DU) selects the beam for the UE to perfrom RACH-less HO.

- When will the target IAB-DU send indication about RACH-less HO completion to UE.

CATT: P10a issues are all within RAN2 scope. RAN2 has worked on RACH-less HO for mIAB together with NTN.

[MOD] If RAN3 believes there are open questions, then we need to discuss those BEFORE we discuss the reply LS. We keep the original P10 and discuss during online session.

Proposal 10: RAN3 to discuss if there are any issues with RACH-less handover from RAN3 perspective.

## Issue 11: SA2 LS on reestablishment of F1 after mIAB de-authorization.

Proposal 11: RAN3 to reply to SA2 on Q1 and Q2 according to R3-235301, Annex 3: “

On Q1: RAN3’s confirms SA2’s understanding.

On Q2: RAN3 does not support a RAN-based mechanism that prevents the mIAB-DU from re-attempting F1 Setup after F1-release due to the mobile IAB node’s authorization status change to “non-authorized”. Nevertheless, RAN3 agrees that the IAB-donor will not allocate backhaul resources to the mobile IAB node if it is “not-authorized”. In that respect, RAN3 assumes that not providing backhaul resources to the mobile IAB node, or the NAS-based signaling to the mobile IAB node provided in TS 23.501 section 5.35A.4, v.18.3.0 is sufficient to prevent the mobile IAB-DU attempting to reestablish F1 in case it is “non-authorized”.”

[MOD] Fine with rewording:

Proposal 11: RAN3 to reply to SA2 on Q1 and Q2 according to R3-235301, Annex 3: “

On Q1: RAN3’s confirms SA2’s understanding.

On Q2: RAN3 does not support a RAN-based mechanism that prevents the mIAB-DU from re-attempting F1 Setup after F1-release due to the mobile IAB node’s authorization status change to “non-authorized”. Nevertheless, RAN3 agrees that the IAB-donor will not allocate backhaul resources to the mobile IAB node if it is “not-authorized”. In that respect, RAN3 assumes that not providing backhaul resources to the mobile IAB node, or the NAS-based signaling to the mobile IAB node provided in TS 23.501 section 5.35A.4, v.18.3.0 is sufficient to prevent the mobile IAB-DU attempting to reestablish F1 in case it is “non-authorized”.”

## Issue 12: SA2 Reply LS on FS_VMR solutions review.

Proposal 12: RAN3 to reply to SA2 according to R3-235301, Annex 1: “

As indicated in RAN3’s reply LS in R3-231011, RAN3 has discussed point#6 raised by SA2 and converged on a solution.

SA2 question on point#6:

- For point#6 (regarding KI#5), based on the SA2 study, NRPPa triggered procedure for the LMF to obtain MBSR location information i.e., location and velocity at a specific scheduled time could be a good alternative to the GMLC based MT-LR solution. Additionally, SA2 would also like to allow the LMF to obtain the UE ID of the MBSR via NRPPa from the donor gNB. SA2 would like to ask RAN3 to consider supporting such solution within Rel-18 timeframe.

RAN3 answer on point#6:

- For point#6 (regarding KI#5): RAN3 has agreed the solution to support UE positioning when the UE is served by MBSR and agreed on the corresponding CRs to TS 38.305/38.455/38.473 (R3-233517/R3-233518/R3-233519) and the TPs(R3-235418 and R3-235419).”

Proposal 12b: Agree R3-235418 and R3-235419 for SA2’s requirement on support UE positioning when the UE is served by MBSR.

[Xiaomi] we think the E-CID measurement also needs to be enhanced to support mobile TRP. The corresponding TPs in R3-235418 and R3-235419 should be discussed before we send the LS.

[ZTE] We suggest to add RAN3 answer on point#7 as well.

During RAN3#119 meeting, the reply LS on R18 VMR solutions was received from SA2 (R3-230032). The point#6 and point#7 in the reply LS are copied in the below.

- For point#6 (regarding KI#5), based on the SA2 study, NRPPa triggered procedure for the LMF to obtain MBSR location information i.e., location and velocity at a specific scheduled time could be a good alternative to the GMLC based MT-LR solution. Additionally, SA2 would also like to allow the LMF to obtain the UE ID of the MBSR via NRPPa from the donor gNB. SA2 would like to ask RAN3 to consider supporting such solution within Rel-18 timeframe. - For point#7 (regarding KI#6), SA2 would like to clarify that additional information besides existing ULI from donor-gNB (as defined in Rel-17) is needed, so that the 5GC can understand that the existing ULI cannot be used directly. This is crucial for the support of services that rely on the cell ID to infer the UE locations, e.g. emergency services. Therefore, SA2 would like to request RAN3 to either confirm the support of additional ULI for the UE serviced by an MBSR or provide an alternative solution.

During RAN3#119 meeting, an reply LS was sent from RAN3 to SA2 in [R3-231011], which is copied in the below

- For point#6 (regarding KI#5): RAN3 needs to conduct further discussions to converge on a solution. RAN3 will inform SA2 for any progress. - For point#7 (regarding KI#6): RAN3 believes that the functionality requested to provide UE location in point#7 can be accommodated within Rel-18.

As we can see, for point 7, RAN3 replied in the last reply LS that RAN3 believes that the functionality requested to provide UE location in point#7 can be accommodated within Rel-18. But RAN3 haven't answered the question asked by SA2 in point#7, i.e. whether to support additional ULI or an alternative solution. So we think we need to include RAN3 answer on point#7 in the reply LS as well.

[MOD] Let’s discuss TPs first and based on outcome we discuss the LS.

Proposal 12a: Agree R3-235418 and R3-235419 for SA2’s requirement on support UE positioning when the UE is served by MBSR.

Proposal 12b: If P12a is agreed, RAN3 to reply to SA2 according to R3-235301, Annex 1: “

As indicated in RAN3’s reply LS in R3-231011, RAN3 has discussed point#6 raised by SA2 and converged on a solution.

SA2 question on point#6:

- For point#6 (regarding KI#5), based on the SA2 study, NRPPa triggered procedure for the LMF to obtain MBSR location information i.e., location and velocity at a specific scheduled time could be a good alternative to the GMLC based MT-LR solution. Additionally, SA2 would also like to allow the LMF to obtain the UE ID of the MBSR via NRPPa from the donor gNB. SA2 would like to ask RAN3 to consider supporting such solution within Rel-18 timeframe.

RAN3 answer on point#6:

- For point#6 (regarding KI#5): RAN3 has agreed the solution to support UE positioning when the UE is served by MBSR and agreed on the corresponding CRs to TS 38.305/38.455/38.473 (R3-233517/R3-233518/R3-233519), and the TPs to TS 38.455/38.473 (R3-235418 and R3-235419).”

## Issue 13: CU- vs. OAM-based selection of DU’s target CU

Proposal 13: RAN3 to discuss how source DU’s CU and OAM coordinate, which of them decides on the information of target DU’s CU for DU migration.

[ZTE] We suggest to add “whether and” before “how” since this issue has not been discussed yet and we are not sure whether the coordination is necessary.

Proposal 13: RAN3 to discuss whether and how source DU’s CU and OAM coordinate, which of them decides on the information of target DU’s CU for DU migration.

[MITRE]: In the context of MT handover during DU migration (Issue 7), does the OAM need to be also aware of the change in IAB transport connection?

[MOD] There are two nodes that can decide on DU migration. They need to coordinate. We need to discuss HOW, not WHETHER.

Proposal 13: RAN3 to discuss how source DU’s CU and OAM coordinate, which of them decides on the information of target DU’s CU for DU migration.