3GPP TSG-RAN WG3 Meeting #121bis	R3-235848

Xiamen, China, 9 – 13 October 2023

Agenda item:	 10.2.1

Source:		Samsung, Ericsson, Qualcomm, Nokia, Nokia Shanghai Bell

Title:	(TP for SON BLCR for 37.340) SON enhancement for SPR

Document for:		Discussion and Decision

1	Introduction

This contribution provides a TP for TS37.340 to capture the agreements at RAN3#121bis meeting.

TP for TS37.340

10.18.B	Successful PSCell Change Report

The objective of Successful PSCell change Report (SPR) is to detect sub-optimal successful PSCell change/CPC or successful PSCell addition/CPA.

For analysis of such sub-optimal successful PSCell change/CPC and successful PSCell addition/CPA, the UE may collect SPR based on the triggers configured by the network, and makes the SPR available to the network as specified in TS 38.331 [4].

For PSCell addition/CPA and PSCell change/CPC (MN or SN initiated), the target SN always decides the T304 trigger for SPR and performs root cause analysis.

For SN-initiated PSCell change/CPC, the source SN decides the T310/T312 triggers for SPR and is responsible for SPR related optimizations e.g., to optimize PSCell change/CPC configuration or associated mobility thresholds or adjust T310/T312 timer values.

For MN-initiated PSCell change/CPC, the MN decides the T310/T312 triggers for SPR. MN may optimize PSCell change/CPC configuration or associated mobility thresholds or both. Source SN may optimize lower layer issues e.g., adjust T310/T312 timer values.

The SPR can be fetched from the UE by the MN while the UE is still connected to the MN, or by a node different from the MN that sent the SPR configuration to the UE. In case the SPR is retrieved in a node different from the MN that sent the SPR configuration to the UE, the SPR is first forwarded to that MN, which then forwards it to the respective SN(s) which should perform the SPR optimization.