3GPP TSG-RAN WG3 Meeting #121	R3-233838

Toulouse, France, Aug. 21st – Aug. 25th, 2023

Agenda Item:	13.4

Source: 	Canon Research Centre France

Title:	PCI Collision Avoidance with Mobile IAB

Document for:	Discussion

# Introduction

In this paper, we discuss the mitigation of interference due to IAB-node mobility, in particular the detection and prediction of PCI collision/confusion. The discussion is based on the following agreements and identified issues from the previous RAN3 meeting reports [1], [2]:

- As baseline, to avoid PCI collision, F1-terminating IAB-donor can reconfigure PCI for the cell of mobile IAB-DU via existing F1AP message. - PCI-change on the IAB-node can be supported via handover of connected UEs between cells using old and new PCI, respectively. - PCI collision can be detected by the F1-terminating IAB-donor of the mobile IAB-node. - How to avoid PCI collision in the scenario with Xn between IAB-DU’s donor and IAB-MT’s donor. - Whether PCI collision between mobile IAB cells can be predicted based on existing UE measurement report.

# Discussion

## PCI conflict detection/prediction

From previous RAN3 agreements, we know that the MT of a mobile IAB-node (mIAB-MT) can be served by a non-F1 donor-CU different from the F1 donor-CU serving the DU (mIAB-DU). Besides, to avoid PCI collision, this is the F1 donor-CU that can reconfigure the PCI for a cell of the mIAB-DU: once a PCI collision is detected or predicted, the F1 donor-CU has to set a new suitable PCI at the mIAB-DU with an appropriate PCI change procedure.

There are still some open issues, especially: how to avoid PCI collision in the scenario with Xn between mIAB-DU’s donor and mIAB-MT’s donor, and whether PCI collision between mobile IAB cells can be predicted based on existing UE measurement report.

When the F1 donor-CU of a mobile IAB-node is different from the non-F1 donor-CU, the F1 donor-CU may be able to receive the necessary information to detect a PCI conflict (i.e. collision or confusion). Indeed, the F1 donor-CU can request to the non-F1 donor-CU and to other CUs around the non-F1 donor-CU, the information related to the cells served by these CUs. Moreover, when the mobile IAB-node serves UEs, the F1 donor-CU receives the measurement reports from these UEs.

However, this may not be sufficient to detect a PCI conflict. First, the F1 donor-CU may not have an accurate knowledge of the IAB topology controlled by the non-F1 donor-CU (e.g. which cells are adjacent to each other). Besides, when the mobile IAB-node does not serve UEs (for instance an empty bus or train), the F1 donor-CU does not receive any measurement report. Last but not least, in case another mobile cell from another mobile IAB-node controlled by a different F1 donor-CU is present in the IAB topology controlled by the non-F1 donor-CU, the F1 donor-CU may detect a PCI conflict between two mobile cells from UE measurement reports (at the condition there are UEs able to send measurement report despite the PCI conflict), but the F1 donor-CU has no clue to predict a PCI conflict.

Observation 1: a F1 donor-CU of a mobile IAB-node may be able to detect PCI conflict between mobile cells from UE measurement reports, but the F1 donor-CU cannot predict this PCI conflict.

To predict a PCI conflict, the F1 donor-CU requires additional information, which may be provided by the non-F1 donor-CU. However, the case where both F1 donor-CUs decide to change the PCI for their respective mobile cell at the same time has also to be considered. As a consequence, the non-F1 donor-CU of a mobile IAB-node appears as the most appropriate CU to analyse the situation, to predict a PCI conflict, and to request a F1 donor-CU to change a PCI when necessary. Indeed, the non-F1 donor-CU of a mobile IAB-node has the accurate knowledge of the environment, it also receives the measurement report from the mIAB-MT, and it controls the mIAB-MT handover. Actually, the information that is missing at the non-F1 donor-CU is the PCI(s) used at the mobile IAB-node.

Observation 2: The non-F1 donor-CU of a mobile IAB-node is able to detect and to predict a PCI conflict (collision/confusion) with a cell at the mobile IAB-node as soon as this CU knows the PCI(s) used for the mobile IAB-node’s cell(s).

To provide the used PCI(s) at a mobile IAB-node to the non-F1 donor-CU, it is proposed that at MT handover of the mobile IAB-node, the source non-F1 donor-CU informs the target non-F1 donor-CU, e.g. with the handover request.

Proposal 1: When the non-F1 donor-CU of a mobile IAB-node is different from the F1 donor-CU, the non-F1 donor-CU of a mobile IAB-node is informed about the PCI(s) used at the mobile IAB-node at the MT handover request. This also applies to consecutive MT handovers.

## PCI conflict avoidance

There are different situations to consider for a mobile IAB-node: at network integration, at DU migration, and during normal operation (including after MT handover).

At network integration:

When the mIAB MT and the co-located mIAB-DU integrate to different donor-CUs, the mobile IAB-node is physically in the IAB topology controlled by the non-F1 donor-CU, which may not know which donor-CU is the F1 donor-CU. It should be up to the F1 donor-CU to request the non-F1 donor-CU to provide the PCI(s) to set at the mIAB-DU. This handshake procedure should be executed to be able to complete the F1 setup procedure between the F1 donor-CU and the mIAB-DU.

At DU migration:

New PCI(s) may need to be set for the cell(s) at the target logical DU of the mobile IAB-node. The target F1 donor-CU of the DU migration should request the non-F1 donor-CU to provide the PCI(s) to set at the target logical DU. This handshake procedure should be executed to be able to complete the F1 setup procedure between the target F1 donor-CU and the target logical DU. This handshake procedure may be the same as the one used for the case of network integration.

During normal operation:

When the non-F1 donor-CU detects or predicts a PCI conflict (collision/confusion) at a mobile IAB-node, the non-F1 donor-CU should request the F1 donor-CU to change the PCI(s) used at the mIAB-DU, and the non-F1 donor-CU should provide the new PCI(s) to set. After executing the PCI change procedure, the F1 donor-CU should inform the result to the non-F1 donor-CU.

In case the conflict concerns two mobile cells from two mobile IAB-nodes with two different F1 donor-CUs, the non-F1 donor-CU (common to both mobile IAB-nodes) should request PCI change to only one F1 donor-CU.

Proposal 2: A XnAP procedure is used between the F1 donor-CU and the non-F1 donor-CU of a mobile IAB-node to allow the F1 donor-CU to request new PCI(s) to set for cell(s) at the mobile IAB-node.

Proposal 3: A XnAP procedure is used between the F1 donor-CU and the non-F1 donor-CU of a mobile IAB-node to allow the non-F1 donor-CU to request the F1 donor-CU to set new PCI(s) for cell(s) at the mobile IAB-node.

Proposal 4: A XnAP procedure is used between the F1 donor-CU and the non-F1 donor-CU of a mobile IAB-node to allow the F1 donor-CU to inform the non-F1 donor-CU about the result of a PCI change procedure.

# Conclusion

In this contribution, we have explained the limitations of PCI conflict (collision/confusion) detection and prediction at a F1 donor-CU of a migrated mobile IAB-node. Then, we have proposed some procedures for PCI conflict avoidance, and we have made the following observations and proposals:

Observation 1: a F1 donor-CU of a mobile IAB-node may be able to detect PCI conflict between mobile cells from UE measurement reports, but the F1 donor-CU cannot predict this PCI conflict.

Observation 2: The non-F1 donor-CU of a mobile IAB-node is able to detect and to predict a PCI conflict (collision/confusion) with a cell at the mobile IAB-node as soon as this CU knows the PCI(s) used for the mobile IAB-node’s cell(s).

Proposal 1: When the non-F1 donor-CU of a mobile IAB-node is different from the F1 donor-CU, the non-F1 donor-CU of a mobile IAB-node is informed about the PCI(s) used at the mobile IAB-node at the MT handover request. This also applies to consecutive MT handovers.

Proposal 2: A XnAP procedure is used between the F1 donor-CU and the non-F1 donor-CU of a mobile IAB-node to allow the F1 donor-CU to request new PCI(s) to set for cell(s) at the mobile IAB-node.

Proposal 3: A XnAP procedure is used between the F1 donor-CU and the non-F1 donor-CU of a mobile IAB-node to allow the non-F1 donor-CU to request the F1 donor-CU to set new PCI(s) for cell(s) at the mobile IAB-node.

Proposal 4: A XnAP procedure is used between the F1 donor-CU and the non-F1 donor-CU of a mobile IAB-node to allow the F1 donor-CU to inform the non-F1 donor-CU about the result of a PCI change procedure.

# References

[1] Chairman notes of RAN3#117bis-e meeting

[2] Chairman notes of RAN3#119bis-e meeting