3GPP TSG-RAN WG3 Meeting #121                                                      	R3-233923

Toulouse, Aug 21st – Aug 25th, 2023

Agenda item:	10.2.2

Source:	Qualcomm Incorporated

Title:	MRO enhancements for CPAC, voice fall back and fast MCG recovery

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss MRO related enhancements i.e., optimizing CPAC, voice fall back, and fast MCG recovery based on agreements and open issues identified last meeting.

# Discussion

  - MRO for CPA/CPC

RAN3 agreed in a previous meeting to reuse the Rel-17 signaling mechanism as seen below:

Reusing R17 signaling mechanism to report CPA/CPC failure related information over Xn from MN to source SN or last serving SN

FFS whether CPA/CPC candidate PSCell list can be sent from MN to the source/last serving SN

In each of the defined CPA/CPC failure types (Too Late CPC Execution/Too Early CPC Execution/CPC Execution to wrong PSCell/CPA Execution to wrong PSCell/Too Early CPA execution), it is possible that the network did not prepare the right set of candidate PSCells or defined inappropriate execution conditions for CPA/CPC which resulted in an SCG failure. It is therefore important to optimize the set of candidate PSCells and execution conditions during CPA/CPC.

Observation 1: The objective of MRO for CPA/CPC is to optimize the set of candidate PSCells and execution conditions during CPA/CPC

Further as the MN is aware of most information during CPA/CPC, MN can inform some useful information (e.g., CPA/CPC Execution conditions that were configured, CPA/CPC candidate cell list that were configured, CPA/CPC related timer information) to last serving SN or source SN for MRO analysis.

Proposal 1: MN should inform the following CPA/CPC related information to source SN or last serving SN for MRO analysis for CPA/CPC in SCG FAILURE INFORMATION REPORT over Xn:

- CPA/CPC Execution conditions that were configured

- CPA/CPC candidate cell list that were configured

- SCG failure type (classic PSCell change/addition vs. CPA/CPC)

- CPA/CPC related timer information (e.g., time since CPA/CPC execution till failure)

  - MRO for voice fall back

The following is the list of cases for MRO for voice fall back which are captured in R3-226004

Case 1: after failure (HOF/RLF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, a suitable E-UTRA cell is selected, and the UE tries RRC connection setup procedure for the voice service in the E-UTRA cell.

Case 2: after failure (HOF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, no suitable E-UTRAN cell can be selected, the UE reverts back to the configuration of the source PCell and initiates RRC re-establishment procedure in NR.

Consider the RLF case of case 1 as depicted in Figure 1, where UE collects an LTE RLF Report if it encounters an RLF shortly after a successful inter-system HO from NG-RAN1  E-UTRAN2 for voice fall back.

After the target E-UTRAN2 receives the FAILURE INDICATION (along with the LTE RLF Report) from a reconnected E-UTRAN3 or NG-RAN3, the target E-UTRAN2 performs root cause analysis and should identify the HO Failure type as “Inter-system mobility failure during voice fall back”. The Target E-UTRAN2 should then send the INTER-SYSTEM HO REPORT to source NG-RAN1 so that it can know the failure type.

Also, the current definition of too early inter-system HO is only for the direction between LTE  NR and doesn’t cover the direction from NR  LTE. So that failure type can’t be reused.

Observation 2: Current definition of too early inter-system HO is only for the direction from LTE  NR and doesn’t cover the direction from NR LTE

The following was agreed in R3#120:

WA: Define a new handover report type in the Inter-system HO Report over S1 and NG

We therefore have the following proposal:

Proposal 2: Convert the WA into agreement “Define a new handover report type in the Inter-system HO Report over S1 and NG”. The new Handover Report Type can be called “Inter-system Mobility Failure for Voice fall back” and can further include source NR cell ID, failed LTE cell ID and UE RLF Report container

  - MRO for Fast MCG Recovery

Case a, b, c and f1 are already agreed

- Case a: SCG fails when the UE is undergoing fast MCG recovery (i.e., SCG failure happens while T316 is running).

- Case b: the signaling delay is longer than the time the UE waits for the response (T316 expired).

- Case c: Fast recovery near failure case, i.e., UE receives the response message from MN via SN while T316 is running which almost expires but not yet.

- Case f1: SCG fails or is deactivated before the UE sends the MCGFailureInformation

Agree to define case c in R18. Solution can be further discussed.

FFS whether the network-based solution or UE based solution should be used for Case c

FFS whether the MRO for fast MCG recovery needs to support pre-R18 UEs

RAN3 is yet to decide whether to use a network-based solution or UE-based solution to support case c i.e., the near failure case of fast MCG recovery.

These are the sequence of steps during a successful fast MCG recovery:

- T1: Time when UE sent MCGFailureInformation (T316 starts at UE)

- T2: Time when gNB received MCGFailureInformation

- T3: Time when response message is sent from MN for fast MCG recovery (RRCRelease or HO command)

- T4: Time when response message is received at UE (T316 stopped at UE)

We compare the network-based solution vs. UE-based solution below:

| Network based solution | UE based solution |
| --- | --- |
| MN calculates the T316 elapsed value (e.g., by calculating the time between receiving MCGFailureInformation from UE and sending the response message i.e., T3-T2) | UE calculates the T316 elapsed value (i.e., T4 -T1) |
| Cons: T316 elapsed value calculation might be less accurate if the RTT takes longer than usual e.g., when SCG is also having DL/UL issues | Cons: Can’t be used for pre-Rel-18 UEs Has RAN2 impact |
| Pros: Can be used for pre-Rel-18 UEs as well No RAN2 impact | Pros: T316 elapsed value would be accurate |

Observation 3: To support case c, the following options are possible:

- Option 1 (Network based solution): MN calculates the T316 elapsed value (e.g., by calculating the time between receiving MCGFailureInformation from UE and sending the response message)

- Option 2 (UE based solution): UE calculates the T316 elapsed value and reports to gNB. It is upto RAN2 which message to use.

Observation 4: Network based solution (while avoiding UE/RAN2 impacts) also can be used to support pre-Rel-18 UEs.

Proposal 3: RAN3 to down select between the network-based solution and UE-based solution to support case c)

# Conclusion

MRO for CPAC

Observation 1: The objective of MRO for CPA/CPC is to optimize the set of candidate PSCells and execution conditions during CPA/CPC

Proposal 1: MN should inform the following CPA/CPC related information to source SN or last serving SN for MRO analysis for CPA/CPC in SCG FAILURE INFORMATION REPORT over Xn:

- CPA/CPC Execution conditions that were configured

- CPA/CPC candidate cell list that were configured

- SCG failure type (classic PSCell change/addition vs. CPA/CPC)

- CPA/CPC related timer information (e.g., time since CPA/CPC execution till failure)

MRO for voice fall back

Observation 2: Current definition of too early inter-system HO is only for the direction from LTE  NR and doesn’t cover the direction from NR LTE

Proposal 2: Convert the WA into agreement “Define a new handover report type in the Inter-system HO Report over S1 and NG”. The new Handover Report Type can be called “Inter-system Mobility Failure for Voice fall back” and can further include source NR cell ID, failed LTE cell ID and UE RLF Report container

MRO for Fast MCG Recovery

Observation 3: To support case c, the following options are possible:

- Option 1 (Network based solution): MN calculates the T316 elapsed value (e.g., by calculating the time between receiving MCGFailureInformation from UE and sending the response message)

- Option 2 (UE based solution): UE calculates the T316 elapsed value and reports to gNB. It is up to RAN2 which message to use.

Observation 4: Network based solution (while avoiding UE/RAN2 impacts) also can be used to support pre-Rel-18 UEs.

Proposal 3: RAN3 to down select between the network-based solution and UE-based solution to support case c)

# 4. References