3GPP TSG-RAN WG3 Meeting #121	R3-233931

Toulouse, France, 21 – 25 August 2023

Agenda item:	 10.2.1

Source:		Samsung

Title:	(TP for SON BLCR for 38.423 and 38.300) SON enhancement for SHR

Document for:		Discussion and Decision

1	Introduction

This contribution discussed on how to support SHR based on the agreements so far in RAN3 and RAN2.

2	Discussion

2.1 Agreements achieved in RAN3 and RAN2

The agreements achieved in RAN3 so far:

RAN3#117-e:

SHR for intra-system inter-RAT, HO from NR to LTE will be treated first

RAN3#117bis-e:

T310 and T312 related triggers are to be considered for inter-RAT SHR from NR to LTE.

RAN3 thinks that at least the following parameters can be useful for optimizing inter-RAT successful handover from NR to LTE. LS RAN2 to confirm and request support. Whether the existing IEs defined in Rel-17 for intra-NR SHR can be reused is up to RAN2 decision.

- Source NR cell information

- Target LTE cell information

- Measurement results for source, target and neighbours

- Cause to indicate which inter-RAT SHR triggering condition was met

- UE location Information

RAN3 thinks that all CHO related information in intra-NR SHR (e.g., time from CHO configuration to execution) is not applicable for inter-RAT SHR.

RAN3#118:

WA: Support inter-RAT SHR from LTE to NR at least for T304 if no impact on LTE.

RAN3#119:

RAN3 sees benefits to support inter-RAT SHR from LTE to NR for T304 trigger with no impact on LTE in Rel-18.

Take Option 3 (The receiving node forwards the inter-RAT SHR to corresponding node which generates the SHR trigger condition that triggers the inter-RAT SHR) as baseline for SHR forwarding mechanism in Rel-18.

WA: The content of inter-RAT SHR from LTE to NR includes at least Source LTE cell, Target NR cell .

RAN3#119bis-e:

If a different NR node (different from source NR node) retrieves the SHR collected during an inter-RAT HO (NR to LTE), reuse ACCESS AND MOBILITY INDICATION message (over XnAP and F1AP) and Uplink/Downlink RAN configuration transfer procedures (over NGAP) to forward the SHR to the source NR node

No further discussion in RAN3 on above RACH related information.

Support collection of SHR during successful inter-RAT HO (LTE to NR) for T304 trigger without any LTE impacts in R18, if the following principles are used. Send LS to RAN2 to confirm the first 4 bullets:

- Target gNB can send SHR configuration (T304 trigger) to UE via NR container (targetRAT-MessageContainer) in MobilityFromEUTRACommand

- UE stores this SHR configuration in NR format

- If T304 trigger is met, UE records SHR in NR format

- UE reports this SHR to only an gNB (either the target gNB or another gNB)

- gNB retrieving this SHR can forward this SHR to the target gNB for SHR optimizations

The SHR collected during inter-RAT HO (LTE to NR) should include at least Source LTE cell and Target NR cell (assuming RAN2 confirms no LTE impacts based on the principles in Proposal 4)

WA: The triggers for SPR should be represented in terms of percentage values (similar to SHR)

RAN2 has the following agreements at RAN2#120 meeting:

  - RAN2 confirmed the support for the parameters for inter-RAT SHR from NR to LTE requested in RAN3 LS, when T310 and T312 are triggering conditions.

  - Do not support T304 trigger for inter-RAT SHR from NR to LTE.

RAN2 agreements at RAN2#121 meeting:

1: For Q1 in the LS R2-2211160, RAN2 agrees to reduce/avoid the impact on LTE specification to support inter-RAT SHR.

2: For handover from NR to LTE,UE generates the NR SHR when SHR for inter-RAT mobility is triggered due to T310 or T312 trigger threshold is fulfilled.

3: For HO from NR to LTE, UE records the SHR for inter-RAT mobility in the VarSuccessHO-Report.

4: For inter-RAT SHR, below parameters is stored, reuse the existing IEs defined in Rel-17 for intra-NR SHR:

a.	Source NR cell information

c.	Measurement results for source, target and neighbours

d.	Cause to indicate which inter-RAT SHR triggering condition was met

e.	UE location Information

5:  A new EUTRA target cell CGI is introduced in inter-RAT SHR.

6: For HO from NR to LTE, the T310 and T312 threshold is provided to the UE by source gNB in the otherConfig.

7: For handover from NR to LTE, cross-RAT reporting is not supported, i.e., UE reports the SHR report to the network when it comes back to NR.

8: RAN2 further discuss if below content is needed for inter-RAT SHR when HO from NR to LTE:

a.	C-RNTI (FFS target or source)

c.	FFS: Time between report generating and fetching

RAN3#120:

A LS is sent to RAN2 in R3-233380 including the solutions and questions on UE context retrieval.

RAN2#122:

=>	intra-NR SHR and Inter-RAT SHR from LTE to NR will be deprioritized in RAN2 for R18.

2.2 SHR for intra-system inter-RAT HO from NR to LTE

There are two open issues on SHR for intra-system inter-RAT HO from NR to LTE:

- Mobility Information retrieval

- Correlation of inter-RAT SHR with RLF Report.

For the first issue, the solutions and questions have been sent to RAN2 in R3-233380 at last RAN3#120 meeting. We should wait for the reply from RAN2 for further discussions.

For the correlation of inter-RAT SHR with RLF Report, the following options are on the table:

Option 1: Support the correlation so that the network can discard SHR if it knows that there was RLF shortly after successful HO.

Option 1-1:  the source gNB performs the correlation based on target C-RNTI (no additional reporting from the UE is needed).

- UE may/may not reports a correlation indication to indicate whether there is a RLF shortly after a successful inter-RAT HO from NR to LTE

Option 1-2:  the source gNB performs the correlation based on the source C-RNTI and time information between HO command and SHR retrieval

Option 1-3: UE assistance-based option to support the correlation indication for SHR and RLF based on new flag reported within the SHR

Option 2: Postpone correlation of inter-RAT SHR and RLF to Rel-19.

Option 3: Do not support SHR and RLF Report correlation

Firstly, Correlation is beneficial in the following two aspects:

- In case of both SHR and RLF are recorded, the source node may only need to optimize RLF as it is a more serious issue.

- For the near failure case during successful handover, the reason may be a little late handover from NR to the target LTE cell. For the RLF shortly after successful handover, the reason is wrong inter-RAT cell handover (i.e. too early to the target cell and too late to the suitable cell). To optimsie the two cases may bring opposite directions.

Observation 1: It is beneficial to support the correlation of inter-RAT SHR and RLF.

Therefore, it is proposed to support the correlation of inter-RAT SHR and RLF. Considering this is the only issue for SHR that RAN3 could discuss for the time being, it makes sense to discuss and selects a solution in Rel-18.

Proposal 1: Support the correlation of inter-RAT SHR and RLF in Rel-18.

Among the three solutions (i.e. Option 1-1, Option 1-2 and Option 1-3), it is the source gNB performing the correlation based on a common reference for Option 1-1 and Option 1-2, it is the UE to decide the correlation for Option 1-3.

UE knows a SHR and a RLF are related to itself. But to decide the correlation, the UE should know whether the RLF occurs shortly after successful handover. E.g. if the RLF occurs longer after the handover, then the RLF is not related with the previous handover procedure. To decide this, a timer needs to be configured to the UE as proposed in [2] R3- 232946.

In MRO failure case, to differentiate too late HO from too early/wrong cell HO, it is the base station to decide whether there is recent HO before the failure by comparing whether timeConntionFailure is smaller than the configured threshold Tstore_UE_cntxt. So the function has been there in network side. To define the same function in UE side may complex the UE and also a threshold needs to be configured to the UE. Duplicated function in UE side needs to be defined for UE based solution.

Observation 2: The network has the function to decide whether RLF occurs shortly after successful HO. For UE based solution, duplicated function will be defined in the UE side.

The comparison of the three solutions are given in the table below.

| Options | Option 1-1 | Option 1-2 | Option 1-3 |
| --- | --- | --- | --- |
| Brief description | The source gNB performs the correlation based on target C-RNTI (no additional reporting from the UE is needed) | the source gNB performs the correlation based on the source C-RNTI and time information between HO command and SHR retrieval | UE assistance-based option to support the correlation indication for SHR and RLF based on new flag reported within the SHR |
| RRC signalling impact | No Target C-RNTI is already included in VarSuccessHO-Report and RLF Report. RAN2 agreed that the UE records the SHR for inter-RAT mobility in the VarSuccessHO-Report | Yes The UE report the source C-RNTI and time information between HO Command and SHR retrieval. | Yes RAN configure a Timer threshold to the UE. The UE reports a flag within SHR. |
| Which node decides RLF shortly after successful HO | NG-RAN node The function is already there in the network side | NG-RAN node The function is already there in the network side | UE Duplicated function in the UE as the network side |
| XnAP impact | The target ng-eNB includes the Target C-RNTI in Handover Report message. RLF Report is in LTE RRC format. The source gNB may not understand it. The solve this, the target ng-eNB can include the target C-RNTI in the Handover Report message. | The target ng-eNB includes the timeConnFailure and timeSinceFailure in Handover Report message. RLF Report is in LTE RRC format. The source gNB may not understand it. The solve this, the target ng-eNB can include timeConnFailure and the timeSinceFailure in the Handover Report message. Source C-RNTI is already included in the HO Report message. | ? depending on what’s the network behavior when receiving the flag in SHR. |
| In line with the solution for intra-NR correlation? | Yes | No | No |

From above comparison, it could be observed that Option 1-1 is a better solution comparing with Option 1-2 and Option 1-3. It has the following benefits:

- No impact on RRC signaling

- No duplicated function to decide whether RLF occurs shortly after successful HO.

- Consistent with the correlation mechanism for intra-NR.

Proposal 2: Agree Option 1-1 as way forward for correlation of inter-RAT SHR and RLF Report.

Proposal 3: Include Target C-RNTI in the Handover Report message.

The TP for TS38.423 is provide in Annex 1.

Proposal 4: Agree the TP for TS38.423 in Annex 1.

The signaling flow is shown in the figure below:

Figure 1: Signalling flow for SHR and RLF

If the target C-RNTI received from step 6 and step 7 are equivalent, the source gNB (gNB1) can know the SHR and RLF are correlated.

2.3 SHR for intra-system inter-RAT HO from LTE to NR

RAN3 has sent LS to RAN2 on SHR for intra-system inter-RAT HO from LTE to NR. RAN2 has the following agreement:

intra-NR SHR and Inter-RAT SHR from LTE to NR will be deprioritized in RAN2 for R18.

RAN3 can wait the progress in RAN2 for further discussion.

Proposal 5: Further discussion on SHR for intra-system inter-RAT HO from LTE is pending to RAN2 progress in Rel-18.

2.4 SHR for intra-NR mobility

SHR for intra-NR mobility has been supported in Rel-17. In Rel-17, RAN2 agreed that the target C-RNTI is used for the correlation of SHR and RLF Report (RLF shortly after successful handover. But this part is not described in stage 2.

A stage 2 TP for TS38.300 is provided in Annex 2. Actually, this could also be a Rel-17 CR considering RAN2 has agreed this in Rel-17.

Proposal 6: Agree the change to TS38.300 in Annex 2. It is proposed for RAN3 to decide whether it is a Rel-17 CR or a TP for Rel-18.

For new features in Rel-18, considering RAN2 agreement at last meeting, any further discussion in RAN3 is pending to the progress in RAN2.

Proposal 7: Further discussion on new feature of SHR for intra-NR mobility in Rel-18 is pending to RAN2 progress in Rel-18.

3	Conclusion

This contribution discussed on how to support SHR. We have the following observations and proposals.

Observation 1: It is beneficial to support the correlation of inter-RAT SHR and RLF.

Proposal 1: Support the correlation of inter-RAT SHR and RLF in Rel-18.

Observation 2: The network has the function to decide whether RLF occurs shortly after successful HO. For UE based solution, duplicated function will be defined in the UE side.

Proposal 2: Agree Option 1-1 as way forward for correlation of inter-RAT SHR and RLF Report.

Proposal 3: Include Target C-RNTI in the Handover Report message.

Proposal 4: Agree the TP for TS38.423 in Annex 1.

Proposal 5: Further discussion on SHR for intra-system inter-RAT HO from LTE is pending to RAN2 progress in Rel-18.

Proposal 6: Agree the change to TS38.300 in Annex 2. It is proposed for RAN3 to decide whether it is a Rel-17 CR or a TP for Rel-18.

Proposal 7: Further discussion on new feature of SHR for intra-NR mobility in Rel-18 is pending to RAN2 progress in Rel-18.

Reference

[1] R3-2323335, Summary of Offline Discussion on CB: # SONMDT1_SHRSPR, Samsung (moderator)

[2] R3- 232946, [TP to 38.423, SON] SHR and Configuration coordination for the successful PSCell change report, Nokia, Nokia Shanghai Bell

[3] TS38.423

Annex 1: TP for TS38.423

**************************** The start of change ***************************

#### 8.4.8.2	Successful Operation

Figure 8.4.8.2-1: Handover Report, successful operation

NG-RAN node1 initiates the procedure by sending the HANDOVER REPORT message to NG-RAN node2. When receiving the message NG-RAN node2 shall assume that a mobility-related problem was detected.

If the Handover Report Type IE is set to "HO too early" or "HO to wrong cell", then NG-RAN node1 indicates to NG-RAN node2 that, following a successful handover from a cell of NG-RAN node2 to a cell of NG-RAN node1, a radio link failure occurred and the UE attempted RRC Re-establishment or re-connected either at the original cell of NG-RAN node2 (Handover Too Early), or at another cell (Handover to Wrong Cell). The detection of Handover Too Early and Handover to Wrong Cell events is made according to TS 38.300 [9].

The HANDOVER REPORT message may include:

-	the Mobility Information IE, if the Mobility Information IE was sent for this handover from NG-RAN node2 (in case the NG-RAN node2 provided it more than once, the most recent Mobility Information IE is included in the HANDOVER REPORT message);

-	the Source cell C-RNTI IE.

-	the CHO Configuration IE, if the CHO Configuration IE was sent for this handover from NG-RAN node2.

If received, NG-RAN node2 uses the above information according to TS 38.300 [9].

If the Handover Report Type IE is set to "Inter-system ping-pong", then NG-RAN node2 shall deduce that a completed handover from a cell of NG-RAN node2 to a cell in another system might have resulted in an inter-system ping-pong and the UE was successfully handed over to a cell of NG-RAN node1 (indicated with Target cell CGI IE).

If Target cell C-RNTI IE is received, NG-RAN node2 uses the information as specified in TS 38.300 [9].

Interaction with the Failure Indication procedure:

If NG-RAN node1 receives a UE RLF Report from an NG-RAN node via the FAILURE INDICATION message, as described in TS 38.300 [9], NG-RAN node1 may also include it in the UE RLF Report Container IE included in the HANDOVER REPORT message.

**************************** The next change ***************************

#### 9.1.3.17	HANDOVER REPORT

This message is sent by NG-RAN node1 to NG-RAN node2 to report a handover failure event, or other critical mobility problem.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Handover Report Type | M |  | ENUMERATED (HO too early, HO to wrong cell, Inter-system ping-pong. …) |  | YES | ignore |
| Handover Cause | M |  | Cause 9.2.3.2 | Indicates handover cause employed for handover from NG-RAN node 2 | YES | ignore |
| Source cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source cell for handover procedure (in NG-RAN node 2) | YES | ignore |
| Target cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of target cell for handover procedure (in NG-RAN node 1). If the Handover Report Type is set to "Inter-system ping-pong", it contains the target cell of the inter system handover from the other system to NG-RAN node 1 cell | YES | ignore |
| Re-establishment cell CGI | C- ifHandoverReportType HoToWrongCell |  | Global Cell Identity 9.2.2.73 | CGI of cell where UE attempted re-establishment or where UE successfully re- connected after the failure | YES | ignore |
| Target cell in E-UTRAN | C- ifHandoverReportType Intersystempingpong |  | OCTET STRING | Encoded according to Global Cell ID in the Last Visited E-UTRAN Cell Information IE, as defined in in TS 36.413 [31] | YES | ignore |
| Source cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the source NG-RAN node (in NG-RAN node 2) | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information provided in the HANDOVER REQUEST message or in the SN STATUS TRANSFER message from NG-RAN node 2. | YES | ignore |
| UE RLF Report Container | O |  | 9.2.2.59 | The UE RLF Report Container IE received in the FAILURE INDICATION message. | YES | ignore |
| CHO Configuration | O |  | 9.2.2.76 |  | YES | ignore |
| Target cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the target NG-RAN node and received from the UE in UE RLF Report Container. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifHandoverReportType HoToWrongCell | This IE shall be present if the Handover Report Type IE is set to the value "HO to wrong cell" |
| ifHandoverReportType Intersystempingpong | This IE shall be present if the Handover Report Type IE is set to the value "Inter-system ping-pong" |

****************************End of change ***************************

Annex 2: TP for TS38.300

15.5.2.7	Successful HO

One of the functions of Mobility Robustness Optimization is to detect a sub-optimal successful handover event. The aim is to identify underlying conditions during successful ordinary handovers, successful DAPS handovers, or successful Conditional handovers.

For analysis of successful handover, the UE supports Successful Handover Report based on configuration by network, if received, and makes the Successful Handover Report available to the network as specified in TS 38.331 [12].

The UE stores the Successful Handover Report until the Successful Handover Report is fetched by the network or for 48 hours after the Successful Handover Report is recorded.

For SHR collected during intra-NR handover, when the target NR node fetches the Successful HO Report from UE and the trigger of SHR is T310/T312, it forwards the information to the source node, i.e the node handling the cell reported as source cell in this Successful HO Report, by using the ACCESS AND MOBILITY INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer over NG.

When the NG-RAN Node fetches the Successful HO Report from UE is neither source node nor target node of the handover, it may forward the information to the node(s) which configured the Successful HO Report trigger causing the Successful HO Report to be generated, by using the ACCESS AND MOBILITY INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer over NG.

Upon retrieval of a Successful Handover Report, the receiving node may analyse whether its mobility configuration needs adjustment.

For SHR collected during intra-NR handover, the target NR node performs the correlation between the received SHR report and the RLF Report when the trigger of SHR is T304 and the source NR node performs the correlation between the received RLF Report and SHR report when the trigger of SHR is T310/T312. The target NR node or the source NR node correlates the received SHR report with the RLF Report if the same target C-RNTI is received in the RLF Report and the SHR. The target NR node or the source NR node may ignore the received SHR in case a RLF Report is correlated with the same handover.

The SHR report can be used to detect one case of Intra-system Too Late Handover, namely when DAPS HO is configured but an RLF is detected in the source cell during a successful DAPS HO.