3GPP TSG-RAN WG3 Meeting #121             	   				    R3-233999

Toulouse, France, Aug 21th – 25th, 2023

Agenda Item:		17.2

Source:			China Telecom

Title:				Discussion on NTN Service Continuity Enhancements

Document for:	 	Discussion, Decision

- Introduction

Last RAN3 meeting discussed mobility issue for NR NTN, and some agreements have been reached as below[1]:

Change the WA to the agreement: Uu Cell ID should be used in Xn Setup and Configuration Update procedures.

WA: Do not exchange multiple TACs over Xn for NTN. How to describe which TAC should be used in semantic description?

When time-based trigger condition is used, the source NG-RAN node should consider the time indicated to the UE to decide when start the early data forwarding to the target NG-RAN node.

In this contribution, we will continue to discuss NTN service continuity issues.

- Discussion

  - Time based CHO

For mobility enhancement in NTN, RAN2 has discussed the solution for source cell to broadcast in advance servingCellConfigCommon from the target cell in order to reduce signalling overhead of handover command which is sent using dedicated (per UE) signalling [2]. In this way, the target´s cell servingCellConfigCommon does not need to be included in each RRCReconfiguration message, which may have some specification impacts on inter-gNB coordination, e.g. the target cell’s servingCellConfigCommon might need to be transferred to the source cell using different means than what’s available in the legacy specification.

On the premise that target cell’s servingCellConfigCommon information can be independently broadcasted in the source cell, the source gNB can obtain relevant information in advance via explicit signaling or based on implementation (such as OAM, decoding RRC containers). In our opinion, considering the necessary information related to handover in NTN case (including service cell and neighboring cell information), OAM participation is inevitable. Referring to experience in the discussion of NTN neighbor cell relationship maintenance topic in Rel-17&18, OAM based solution may be simpler compared to signaling based solution.The common part of serving cell config can be bound to the cell relationship maintenance without introducing additional complexity. Therefore, from RAN3 point of view we think it is feasible for the target cell’s servingCellConfigCommon to be independently provided to source gNB for the purpose of inter-gNB handover without specification changes.

Proposal 1: The source gNB can obtain the servingCellConfigCommon of the target cell by implementation.

  - TACs over Xn

RAN3 already has the working assumption of not exchanging multiple TACs over Xn for NTN. Another open issue is How to describe which TAC should be used, as it is mandatory in served cell information NR IE. We noticed that during the Xn Setup and Configuration Update procedure, the List of Served Cells NR to be exchanged between the gNBs is optional, one way is to not exchange the served cells and neighbour relations. The served cell information and its neighbour relations could be left to OAM, and no enhancement is required in this case.

Observation 1: It’s not necessary to exchange the List of Served Cells NR with the neighbour NTN-gNBs.

On the other hand, in the Rel-17 NTN, the agreement was reached that Serving/neighbor NTN cell information, if any, may be exchanged between gNBs via Xn. In this case, we suggest that the TAC IE in the served cell information should be ignored.

Proposal 2: The mandatory TAC IE in the served cell information should be ignored for NTN case.

- Conclusions

In this contribution, we discussed the open issues to support NTN Mobility enhancement in Rel-18 and have the following proposals:

Proposal 1: The source gNB can obtain the servingCellConfigCommon of the target cell by implementation.

Observation 1: It’s not necessary to exchange the List of Served Cells NR with the neighbour NTN-gNBs.

Proposal 2: The mandatory TAC IE in the served cell information should be ignored for NTN case.

- References

[1]	RAN3#120 Meeting report.

[2]	R3-233721, LS on common signalling in (C)HO.