# 3GPP TSG-RAN WG3 Meeting #121	R3-234113

# 21-25 August 2023, Toulouse, France

Agenda Item:	16.3

Source:	CATT

Title:	(TP for SL relay 38.300, 38.413, 38.423 and 38.473) Discussion on Support Service Continuity Enhancements

Document for:	other

Introduction

Based on the hard work from RAN3 Sidelink fans, the most important procedure of inter-gNB-CU switch from direct to indirect path is achieved and only some small open issues are left. In this contribution, we further discuss these open issues and U2U authorization.

Down-select between D4 and D5 in next meeting.

FFS whether move step 7 before step 5 in inter-gNB d2i/i2i path switch procedures.

Discussion

2.1 Packet loss

Agreements:

RAN2 will not specify any Rel-18 enhancement for lossless behaviour for uplink service continuity in L2 U2N relay.

RAN2 will not specify any Rel-18 enhancement from UE perspective for lossless behaviour for downlink service continuity in L2 U2N relay.

RAN2 discussed the lossless data delivery in the inter-gNB i2x cases and they decided not to specify any enhancement in R18 for UL and DL from UE perspective [1]. It means that solution D4 and D5 for DL are on the table from RAN3 perspective. We will further discuss the RAN3 impacts of these two solutions.

-        Solution-D4: Enhanced Data forwarding from source gNB to target gNB per target gNB request (legacy PDCP status report based)

-        Solution-D5: Proactive Data forwarding from source gNB to target gNB

Take below figure as an example to discuss solution D4 and D5. For example, there are one hundred packets from 1-100. Source gNB sends 1-60 packets in source side and receives HARQ ACK from source relay UE RLC. In fact, there are only 1-50 packets are received by remote UE since some packets are buffered in source relay UE. When performing inter-gNB i2x path switch, source gNB will not send packet 51-60 to target gNB but send only 61-100 during data forwarding.

For solution D4, target gNB may identify that the first packet of data forwarding is packet 61 while the PDCP status report shows that UE only successfully receive packet until packet 50. Target gNB can know that packets 51-60 were lost during path switch. And then, target gNB may send an Xn message to source gNB to ask for forwarding packets from 51-60 then target gNB is able to perform data re-transmission from packet 51. It requires additional Xn messages to retrieval “lost” packet which increase Xn interface complexity. Also, PDCP status report is sent to target gNB only after successful handover and the “lost” packet retrieval procedure has to be performed after handover procedure thereby the new Xn procedure introduces additional delay for data transmission. From other side, source gNB has to store some packets even it has been acknowledged by source relay UE.

Observation 1: Solution-D4 requires

- additional Xn messages to retrieval “lost” packet

- data transmission delay

- source gNB has to store packets even it has been acknowledged by source relay UE

For solution D5, it proposes to send “more” packet during data forwarding. The start packet of data forwarding can be decided by source gNB based on implementation e.g., data forwarding starts from packet 40 to packet 100. Based on this, target gNB can perform data re-transmission on target according to PDCP status report. And the “lost” packet forwarding procedure is performed during handover which will not introduce additional delay but it may increase the load of Xn-U.

Observation 2: Solution-D5 does not have stage-3 impact and no additional delay but it will increase the load of Xn-U.

Compared with two options, the increased load in Xn-U in Solution-D5 is acceptable because how many additional packets are forwarded to target depends on the source gNB capability/decision. It is also able to avoid stage-3 impact and additional delay. Therefore, we prefer Solution-D5 to Solution-D4.

Proposal 1: For DL packet loss, RAN3 support Solution-D5.

As long as data forwarding of DL user data packets takes place, the source NG-RAN node shall forward user data in the same forwarding tunnel, i.e.

-	for any QoS flow accepted for data forwarding by the target NG-RAN node and for which a DRB DL forwarding tunnel was established for a DRB to which this QoS flow was mapped at the source NG-RAN node, any fresh packets of this QoS flow shall be forwarded as PDCP SDUs via the mapped DRB DL forwarding tunnel.

-	for DRBs for which preservation of SN status applies, the source NG-RAN node may forward in order to the target NG-RAN node via the DRB DL forwarding tunnel all downlink PDCP SDUs with their SN corresponding to PDCP PDUs which have not been acknowledged by the UE except inter-gNB i2x path switch scenario. In case of inter-gNB i2x path switch, the source NG-RAN may forward downlink PDCP SDUs with their SN corresponding to PDCP PDUs which have been acknowledged by the source relay UE to avoid DL packet loss.  It is up to the source gNB implementation which additional PDCP SDU to select.

Proposal 2: Agree Annex1 for 38.300 to support solution D5.

2.2 Inter-gNB d2i/i2i path switch in TS38.300

For inter-gNB switching from direct/indirect to indirect path, if RRC reconfiguration for target relay UE is performed before step 5, it can increase path switch success rate in case of target relay UE reconfiguration failed. While, RRC reconfiguration for target relay UE performed after step 5 can speed up the path switch procedure. There is no need to introduce strict restriction for when to perform RRC reconfiguration for target relay UE. It can be performed after step 4 at earliest but no later than step 8.

4.	Admission Control may be performed by the target gNB.

5.	The target gNB prepares the path switch and sends the HANDOVER REQUEST ACKNOWLEDGE message to the source gNB.

6.  The source gNB sends the RRCReconfiguration message to the L2 U2N Remote UE.

7.  The target gNB sends the RRCReconfiguration message to L2 U2N Relay UE for relaying configuration. It can be performed after step 4 at the earliest.

Proposal 3: For inter-gNB i2d/d2d path switch, step 7 can be performed between before step 5 at earliest but no later than step 8.

2.3 ProSe Authorization information related to UE-to-UE Relay

In [2], RAN2 agree to introduce authorised information to support U2U Relay i.e., whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-UE Relay/5G ProSe Layer-2 U2U UE. RAN3 shall add such authorised information in "5G ProSe authorised" IE in Xn/F1/NG accordingly.

RAN2’s Feedback The following two bullets should be included:

- whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-UE Relay;

- whether the UE is authorized to act as a 5G ProSe Layer-2 U2U UE.

5G ProSe Authorized

This IE provides information on the authorization status of the UE to use the 5G ProSe services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 5G ProSe Direct Discovery | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Direct Discovery |
| 5G ProSe Direct Communication | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Direct Communication |
| 5G ProSe Layer-2 UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-Network Relay |
| 5G ProSe Layer-3 UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-3 UE-to-Network Relay |
| 5G ProSe Layer-2 Remote UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 Remote UE |
| 5G ProSe Layer-2 UE-to-UE Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-UE Relay |
| 5G ProSe Layer-2 UE-to-UE UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-UE UE |

Proposal 4: RAN3 introduce U2U Relay authorised information in "5G ProSe authorised" IE in Xn/F1/NG accordingly.

Conclusions

Observation 1: Solution-D4 requires

- additional Xn messages to retrieval “lost” packet

- handover delay

- source gNB has to store packets event has been acknowledged by source relay UE.

Observation 2: Solution-D5 does not have stage-3 impact and no additional delay but it will increase the load of Xn-U.

Proposal 1: For DL packet loss, RAN3 support Solution-D5.

Proposal 2: Agree Annex for 38.300 to support solution D5.

Proposal 3: For inter-gNB i2d/d2d path switch, step 7 can be performed between before step 5 at earliest but no later than step 8.

Proposal 4: RAN3 introduce U2U Relay authorised information in "5G ProSe authorised" IE in Xn/F1/NG accordingly.

# 4. Reference

[1] Chair notes, RAN3#120

[2] R3-233716, Reply LS on ProSe Authorization information related to UE-to-UE Relay operation, ZTE

# 5. Annex1: TP for TS38.300 for packet loss

9.2.3.2.3	Data Forwarding

<<<unchanged part is omitted>>>

As long as data forwarding of DL user data packets takes place, the source NG-RAN node shall forward user data in the same forwarding tunnel, i.e.

-	for any QoS flow accepted for data forwarding by the target NG-RAN node and for which a DRB DL forwarding tunnel was established for a DRB to which this QoS flow was mapped at the source NG-RAN node, any fresh packets of this QoS flow shall be forwarded as PDCP SDUs via the mapped DRB DL forwarding tunnel.

-	for DRBs for which preservation of SN status applies, the source NG-RAN node may forward in order to the target NG-RAN node via the DRB DL forwarding tunnel all downlink PDCP SDUs with their SN corresponding to PDCP PDUs which have not been acknowledged by the UE except inter-gNB i2x path switch scenario. In case of inter-gNB i2x path switch, the source NG-RAN may forward downlink PDCP SDUs with their SN corresponding to PDCP PDUs which have been acknowledged by the source relay UE to avoid DL packet loss.  It is up to the source gNB implementation which additional PDCP SDU to forward.

# 6. Annex2: 5G ProSe Authorized

# TS38.413

9.3.1.233	5G ProSe Authorized

This IE provides information on the authorization status of the UE to use the 5G ProSe services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 5G ProSe Direct Discovery | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Direct Discovery |
| 5G ProSe Direct Communication | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Direct Communication |
| 5G ProSe Layer-2 UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-Network Relay |
| 5G ProSe Layer-3 UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-3 UE-to-Network Relay |
| 5G ProSe Layer-2 Remote UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 Remote UE |
| 5G ProSe Layer-2 UE-to-UE Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-UE Relay |
| 5G ProSe Layer-2 UE-to-UE UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-UE UE |

# TS38.423

9.2.3.159	5G ProSe Authorized

This IE provides information on the authorization status of the UE to use the 5G ProSe services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 5G ProSe Direct Discovery | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Direct Discovery. |
| 5G ProSe Direct Communication | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Direct Communication. |
| 5G ProSe Layer-2 UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-Network Relay. |
| 5G ProSe Layer-3 UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-3 UE-to-Network Relay. |
| 5G ProSe Layer-2 Remote UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 Remote UE. |
| 5G ProSe Layer-2 UE-to-UE Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-UE Relay |
| 5G ProSe Layer-2 UE-to-UE UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-UE UE |

# TS38.473

9.3.1.268	5G ProSe Authorized

This IE provides information on the authorization status of the UE for NR ProSe services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 5G ProSe Direct Discovery | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Direct Discovery |
| 5G ProSe Direct Communication | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Direct Communication |
| 5G ProSe Layer-2 UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-Network Relay |
| 5G ProSe Layer-3 UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-3 UE-to-Network Relay |
| 5G ProSe Layer-2 Remote UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 Remote UE |
| 5G ProSe Layer-2 UE-to-UE Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-UE Relay |
| 5G ProSe Layer-2 UE-to-UE UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-UE UE |