3GPP TSG-RAN WG3 Meeting #121	R3-234130

Toulouse, France, 21 – 25 August, 2023

Agenda item:	11.4

Source:	Nokia, Nokia Shanghai Bell

Title:	LS-related discussions

Document for:	Discussion and Decision

# 1	Introduction

This paper provides discussion relative to the following topics:

- Intra NG-RAN inter-RAT QoE following LS [1] from RAN2

- Threshold-based RVQoE reporting, taking into account recent LS [3] from SA4

# 2	Intra NG-RAN inter-RAT QoE

We here aim at replying back to RAN2 on their question relative to support of intra NG-RAN inter-RAT QoE. There is no agreement yet in RAN3, only the following working assumption:

WA: From NR to LTE, the source node adapts to the target node on deciding which one of the QoE configuration to keep.

There is no RAN3 WA or agreement in the other direction (LTE to NR).

The detailed target included in the Rel-18 QoE WID (RP-223488) is:

- Support the continuity of legacy QoE measurement job for streaming and MTSI service during intra-5GC inter-RAT handover process [RAN2, RAN3].

In NG-RAN, a legacy QoE measurement job can be activated only when the UE is served by a gNB. This is clear e.g. from stage 2 (TS 28.405) and from the Rel-17 WID (RP-210913):

•	Specify the support for QoE measurement collection in NR standalone mode. [RAN2, RAN3].

RAN#98 (Dec. 2022) also took the decision that Rel-18 QoE shall not impact E-UTRA RRC (TS 36.331), and at the same occasion also E-UTRAN stage 2 (TS 36.300) was removed as impacted specification in the Rel-18 WID.

The focus of the WID objective is therefore continuity of QoE session initially configured at the gNB. Taking into account that the mandatorily present area scope of a legacy QoE session doesn’t contain E-UTRA cells or E-UTRA(N) TAC, and that QoE activation by the ng-eNG is covered neither by WID nor by stage 2, and presumably would have impact to TS 36.331, one could also deduce that the intention of RAN is that only PLMN-area based QoE sessions may be transferred to a target ng-eNB. However, we suggest that RAN3 discuss and confirm this interpretation of the WID objective.

Proposal 1: RAN3 to discuss and confirm that Rel-18 QoE area scope is not extended to cover E-UTRA cells or E-UTRA(N) TAC.

Next, RAN3 would need to confirm whether QoE measurement continuity applies only for handover from NR to LTE, or whether additionally handover from LTE to NR should be covered. The latter direction would have most sense in case of QoE activation at the ng-eNB side, however it also seems reasonable that a QoE session activated at the gNB (with PLMN-wide area scope) will continue in case of HO to an ng-eNB and then back again to a gNB.

Proposal 2: QoE session continuity to apply in case of handover from NR to LTE and from LTE to NR.

We here detail further how the handover process could be enhanced for support as per proposal 2 above.

NR to LTE: The source gNB selects the QoE sessions to be kept (DASH and MTSI service types are eligible). However the UE needs to be informed about the QoE sessions to be stopped. According to RAN3’s working assumption, the source node decides which of the QoE configurations to keep. This information may be sent to the target ng-eNB during the handover preparation, and there seems to be no XnAP/NGAP impact.

LTE to NR: A single QoE session will be handed over from LTE to NR, so no RAN3 impact would be expected for support of this direction.

RAN3 may therefore confirm feasibility of QoE session continuity in case of handover between NR and LTE (both directions), without RAN3 specification impact.

Proposal 3: RAN3 to provide reply LS to RAN2 confirming feasibility of QoE session continuity in case of handover between NR and LTE (both directions), without RAN3 specification impact.

# 3	Threshold-based RVQoE reporting

With respect to the threshold-based RVQoE reporting it was agreed in RAN3#118 to introduce buffer level as a threshold-based trigger for RVQoE reporting, whereas no threshold-based trigger for reporting playout delay for media startup shall be introduced. SA4 confirmed that application layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18 in their reply LS [2] received by RAN3#119bis-e. Furthermore, in RAN3#120 regarding the threshold-based triggers for RVQoE the following was agreed:

The network can configure UE with two types of events for threshold based RVQoE reporting, the RVQoE reporting is triggered when the buffer level is below a threshold or above a threshold. Whether these two types of events for threshold based RVQoE reporting can be configured simultaneously needs to be further discussed.

As can be seen, the introduction of two thresholds was agreed in RAN3, with an open point whether these two types of threshold-based RVQoE reporting can be simultaneously configured.

However, RAN3 has at this meeting received a second reply LS [3] from SA4 where they “[do] not confirm RAN2 preference that application layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18 based on the corresponding QoE configuration received from the AS layer”.

The second reply LS [3] therefore creates an open point whether Rel-18 UEs will support threshold-based RVQoE reporting, however such decision on UE support needs to be taken by RAN2. It therefore seems needed that RAN3 postpones further discussion on threshold-based RVQoE reporting until RAN2 has taken the corresponding decision on UE support.

Observation: Following second reply LS from SA4, it is not sure whether Rel-18 UEs will support threshold-based RVQoE reporting.

Proposal 4: RAN3 postpones further discussion on threshold-based RVQoE reporting until RAN2 has taken the corresponding decision on UE support.

# 3	Conclusion

We have made the following observations and proposals:

Intra NG-RAN inter-RAT QoE

Proposal 1: RAN3 to discuss and confirm that Rel-18 QoE area scope is not extended to cover E-UTRA cells or E-UTRA(N) TAC.

Proposal 2: QoE session continuity to apply in case of handover from NR to LTE and from LTE to NR.

Proposal 3: RAN3 to provide reply LS to RAN2 confirming feasibility of QoE session continuity in case of handover between NR and LTE (both directions), without RAN3 specification impact.

Threshold-based RVQoE reporting:

Observation: Following second reply LS from SA4, it is not sure whether Rel-18 UEs will support threshold-based RVQoE reporting.

Proposal 4: RAN3 postpones further discussion on threshold-based RVQoE reporting until RAN2 has taken the corresponding decision on UE support.

# References

[1]		R3-231110 – R2-2302020, LS on Continuity of QoE measurements during intra-5GC inter-RAT HO, from RAN2 to RAN3, SA4SA4

[2]		R3-232105 – S4-230684, Reply LS on buffer level threshold-based RVQoE reporting, from SA4 to RAN2, RAN3

[3]		R3-233813 – S4-231119, Reply LS on buffer level threshold-based RVQoE reporting, from SA4 to RAN2 cc RAN3