3GPP TSG-RAN WG3 Meeting #121	R3-234131

Toulouse, France, 21 - 25 August, 2023

Agenda item:	10.2.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on RACH optimization

Document for:	Discussion

# 1	Introduction

1) According to RAN3#120, RAN3 will continue discussing the following options as the additional RA report mechanism to facilitate the network to identify the RA configuration that is related to the RA Report:

Opt1: Signal the feature priority

Opt2: Signal the RACH partition configuration information

Opt3: Signal the time between RACH access that led to the generation of a RACH Report and reporting of the RACH Report to the NG-RAN

Opt4: The network controls the UE to report RA information

Opt5: Mobility information

This paper discusses the different options and proposes our view on the down-selection of options.

2) In RAN3#120, the following open issue was identified for forwarding RA Report from MN to SN:

FFS whether and how to forward the RA Report in case the MN that retrieves the RA Report is not connected to any SN associated to the PSCells received with the RA Report.

This paper also provides the discussion on support of RA report forwarding from MN to SN in (NG)EN-DC or EN-DC scenario.

# 2	Discussion

## 2.1 RACH Partitioning

RA Report as part of SON feature for RACH optimization can be used by the NW to identify e.g. the need of RA configuration. Therefore, the NW should be able to associate the RA report with the corresponding RA configuration. As UE can log maximum 8 RA entries in the RA report, the RA entries in the RA report may not always be related to the latest RA configuration. Especially when RACH partitioning feature is supported, the configuration of RACH partitioning may depend on real-time UE distribution that supports different features in the feature combination of RACH partitioning. Thus RA reconfiguration for different RA partitioning among the features may happen more often than in the earlier release when RA partitioning was not supported. In case of RA reconfiguration triggered in the NW e.g. for RA partitioning, the RA report sent by the UE may be related to the previous RA configuration. To facilitate the NW to identify which RA configuration the RA report from the UE is related to, RAN3 discussed the different options:

Option 1: Include the feature priority

Option 2: Include the RACH partition configuration related information (e.g., start preamble / number of preambles in the RA partition)

Option 3: Include the time between RACH access that led to the generation of a RA Report and reporting of the RA Report

Option 4: The network controls the UE to report RA information

Option5: Mobility information

For opt1, RAN2 has agreed that the feature or feature combination that triggers the RA procedure and the used feature combination will be reported in RA report for RACH partitioning. The feature priority is also configured by the NW, and it therefore seems that feature priority signalled by the UE would be redundant information. Thus opt1 may not be needed.

For opt2, signal the RA partitioning configuration information can be used by the NW to identify which RA partitioning configuration the RA report is related to. The concern is to increase the signaling overhead on RA report by adding the RA partitioning configuration information in the report. If only part of RA partitioning configuration information such as start preamble and/or number of preambles in the RA partition is included in the report, it may not be used to derive the correct RA partitioning configuration by the NW if the reported part of RA partitioning configuration is not re-configured.

For opt3, signal the time between RA access and reporting of corresponding RA report can also be used by the NW to identify the corresponding RA configuration that RA report is related to. The concern is about the timers that UE needs to maintain for each RA entry in the RA report.

For opt4, when NW reconfigures the RA configuration e.g. as the outcome of SON feature for RACH optimization, the NW may not be interested in the RA report that is related to the previous or out-dated RA configuration. Thus NW can control the UE what RA report (e.g. that is related to the latest RA configuration or all RA report regardless which RA configuration) should be reported to the NW. If NW controls to report the RA report only related to the latest RA configuration, it can reduce the signalling overhead in opt2 or reduce the UE complexity on maintaining multiple timers in opt3.

Opt5 was added in RAN3#120 meeting to try to identify the NW configuration or UE context by using an index. The aim is to have a common solution for different SON features, i.e. for retrieval of UE context while performing SHR/SPR optimizations, identifying RACH configuration while optimizing RACH partitions, and also for NR-U to identify LBTFailureRecoveryConfig while optimizing LBT failures and other future optimizations. In practice we believe that RACH configuration, which may be updated independently of UE context, would need to be identified by a different identifier than configuration for MRO – for connected mode mobility the latter is signalled via dedicated RRC signalling and hence dependent on the UE context. We believe that Opt5 therefore corresponds more to a common principle for solving similar problems encountered in different parts of the currently ongoing Rel-18 SON MDT work item, more than a fully common solutions for these problems.

In this option, each NW configuration e.g. related to RA partitioning configuration may have a corresponding index. When UE logs the RA report, the corresponding RA partitioning configuration index is also recorded and reported to the NW together with RA report. In this way, the RA partitioning configuration related to the RA report can be identified by the NW. In addition, Opt4 can be also integrated into this option to avoid unnecessary RA report from UE to the NW.

Therefore, we propose:

Proposal 1: RAN3 to agree support Opt4 that NW can control and indicate to the UE which RA report (i.e. related to the latest RA configuration or all RA configurations) should be reported or not. Opt4 can be combined with Opt2, Opt3 or Opt5 whichever RAN3 agrees to support.

## 2.2 RA report forwarding from MN to SN in (NG)EN-DC or EN-DC scenario

In RAN3#120, it was agreed regarding RA report forwarding from MN to SN in (NG)EN-DC or EN-DC scenario:

Not consider the NG and S1 forwarding of RA report.

Thus the RA report forwarding will only happen over Xn/X2 interface, but not via NG or S1 interface. There is also one open issue from RAN3#120 on the RA report forwarding:

FFS whether and how to forward the RA Report in case the MN that retrieves the RA Report is not connected to any SN associated to the PSCells received with the RA Report

In both XnAP and X2AP procedure, it supports to indicate the information of neighboring gNB from one eNB/gNB to another eNB/gNB in Neighbour Information NR IE of XnAP or X2AP procedure message. The neighboring gNB information includes the NR cells that are served by the neighboring gNB. But it doesn’t indicate whether there is Xn connection between the neighboring gNBs and the indicated gNB/eNB. If the Neighbour Information NR IE is extended to include the further information of Xn connection availability as shown in the table below, upon receiving the Neighbour Information NR IE the MN knows to which neighboring gNB the SN in the PScell ID list has Xn connection. By knowing the connected SNs of MN’s own neighboring gNB, MN can forward the retrieved RA report to the connected neighbouring gNBs that has the Xn connection with the SNs in the PScell list even though MN doesn’t have direct Xn connection with the SNs. Then the MN’s neighbouring gNB can forward the RA report to the relevant SNs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Neighbour Information NR |  | 1 .. <maxnoofNeighbours> |  |  |
| >NRPCI | M |  | INTEGER (0..1007) | NR Physical Cell ID |
| >NR CGI | M |  | 9.2.2.7 |  |
| >TAC | M |  | 9.2.2.5 | Tracking Area Code |
| >RANAC | O |  | RAN Area Code 9.2.2.6 |  |
| >CHOICE NR-Mode-Info | M |  |  |  |
| >>FDD |  |  |  |  |
| >>>FDD Info |  | 1 |  |  |
| >>>>UL NR FreqInfo | M |  | NR Frequency Info 9.2.2.19 | This IE is ignored for NR operating bands for which uplink range of NREF is not defined in section 5.4.2.3 of TS 38.104 [24]. |
| >>>>DL NR FreqInfo | M |  | NR Frequency Info 9.2.2.19 |  |
| >>TDD |  |  |  |  |
| >>>TDD Info |  | 1 |  |  |
| >>>>NR FreqInfo | M |  | NR Frequency Info 9.2.2.19 |  |
| >Connectivity Support | M |  | 9.2.2.28 |  |
| >Measurement Timing Configuration | M |  | OCTET STRING | Includes the MeasurementTimingConfiguration inter-node message for the neighbour cell, as defined in TS 38.331 [10]. |
| > Xn connection availability indication | O |  | ENUMERATED (false, true, ...) | Indicates whether there is Xn connection to the neighbour NG-RAN node. |

Proposal 2: Extend the XnAP Neighbour Information NR IE to indicate the Xn connection availability to the neighbouring NG-RAN node.

# 3	Conclusions

Proposal 1: RAN3 to agree support Opt4 that NW can control and indicate to the UE which RA report (i.e. related to the latest RA configuration or all RA configurations) should be reported or not. Opt4 can be combined with Opt2, Opt3 or Opt5 whichever RAN3 agrees to support.

Proposal 2: Extend the XnAP Neighbour Information NR IE to indicate the Xn connection availability to the neighbouring NG-RAN node.