3GPP TSG-RAN WG3 Meeting #121	R3-234163

Toulouse, France, 21st – 25th August 2023

Agenda item:	  25.2

Source:		Lenovo

Title:		Discussion on PDU Set based Discard

Document for:		Discussion and Decision

- Introduction

The contribution discusses the impact from PDU set based discard according to RAN2 and SA2 agreements.

- Discussion

- PSI based discard timer configuration

In RAN2#121, it was agreed [1]:

RAN2 thinks PSI can be useful for PDU set-based discard. RAN2 aims to introduce a mechanism to allow UE to handle discarding of packets with different PSI in case of congestion. FFS for other cases.

As per RAN2’s agreement, it means that gNB can configure per PSI based PDCP discard timer to the UE.  PDU Set Importance (PSI) identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow.

Currently, the PSI is only carried in the GTP-U header of the QoS flow, which means that only gNB-CU-UP can get PSI information after receiving the user data of the QoS flow. But the PSI based discard timer configuration is usually configured by the gNB-CU-CP. In order to support PSI based discard timer configuration, the gNB-CU-CP needs to be aware of PSI information.

One of solution is the gNB-CU-UP sends PSI information to the gNB-CU-CP. But the gNB-CU-UP can only get the PSI information only after receiving the user data of the QoS flow since the PSI information is carried in the GTP-U header of the user data as per SA2 agreement. It means that the gNB-CU-CP may not be able to configure the PSI based discard timer at DRB setup.

The other more reasonable solution is the SMF sends the PSI information by NG-C signalling as a part of PDU Set QoS parameters to the gNB-CU-CP.

In case of aggregated gNB, the gNB should also be aware of PSI information before data arrival e.g., for DRB establishment.

PSI information should be provided to gNB or gNB-CU-CP for setting PSI based discard timer. Send an LS to SA2 to ask providing PSI information by NG-C signalling from SMF.

For UL data transmission, the gNB-CU-CP configures PSI based discard timer to the UE by RRC message. For DL data transmission, the gNB-CU-CP configures PSI based discard timer to the gNB-CU-UP in the PDCP Configuration IE via E1AP Bearer Context Setup Request or Bearer Context Modification request messages.

The PSI based discard timer is included in the PDCP Configuration IE in E1 AP.

- DL PDU Set Discard in F1-U

The gNB-CU may decide to discard the whole PDUs of the PDU set for example, when the PDSB or PDCP discard timer expires and The PDU Set Integrated Handling Information is received. In this case, the gNB-CU needs to indicate the gNB-DU to discard the PDCP PDUs of the PDU set that have not been delivered to lower layer. Currently, the information of downlink NR PDCP PDUs to be discarded for user data associated with a specific data radio bearer has been supported in the DL USER DATA frame as specified in the TS 38.425. if the gNB-CU decides to discard the PDU set, the gNB-CU can provide related PDCP SNs to be discarded to the gNB-DU following the TS 38.425. But to support more efficient discard information, it is worth to discuss whether the gNB-CU can indicate a PDU Set SN to be discard or PSI of PDU sets to be discard. For example, when the discard indication is to indicate to discard user data for a certain PSI level, the gNB-DU discard all PDCP PDUs as indicated with the certain PSI level, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers; when the discard indication is to indicate to discard user data for a certain PDU Set, the gNB-DU discard all PDCP PDUs as indicated with PDU set SN, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers;

RAN3 discusses to introduce PDU set or PSI based discard indication in F1-U protocol.

When the gNB-DU discard some PDCP PDUs that would be cause PDCP SN gap in the receiving side which causes PDCP window stalling if the PDCP SN is not used anymore.  How to avoid PDCP SN gap when PDCP discard performing. In order to reduce PDCP SN gap in the receiving side caused by PDCP discard, the gNB-DU provides discard feedback to gNB-CU. The discard feedback includes the PDCP SN information for the discarded PDCP PDUs. The discarded PDCP SN information may be in the F1-U user plane protocol, e.g., in the DL Data Delivery Status frame. The discarded PDCP SN information includes the PDCP SNs for all discarded PDCP PDUs. When receives the discarded PDCP SN information, the gNB-CU assigns the PDCP SNs that has been discarded by gNB-DU to new PDCP SDU, so that PDCP SN gap issue will not happen in UE side.

gNB-DU provides discard feedback that includes PDCP SNs of discarded PDCP PDUs to gNB-CU in F1-U protocol.

3	Conclusion

The contribution discusses the impact from PDU set based discard according to RAN2 and SA2 agreements.

- PSI information should be provided to gNB or gNB-CU-CP for setting PSI based discard timer. Send an LS to SA2 to ask providing PSI information by NG-C signalling from SMF.

- The PSI based discard timer is included in the PDCP Configuration IE in E1 AP.

- RAN3 discusses to introduce PDU set or PSI based discard indication in F1-U protocol.

- gNB-DU provides discard feedback that includes PDCP SNs of discarded PDCP PDUs to gNB-CU in F1-U protocol.

Reference

[1] RAN2-121 LTE MUSIM QoE XR_2023-03-03-EOM