3GPP TSG-RAN WG3 Meeting #121	R3-234167

21 – 25 August 2023, Toulouse, France

Title: 	Discussion on CHO in NR-DC

Source: 	Lenovo

Agenda item:	14.3

Document Type:	Other

1. Introduction

The CHO in NR-DC was discussed in RAN3#120 with the following achievements. The summary of offline discussion could be found in [1].

Regarding avoiding duplication of the data forwarding: RAN3 confirms the problem that is to be solved is avoiding that the single T-SN receives the same data from multiple T-MNs prepared for CHO (assuming there are multiple T-MNs prepared with the same T-SN). RAN3 agrees to enable the T-SN to let the T-MN know if it has direct path available to S-MN (or to both, S-SN and S-MN). It is FFS if any enhancements for the case with a single T-MN are needed. Regarding avoiding unnecessary CHO cancellation: RAN3 agrees to enable the T-MN to inform the S-MN if the CHO is prepared with full or delta configuration.

In this contribution, we will further discuss the remaining issues of CHO in NR-DC related to RAN3 impact.

2. Discussion

At the RAN3#119bis-e meeting, RAN3 agreed that T-MN provides the PDU session admission results of different T-SN(s) in the HO procedure considering the pair of candidate T-MN and T-SN(s). The reason is that, according to the current specification, the T-MN could only provide the same PDU session resources admitted list to report success of the establishment of PDU session resources from multiple candidate SNs, which requires the same bearer setup in different candidate SNs including the same TEIDs for early data forwarding.

RAN3 also agreed that a set of data forwarding addresses are provided from candidate T-MN to the source node. However, it was not determined how the set of data forwarding addresses are provided, i.e., per pair of <T-MN, T-SN> or per pair of <PCell, PSCell>.

In Rel-17 CPAC, we only have per T-SN data forwarding, but not per PSCell data forwarding. Therefore, it is not preferrable to provide the set of data forwarding addresses per pair of <PCell, PSCell> from candidate T-MN to the source node.

A possible way is to enable providing to the source MN a separate set of TEIDs for each pair <T-MN, T-SN>. For example, during the PDU session resource setup, each target SN may prepare different TEIDs for early data forwarding from the same candidate T-MN. Therefore, each candidate T-MN may receive different TEIDs for early data forwarding from all T-SNs. That is, for direct data forwarding, the source MN/SN may receive different TEIDs towards different candidate T-SNs, as illustrated by the Figure 1:

Figure 1: Direct data forwarding for CHO with multiple SCGs using separate set of TEIDs for each pair <T-MN, T-SN>

Proposal 1: Candidate MN should provide to the source MN/SN a separate set of data forwarding addresses for each pair <candidate MN, candidate SN>.

If the source MN/SN receives a separate set of data forwarding addresses for each pair <candidate MN, candidate SN>, there will be multiple data forwarding paths between the source MN/SN and the candidate SN, e.g., the data forwarding path 1 towards the candidate SN1, the data forwarding path 2 towards the candidate SN2, etc.

In our understanding, the purpose of direct data forwarding optimization is to avoid duplicated data forwarding, which means the source MN/SN forwards the data to the candidate SN via a single path. Therefore, solution to the direct data forwarding optimization for the CHO with multiple SCGs

Proposal 2: RAN3 to seek solution to direct data forwarding optimization for CHO with multiple SCGs.

For CHO with candidate SCGs, the RAN2 made the following agreements at the RAN2#122 meeting [2]:

P3: The CHO execution conditions (for candidate PCells) and CPA/CPC execution conditions (for candidate PSCells) are provided based on the source MeasConfig. P4: For CHO execution conditions, the source MN determines the execution conditions on candidate PCells, based on the source MCG MeasConfig. P5: For CPA/CPC execution conditions, the candidate MN determines the parameters of the execution conditions for candidate PSCells (e.g. event A4 threshold). P6: The candidate MN informs the source MN about the prepared candidate PSCells and parameters of the associated execution conditions (e.g. event A4 threshold). According to the received information from the candidate MN, the source MN generates the corresponding execution conditions based on the source MCG MeasConfig to the UE. FFS how, if to support event A3/A5. P8: For CHO with candidate SCGs for CPA/CPC, the RRCReconfigurtaion message in one CHO container includes one MCG configuration and one SCG configuration (i.e. similar to Rel-17 CHO with SCG configuration). P9: The execution conditions associated with one CHO container includes both CHO execution condition(s) and CPA/CPC execution condition(s), i.e. triggering conditions on both candidate PCell and candidate PSCell. P10: If there are multiple candidate PSCells associated with one candidate PCell, the NW can provide multiple CHO configurations for the same candidate PCell, i.e. each one contains one MCG configuration (for the same candidate PCell) and one SCG configuration (for different candidate PSCell).

RAN2 agreed that the candidate MN determines the parameters of the execution conditions for candidate PSCells (e.g. event A4 threshold). The candidate MN should provide these parameters of the execution conditions to the source MN in the Handover Request Acknowledge message, to enable the source MN generates the corresponding execution conditions by using these parameters for CPAC to the UE.

Proposal 3: 	RAN3 to confirm that the source MN generates the CPAC execution condition configuration for the CHO with multiple SCGs.

At the RAN2#121bis-e meeting, the RAN2 made the following baseline [3]:

When both CHO and CPC conditions are met, both CHO and CPC cell change is executed. Baseline: The UE waits until both CHO and CPC conditions are met (always). (furthermore, it is assumed that if needed the network can provide a complementary CHO-only configuration, to avoid failures in deployments where failure would otherwise be likely to happen). Alternative: FFS if When CHO condition is met, but CPC condition is not met, CHO execution is triggered (and somehow source SCG can be released). IF allowed in the new configuration the UE may continue evaluation of CPC/CPA conditions.

At the RAN2#122 meeting, RAN2 made the further agreements:

P12: When the CPA/CPC execution condition is met but no CHO execution condition is met, the UE continues to evaluate both CHO and CPA/CPC execution conditions. For CHO+CPC we only consider execution when BOTH conditions are met. - (When the CHO execution condition is met but no CPC execution condition is met, if there is an available CHO-only or Rel-17 CHO with SCG configuration for which the CHO condition is met, the UE performs the CHO-only or Rel-17 CHO with SCG execution, and THUS the network can handle such situation by providing proper configurations).

According to the RAN2 agreements, the CHO-only configuration is a complementary to avoid failures. That is, if there is no CPAC execution conditions are fulfilled after CHO execution conditions are fulfilled for a certain time, the UE is allowed to perform CHO only without SCG.

The first question is which node decides whether to prepare the configuration of CHO-only. Since it is the source MN to decide to use CHO, it is reasonable that the source MN decides whether the CHO-only configuration is needed. For example, based on the measurement results of candidate target cells, the source MN determines that the CHO-only configuration is needed for the UE. If the source MN determines the CHO-only configuration is needed, it requests the candidate MN to prepare the configuration of CHO-only.

Proposal 4: The source MN decides whether to request the candidate MN to prepare the complementary configuration of CHO-only.

In the Rel-16 CHO or Rel-17 CHO with target SCG, the source gNB decides the CHO candidate cells and the corresponding execution conditions based on the received L3 measurement results. In Rel-18 CHO with multiple SCGs, the above similar procedure will be reused. Therefore, it is proposed that the CHO-only execution conditions are determined by the source MN.

Proposal 5: The execution condition for complementary CHO-only switch should be configured by the source MN for the CHO with multiple SCGs.

There are two conditional handover configurations for the UE: CHO with CPAC configuration, and CHO-only configuration. To identify underlying conditions during successful conditional handovers and to detect a sub-optimal successful handover event, the candidate MN may indicate to the source MN that the UE has successfully accessed to the candidate MN using CHO-only configuration.

Upon receipt of the indication, the source MN may analyse whether its conditional handover configuration needs adjustment. For example, the source MN may adjust the candidate cells which needs to be prepared with CHO with CPAC configurations.

Proposal 6: The candidate MN should indicate to the source MN that the UE has successfully accessed to the candidate MN using the complementary CHO-only configuration.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

Proposal 1: Candidate MN should provide to the source MN/SN a separate set of data forwarding addresses for each pair <candidate MN, candidate SN>.

Proposal 2: RAN3 to seek solution to direct data forwarding optimization for CHO with multiple SCGs.

Proposal 3: RAN3 to confirm that the source MN generates the CPAC execution condition configuration for the CHO with multiple SCGs.

Proposal 4: The source MN decides whether to request the candidate MN to prepare the complementary configuration of CHO-only.

Proposal 5: The execution condition for complementary CHO-only switch should be configured by the source MN for the CHO with multiple SCGs.

Proposal 6: The candidate MN should indicate to the source MN that the UE has successfully accessed to the candidate MN using the complementary CHO-only configuration.

4. Reference

- R3-233358, Summary of offline discussion on selected aspect of the CHO with MR-DC (CB: # MoblityEnh2_CHO), Nokia (moderator).

- RAN2#122 chair notes.

- RAN2#121bis-e chair notes.