3GPP TSG-RAN WG3 Meeting #121	R3-234182

Toulouse, France, 21st – 25th August 2023

Agenda item:	  20.2

Source:		Lenovo

Title:		 (TP to TS 38.401) Remaining issues of MT-SDT

Document for:		Discussion and Decision

- Introduction

This contribution discusses remaining issues of MT-SDT.

- Discussion

- Regarding the support MT-SDT in E1-AP there is still an FFS:

When new DL data is coming through non-SDT bearer, the gNB-CU-UP shall send DL DATA NOTIFICATION message. 2

FFS on either excluding MT-SDT Information, or introducing a new indicator (e.g., Non MT-SDT Data) or other method.

- Using a new indication, i.e., non SDT Data arrival, in the DL DATA NOTICATION message is a clean and robust solution.  When the gNB-CU-UP receives a data of non-SDT DRB from UPF, the gNB-CU-UP sends the DL DATA NOTIFICATION message with the Non-SDT DAT Arrival IE.

A new Non-SDT DATA Arrival IE is added in the DL DATA NOTIFICATION message in E1AP for notifying data arrival of a non-SDT DRB.

- There is another FFS:

When large size of new DL data is coming through SDT bearer, the gNB-CU-UP shall send DL DATA NOTIFICATION message.

- FFS on either excluding MT-SDT Information, or introducing a new indicator (e.g., MT-SDT Oversize), or other method.

- It is more reasonable that the gNB-CU-CP controls when and how the gNB-CU-UP to notify the ‘data oversize’ to the gNB-CU-CP. One of possible solution is the gNB-CU-CP provides a threshold to gNB-CU-UP. When the data size buffered in gNB-CU-UP is higher that the threshold, the gNB-CU-UP provides an ‘data oversize’ indication to the gNB-CU-CP. Then the gNB-CU-CP can terminate the SDT procedure.

The gNB-CU-CP provides a data oversize threshold to the gNB-CU-UP.

When the data size buffered in gNB-CU-UP is higher that the threshold, the gNB-CU-UP provides an ‘data oversize’ indication to the gNB-CU-CP in the DL DATA NOTIFICATION message.

- For the other FFS:

FFS: How to calculate MT-SDT Data Size for SDT DL data packets.

- We believe that how to calculate MT-SDT Data size can let to network implementation that does not need to be specified.

How to calculate MT-SDT data size for SDT DL data packets relies on network implementation that does not needs to be specified.

- Text Proposal for 38.401

- In current text of 38.401, only two cases are considered for triggering MT-SDT paging:

-	the gNB-CU-CP receives the MT-SDT information in the DL DATA NOTIFICATION message from the gNB-CU-UP.

-	the gNB-CU-CP receives DL signalling over NGAP

- There is a missing case that the gNB-CU-CP receives XnAP RAN paging message with MT-SDT information.

In TS 38.401, add the case that the gNB-CU-CP receives XnAP RAN paging message with MT-SDT information for triggering MT-SDT paging.

3	Conclusion

This contribution discusses remaining issues of MT-SDT. And we propose:

- A new Non-SDT DATA Arrival IE is added in the DL DATA NOTIFICATION message in E1AP for notifying data arrival of a non-SDT DRB.

The gNB-CU-CP provides a data oversize threshold to the gNB-CU-UP.

When the data size buffered in gNB-CU-UP is higher that the threshold, the gNB-CU-UP provides an ‘data oversize’ indication to the gNB-CU-CP in the DL DATA NOTIFICATION message.

How to calculate MT-SDT data size for SDT DL data packets relies on network implementation that does not needs to be specified.

In TS 38.401, add the case that the gNB-CU-CP receives XnAP RAN paging message with MT-SDT information for triggering MT-SDT paging.

Reference

[1] RAN2#120 chairman notes.

Text proposal for 38.401 BL CR:

### 8.18.x	MT-SDT

The procedure for mobile terminated small data transmission in RRC Inactive is shown in Figure 8.18.x-1.

Figure 8.18.x-1: Mobile Terminated Small Data Transmission in RRC Inactive state.

1. During the setup or modification of the bearer context as specified in 8.9.2, the gNB-CU-CP requests the gNB-CU-UP to provide MT-SDT information.

2a-0.	The gNB-CU-UP receives DL data for the UE in RRC Inactive on NG-U interface.

2a-1.	The gNB-CU-UP sends DL DATA NOTIFICATION message to the gNB-CU-CP. If determining that DL data packets are only mapped to SDT bearers, as requested in step 1, the gNB-CU-UP includes the MT-SDT information in the DL DATA NOTIFICATION message.

2b. The gNB-CU-CP receives DL NAS signalling over NGAP.

2c. The gNB-CU-CP receives the MT-SDT information in XnAP RAN PAGING message.

3.	After 2a or 2b or 2c, the gNB-CU-CP sends PAGING message to the gNB-DU. The MT-SDT information may be included in the PAGING message.

4.	The gNB-DU sends the Paging message to the UE. In case the MT-SDT information is received in step 3, the gNB-DU includes the MT-SDT indicator in the Paging message.

5. If the UE has been successfully reached, it initiates the RRC connection resume procedure as described in 8.6.2 or 8.9.6.2, or initiates the SDT procedure as described in 8.18.1 or from step 9 in 8.18.2 or from step 1 in 8.18.3 with the following difference:

- In case SDT procedure is initiated, the UE may indicate the RRC Resume Request is for MT-SDT, which may be without UL data.