3GPP TSG-RAN WG3 #121	R3-234189

Toulouse, France, 21st – 25th August 2023

Agenda Item:	12.2.2.2

Source:	Lenovo, Intel Corporation, ZTE

Title: 	(TP for TS38.423 BLCR) Left issues related to UE trajectory collection by the source gNB

Document for:	Discussion and Approval

# 1	Introduction

In the last RAN3 meeting, the following agreements are made to allow source NGRAN node collecting measured UE trajectory from target NGRAN node after handover. In this paper, we further discuss the related procedure and signalling.

In Rel18, the UE trajectory prediction is only limited to the next one hop target NGRAN node, and the source NGRAN node can collect measured UE trajectory (in the form of UE History Information) related to one or multiple cells within the single next hop target NGRAN node after handing over the concerned UE(s). Take the procedure for UE performance feedback report as the baseline to also support the measured UE trajectory collection by the source gNB. The details on how to request the measured UE trajectory collection needs to be further discussed.

# 2	Discussion

## 2.1	Other related issues

In this section, few issues raised by companies during the discussion are analysed.

Issue#1: UE context maintenance

Some companies are concerned that in order for the source gNB to understand and map the received future UE trajectory to the corresponding UE, the source gNB needs to store the UE context for very long time.

First of all, to predict UE trajectory, source gNB only needs to understand the UE trajectory, i.e., visited cells and dwelling time, in the past and in the future. UE related RRC configurations are not relevant. In this sense, after handover, source gNB only needs to keep the identification information of the concerned UE, e.g., UE XnAP ID or some other ID, which will be indicated by the target gNB when providing the measured future UE trajectory back to the source gNB. Other UE contexts such as RRC configuration can be released after successfully handover.

In addition, as clarified in the previous discussion, even after source gNB receives the UE context release message from the target gNB, the standard does not forbid the source gNB from keeping the UE context. It can be eventually left to gNB implementation to keep a minimum UE context, instead of full UE context, for the sake of future UE trajectory collection.

We also notice the similar issue exists for the UE performance feedback collection as well, wherein the source gNB does not need to keep the full UE context to understand/analyse the UE performance feedback provided by the target gNB

After successful handover, source gNB only needs to maintain some identification information of the concerned UE, instead of full UE context, in order to understand the future UE trajectory provided by the target gNB.

After successful handover and receiving the UE CONTEXT RELEASE message, it is upon source gNB implementation to keep minimum UE context, e.g., UE XnAP ID, to further receive the future UE trajectory from the target gNB.

Issue#2: UE enters RRC Inactive/Idle state in the target gNB

Another issue raised by some companies is that what if UE enters RRC Inactive/Idle after being handed over to the target gNB.

In that case, of course target gNB can no longer provide the UE trajectory measurement. One simple implementation would be the target gNB indicate to the source gNB that the requested data for certain UE can no longer be provided.

Also, we believe the same issue also exist for the UE performance feedback collection, wherein the target gNB can no longer measure the UE performance if the UE enters RRC Inactive/Idle states.

Similar as for UE performance feedback collection, the target gNB can no longer provide the requested future UE trajectory to source gNB if UE enters RRC Inactive/Idle state.

## 2.2	Procedure and signalling

In our understanding, the same procedures for UE performance feedback collection can be reused with modification for source gNB to collect measured UE trajectory from target gNB as well considering both follow similar principles with respect to:

- The target gNB can only start measuring after the handover is successful

- The data collection request is better to be decoupled from the handover request to not impact the handover procedure

- The source gNB needs to keep (minimum) UE context to receive the UE performance feedback or UE trajectory from the target gNB

- The target gNB cannot measure if UE enters RRC Inactive/Idle state

A high-level exemplary procedure is given in Figure 1. Similar as the UE performance feedback collection, before the handover takes place, the source gNB requests the target gNB to measure and provide future UE trajectory back to the source gNB if indicated by the handover request. Then, in the HANDOVR REQUEST message, the source gNB indicates that the target gNB should provide the future UE trajectory measurement for this particular UE. After handover completion, the target gNB measures and provides the UE trajectory as requested conveyed in the AI/ML INFORMATION UPDATE (FFS naming). Each AI/ML INFORAMTION UPDATE (FFS naming) contains the visited cells and dwelling time of concerned UE(s) since the last update message or since the handover completion.

Figure 2: High level procedure for the future UE trajectory collection

RAN3 extends the agreed procedure for UE performance feedback collection to also support the future UE trajectory collection by the source gNB, and agree the proposed TP.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	After successful handover, source gNB only needs to maintain some identification information of the concerned UE, instead of full UE context, in order to understand the future UE trajectory provided by the target gNB.

Observation 2	Similar as for UE performance feedback collection, the target gNB can no longer provide the requested future UE trajectory to source gNB if UE enters RRC Inactive/Idle state.

Based on the discussion above, we propose:

Proposal 1	After successful handover and receiving the UE CONTEXT RELEASE message, it is upon source gNB implementation to keep minimum UE context, e.g., UE XnAP ID, to further receive the future UE trajectory from the target gNB.

Proposal 2	RAN3 extends the agreed procedure for UE performance feedback collection to also support the future UE trajectory collection by the source gNB.

# TP for TS 38.423 BLCR

<<<<<<<<<<<<<<<<<<<< Start of the Change >>>>>>>>>>>>>>>>>>>>

### 8.4.AA	AI/ML Information Reporting Initiation (FFS on the name)

#### 8.4.AA.1	General

This procedure is used by an NG-RAN node to request the reporting of AI/ML related information to another NG-RAN node.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be requested using this procedure.

Editor’s Note: FFS content of AL/ML related information.

#### 8.4.AA.2	Successful Operation

Figure 8.4.AA.2-1: AI/ML Information Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the AI/ML INFORMATION REQUEST message to NG-RAN node2 to start AI/ML related information reporting and stop AI/ML related information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested AI/ML related information reporting according to the parameters given in the request in case the Registration Request IE is set to “start”; or

-	shall stop all cells AI/ML related information reporting and terminate the reporting in case the Registration Request IE is set to “stop”; or

-	FFS

If the Registration Request IE is set to “start” in the AI/ML INFORMATION REQUEST message and the Report Characteristics IE indicates cell specific AI/ML related information reporting, the Cell To Report List IE shall be included.

If NG-RAN node2 is capable to provide all of the requested information, it shall initiate the AI/ML related information reporting as requested by NG-RAN node1 and respond with the AI/ML INFORMATION RESPONSE message.

If NG-RAN node2 is capable to provide some but not all of the requested information, it shall initiate the AI/ML related information reporting for the admitted requested information and include the Node Measurement Initiation Result IE or the Per Cell Measurement Initiation Result IE or both in the AI/ML INFORMATION RESPONSE message.

If the Reporting Periodicity IE in the AI/ML INFORMATION REQUEST is present, this indicates the periodicity for the reporting of periodic AI/ML related information. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

If the Requested Prediction Time IE in the AI/ML INFORMATION REQUEST message is present, it indicates the specific point in time to which the prediction of the information requested applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the AI/ML INFORMATION REQUEST indicates the type of objects NG-RAN node2 shall perform measurements or prediction on. NG-RAN node2 shall include in the AI/ML INFORMATION UPDATE message:

-	the Predicted Radio Resource Status IE, if the first bit, “Predicted Radio Resource Status” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”. FFS on the details of Predicted Radio Resource Status IE.

-	the Predicted Number of Active UEs IE, if the second bit, “Predicted Number of Active UEs” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”;

-	the Predicted RRC Connections IE, if the third bit, “Predicted RRC Connections” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-	the Average UE Throughput DL IE, if the fourth  bit, “Average UE Throughput DL” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-	the Average UE Throughput UL IE, if the fifth bit, “Average UE Throughput UL” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-	the Average Packet Delay IE, if the sixth bit, “Average Packet Delay” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

-	the Average Packet Loss IE, if the seventh bit, “Average Packet Loss” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

- 	the UE Trajectory IE, if the ninth bit, "UE trajectory" of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST (FFS naming) message is set to "1";

#### 8.4.AA.3	Unsuccessful Operation

Figure 8.4.AA.3-1: AI/ML Information Reporting Initiation, unsuccessful operation

If none of the requested AI/ML related information reporting cannot be initiated, NG-RAN node2 shall send the AI/ML INFORMATION FAILURE message with an appropriate cause value.

#### 8.4.AA.4	Abnormal Conditions

For the same Measurement ID, if the initiating NG-RAN node1 does not receive either the AI/ML INFORMATION RESPONSE message or the AI/ML INFORMATION FAILURE message, the NG-RAN node1 may reinitiate the AI/ML Information Reporting Initiation procedure towards the same NG-RAN node, provided that the content of the new AI/ML INFORMATION REQUEST message is identical to the content of the previously unacknowledged AI/ML INFORMATION REQUEST message.

If the NG-RAN node2 receives an AI/ML INFORMATION REQUEST message which includes the Registration Request IE set to “stop” and if the NG-RAN node2 Measurement ID value received in the AI/ML INFORMATION REQUEST message is not used, the NG-RAN node2 shall initiate AI/ML INFORMATION FAILURE message with an appropriate cause value.

If the Report Characteristics IE bitmap is set to “0” (all bits are set to “0”) in the AI/ML INFORMATION REQUEST message then NG-RAN node2 shall initiate an AI/ML INFORMATION FAILURE message with an appropriate cause value.

If the NG-RAN node2 receives a AI/ML INFORMATION REQUEST message which includes the Registration Request IE set to “start” and the NG-RAN node1 Measurement ID IE corresponding to an existing on-going AI/ML information reporting, then NG-RAN node2 shall initiate a AI/ML INFORMATION FAILURE message with an appropriate cause value.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.CC	AI/ML INFORMATION REQUEST (FFS on the name)

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested AI/ML related information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) (FFS on others) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss Eighth Bit = Energy Cost Ninth Bit = UE Trajectory FFS on the coding | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | FFS |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.FF	AI/ML INFORMATION UPDATE (FFS on the name)

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested AI/ML related information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell AI/ML Info Result (FFS on the name) |  | 0..1 |  |  | YES | ignore |
| >Cell AI/ML Info Result Item(FFS on the name) |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  |  |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>Predicted Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >>Predicted RRC Connections | O |  | 9.2.2.56 |  | – |  |
| UE Associated Info Result |  | 0..1 |  |  |  |  |
| > UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  |  |  |
| >> UE Assistant Identifier (FFS on the name) | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1 |  |  |
| >>UE Performance | O |  | 9.2.3.Y |  |  |  |
| >>UE trajectory | O |  | 9.2.3.64 |  |  |  |
| Energy Cost | O |  | FFS | It represents the actual measurement of the Energy Cost (FFS) |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE that can be served by a NG-RAN node. Value is FFS. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.64	UE History Information

The UE History Information IE may contain information about cells that a UE has been served by in active state prior to the target cell. The overall mechanism is described in TS 36.300 [12]. The UE History Information IE may alternatively contain information about cells that a UE has been served by the current NG-RAN node.

NOTE:	The definition of this IE is aligned with the definition of the UE History Information IE in TS 38.413 [5].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Last Visited Cell List |  | 1..<maxnoofCellsinUEHistoryInfo> |  | Most recent information is added to the top of this list |
| >Last Visited Cell Information | M |  | 9.2.3.65 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinUEHistoryInfo | Maximum number of last visited cell information records that can be reported in the IE. Value is 16. |

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>