3GPP TSG-RAN WG3 Meeting #121	R3-234282

Toulouse, France, August 21 – 25, 2023

Title: 	(TP for AI/ML BL CR for TS 38.423) Remaining open issues for the Energy Saving use case

Source: 	Huawei

Agenda item:	12.2.2.3

Document Type:	Discussion and decision

# 1. Introduction

In last RAN3#120 meeting the discussion on how to support the Energy Saving (ES) use case using AI/ML progressed and the following agreement was reached [1][2]:

EC is represented as an index, which should be normalized and defined by OAM. The index value could be encoded as an integer from 0 to a maximum. The maximum value should guarantee enough accuracy.

and, due to the involvement of the OAM in defining and normalizing the Energy Cost (EC) index, an LSout to SA5 was agreed in [3].

Nonetheless, there are still important open issues that need to be further discussed, reported below from [1]:

FFS on whether the inferred EC should be introduced in R18.

FFS on whether the additional load needs to be introduced in R18.

In this paper, we discuss the above listed left issues for enabling the ES use case by means of AI/ML.

# 2. Discussion

## 2.1 On the need of “Additional Load” / inferred EC in Rel-18

In RAN3#119bis-e meeting the following list of information was proposed for the definition of the “Additional Load” [4]:

The following information are supported for the definition of “Additional Load”:

-	Number of RRC connections to be offloaded,

-	Number of Active UEs to be offloaded

-	PRB load to be offloaded (the definition needs to be discussed further)

-	Average UL/DL PDCP SDU data volume to be offloaded

-	Target Cell of the offloading action

and the “Average UE Throughput” was also added to the above list of items, with companies in RAN3 expressing their views on each of these items as reported in the SoD on CB: # AIRAN4_ES from the last meeting [2].

The definition of the “Additional Load” is strictly related to the inferred EC, i.e., the EC prediction value that corresponds to a predicted energy consumption value resulting from an AI/ML inference as a result of transferring a certain amount of UEs/traffic (the so-called “Additional Load”) from a source NG-RAN node towards a neighbour NG-RAN node. This means that the source NG-RAN node should be able to request and receive from the neighbour NG-RAN node a prediction of the EC related to this “Additional Load”, which is signalled over RAN interfaces (e.g., Xn), so that the source NG-RAN node can derive potential ES actions towards the neighbour NG-RAN node.

The discussions happened so far on the “Additional Load” [2][5] showed that companies have very diverging views of what should be signalled to the neighbour NG-RAN node as part of the “Additional Load” in order for the neighbour NG-RAN node to infer its own EC value.

From [2] it can be seen that a clear majority thinks that at least “Number of RRC connections to be offloaded” and “Number of Active UEs to be offloaded” should be part of the “Additional Load” to provide an idea of the numerosity of devices involved, while some companies believe that also “PRB load to be offloaded” and “Average UL/DL PDCP SDU data volume to be offloaded” should be included to reflect the actual traffic to be offloaded. However, for the latter two items, some other companies could not agree on their benefit: in particular, for the “PRB load to be offloaded” (representing the physical resource usage), it is observed that it will be different in the neighbour (target) NG-RAN node with respect to the source NG-RAN node because it highly depends on the radio environment and scheduling policy, hence it cannot accurately reflect the traffic volume; to solve this issue some companies in [5] suggested sending pathloss information of the UEs’ to be offloaded from the source to the neighbour (target) NG-RAN node, which basically increases the number of items that constitute the “Additional Load” and the related signalling complexity even more. Concerning the “Average UL/DL PDCP SDU data volume to be offloaded”, instead, it is acknowledged that it is service-related and the UE service is time-varying, hence the source NG-RAN node cannot know how much data volume it needs to offload.

In addition to the above statements – which concerns the items which are considered by most of the companies – there are also other metrics proposed in the past two meetings, e.g., type and performance of UEs to be offloaded [6], offloading scenario (e.g., handover, PSCell change/addition) [7], Number of bearer/PDU session per UE [8], information on service or traffic characteristics of UEs to be offloaded [9], just to mention a few.

So, based on the above, it seems evident that companies identified a significant number of information for the “Additional Load” which are available at source NG-RAN node and that are relevant to determine an accurate prediction of the EC at neighbour NG-RAN node side. However, the corresponding amount of signalling needed to transfer this information over RAN interfaces from the source NG-RAN node to the neighbour (target) NG-RAN node could also be significant, and the complexity in designing such signalling seems not motivated by clear benefits provided by EC inferences related to the “Additional Load” in determining ES decisions/strategies at source NG-RAN node.

Observation 1: A significant number of information for the “Additional Load” available at source NG-RAN node are relevant to determine an accurate prediction of the EC at neighbour NG-RAN node side. But the corresponding amount of signalling needed to transfer this information over RAN interfaces from the source NG-RAN node to the neighbour (target) NG-RAN node could also be significant and the related complexity seems not motivated by clear benefits provided by EC inferences related to the “Additional Load” in determining ES decisions/strategies at source NG-RAN node.

In addition, in the last meeting there were proposals in [9], [10] and [11] claiming that the introduction of the “Additional Load” in RAN3 specifications can be avoided and they provide alternative solutions for enabling the AI/ML-driven ES use case. In [9], the “Additional Load” and the corresponding inferred EC are not considered for determining ES strategies at the source NG-RAN node, which in fact requests to periodically receive actual EC measurements from a neighbour (target) NG-RAN node, such actual EC measurements along with other relevant information already available at the source NG-RAN node (e.g., PRB load) are used to determine an ES action (e.g., cell shutoff at source NG-RAN node and subsequent UE/traffic offloading to the neighbour NG-RAN node) that minimizes the overall EC of the network. In our understanding, a similar approach is also reported in [10], in the sense that the “Additional Load” is not used but actual EC measurements from the neighbour NG-RAN node(s) are used to assess the overall EC of the network before a certain ES action is taken as well as to infer what could be the overall network EC after that ES action is performed. For both [9] and [10] there will be another collection of actual EC measurements from the neighbour (target) NG-RAN node after the ES action, to assess whether such ES action has actually determined an improvement (i.e., a reduction) of the network EC. Finally, in [11], it is suggested focusing on the concept of inferred EC related to the current total load of the neighbour (target) NG-RAN node, which has the capability to predict its inferred EC by analysing its historical EC patterns and such information can then be relayed to the source NG-RAN node to facilitate appropriate ES actions. It should also be noted that in our paper from the last meeting in [12] we also suggested an approach – described in the next section of this contribution – which does not consider both the “Additional Load” and the inferred EC.

Observation 2: Alternative solutions for deriving inference-based ES actions that do not exploit the “Additional Load” and/or the inferred EC have been proposed by companies in RAN3. Such solutions seem to achieve the purpose of the ES use case with a good trade-off in terms of signalling design complexity.

Therefore, for the sake of progressing and also considering that we are approaching the end of the Work Item, with only three remaining meetings (including the current one), we have the following proposals:

Proposal 1: The definition and signalling over RAN interfaces of the “Additional Load” as well as inferred EC are not pursued in Rel-18.

Proposal 1bis: In order to keep the complexity at reasonable levels, RAN3 to agree that inference-based ES actions can be derived at the source NG-RAN node based on current, measured EC values from the neighbour NG-RAN nodes without the need to preliminary exchange “Additional Load” information and corresponding inferred EC values over RAN interfaces.

## 2.2 Use and interpretation of EC measurements to derive AI/ML-based ES actions

Based on the proposals in the previous section, we now discuss how actual EC measurements from the neighbour NG-RAN nodes can be used at the source NG-RAN node side to determine the ES action that better maximizes the overall ES gain (i.e., minimizes the overall EC of the area of the network where the source NG-RAN node and its neighbours operate). Such ES scenario does not foresee any signalling of the “Additional Load” and the corresponding inferred EC values from neighbour NG-RAN nodes but it is enabled by an exchange of actual measured EC values based on the current load locally managed by each of the neighbour NG-RAN nodes in a certain area where the source NG-RAN node is located.

By referring to Figure 1, the basic assumption is that Node A has knowledge of the current overall EC (i.e., before the ES action is performed) within a certain area covered by Node A and its neighbours (e.g., Node B in Figure 1); by means of AI/ML, Node A makes predictions (i.e., AI/ML inferences for ES) of the future overall EC in that area of the network based on an initial exchange over RAN interfaces (Xn) of the current measured EC values from neighbour NG-RAN nodes (Node B) which is only dependent on the current load locally managed by these neighbour NG-RAN nodes, and this load information is not signalled to Node A but remains internal to Node B. It should be noted that other additional relevant information already available at Node A side, i.e. not exchanged over RAN interfaces, e.g., one or more of the items proposed so far for the “Additional Load” definition, could also be exploited by Node A to predict the future overall EC of the concerned area of the network. It is up to the Node A to decide which and how many additional relevant information needs to be used to make the overall EC prediction of the concerned area of the network. Based on such prediction, Node A determines the most appropriate ES action to be performed to achieve a future overall EC value as per the prediction previously computed, e.g. a traffic offloading from Node A to Node B.

Figure 1 – ES scenario realized with no preliminary “Additional Load” information exchange.

After such ES action has been performed, Node A could be interested in evaluating whether the ES action itself provided the expected improvement (i.e., reduction) of the overall EC: to this end, Node A may ask neighbour NG-RAN nodes to provide the new (i.e., after the ES action) measured EC values so that Node A can now re-assess the overall EC and determine whether such prediction was accurate and beneficial in terms of ES. If necessary, Node A could use such feedback information (post ES action neighbour NG-RAN nodes’ EC values) for further EC predictions.

As already stated, such ES scenario can be realized without any exchange over RAN interfaces (Xn) of the “Additional Load” and corresponding inferred EC but simply exploits the current, measured EC values from the neighbouring NG-RAN nodes being reported to the source NG-RAN node.

Based on the scenario description above we propose the following:

Proposal 2: It is up to the source NG-RAN node to determine how to estimate the overall EC of the area of the network where it and its neighbour NG-RAN nodes operate based at least on the measured EC values provided by the neighbour NG-RAN nodes.

Proposal 3: After an ES action has been performed, the measured EC can be signalled from neighbour NG-RAN nodes to the source NG-RAN node. It is up to the source NG-RAN node to decide how to use this information (e.g., for feedback purposes).

In RAN3#119bis-e meeting there was a discussion on how to interpret EC values, i.e., whether they represent either the absolute EC values or the relative (i.e. "delta") EC values, as follows:

Which of the following two options to be selected for inferred and measured EC definition:

1)	Inferred EC represents the node level EC value assuming that an additional load is served; Measured EC represents the actual node level EC value, e.g. after an additional load is transferred

2)	Inferred EC represents the delta increase of the EC value assuming that an additional load is served; Measured EC represents the delta increase of the EC value after an additional load is transferred

Note that the above FFS concerns both the inferred and the measured EC values, but considering our previous proposals we focus on the measured EC metric (see highlighted text in green above). It should be recalled that, in RAN3#119bis-e [4][13], there was a common understanding in RAN3 that the case of “Inferred EC represents the delta increase of the EC value assuming that an additional load is served; Measured EC represents the actual node level EC value” is not considered.

In our understanding, the delta value alone does not represent enough information for the source NG-RAN node to understand the corresponding EC value, unless a reference EC value is provided. For the sake of keeping the specification work at a reasonable complexity level, for the case the delta information needs to be determined, the source NG-RAN node could compute the delta value internally by comparing the measured absolute EC values from the (target) neighbour NG-RAN node reported before and after the ES action is performed.

Proposal 4: The measured EC represents a mapped value corresponding to an absolute actual node-level energy consumption value, not to a delta increase of the energy consumption value after an ES action is performed.

For the sake of clarity, concerning the following FFS from RAN3#119bis-e [4]:

Whether this EC can only be reported after HO needs to be further discussed.

Based on the exemplary ES scenario illustrated in Figure 1, it should be clear enough that the measured EC can also be reported before a handover or, in general, before any ES action is performed. Referring back to Figure 1, in fact, Node A (the source NG-RAN node) has knowledge of the current (i.e., before the ES action is performed) overall EC within a certain area covered by Node A and its neighbours (Node B). However, after an ES action has been performed, Node A could be interested in evaluating whether the ES action itself provided the expected improvement (i.e., reduction) of the overall EC: to this end, Node A may ask neighbour NG-RAN nodes to provide the new (i.e., after the ES action) measured EC values so that Node A can now re-assess the overall EC and determine whether such prediction was accurate and beneficial in terms of ES. If necessary, Node A could use such feedback information for re-training purposes.

Proposal 5: The current, measured EC from neighbour NG-RAN nodes can be reported before and/or after an ES action (e.g., handover).

The corresponding TP to TS 38.423 reflecting our proposals is provided in the Annex.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

Observation 1: A significant number of information for the “Additional Load” available at source NG-RAN node are relevant to determine an accurate prediction of the EC at neighbour NG-RAN node side. But the corresponding amount of signalling needed to transfer this information over RAN interfaces from the source NG-RAN node to the neighbour (target) NG-RAN node could also be significant and the related complexity seems not motivated by clear benefits provided by EC inferences related to the “Additional Load” in determining ES decisions/strategies at source NG-RAN node.

Observation 2: Alternative solutions for deriving inference-based ES actions that do not exploit the “Additional Load” and/or the inferred EC have been proposed by companies in RAN3. Such solutions seem to achieve the purpose of the ES use case with a good trade-off in terms of signalling design complexity.

Proposal 1: The definition and signalling over RAN interfaces of the “Additional Load” as well as inferred EC are not pursued in Rel-18.

Proposal 1bis: In order to keep the complexity at reasonable levels, RAN3 to agree that inference-based ES actions can be derived at the source NG-RAN node based on current, measured EC values from the neighbour NG-RAN nodes without the need to preliminary exchange “Additional Load” information and corresponding inferred EC values over RAN interfaces.

Proposal 2: It is up to the source NG-RAN node to determine how to estimate the overall EC of the area of the network where it and its neighbour NG-RAN nodes operate based at least on the measured EC values provided by the neighbour NG-RAN nodes.

Proposal 3: After an ES action has been performed, the measured EC can be signalled from neighbour NG-RAN nodes to the source NG-RAN node. It is up to the source NG-RAN node to decide how to use this information (e.g., for feedback purposes).

Proposal 4: The measured EC represents a mapped value corresponding to an absolute actual node-level energy consumption value, not to a delta increase of the energy consumption value after an ES action is performed.

Proposal 5: The current, measured EC from neighbour NG-RAN nodes can be reported before and/or after an ES action (e.g., handover).

# 4. Reference

- 3GPP RAN3#120 Chairman’s notes (RAN3_120_agenda_20230530_EOM1.doc)

- R3-233349, SoD on CB: # AIRAN4_ES, CMCC

- R3-233515, LS on AI/ML for NG-RAN Energy Saving Energy Cost index, RAN3

- 3GPP RAN3#119bis-e Chairman’s notes (RAN3_119bis-e_agenda_202304026_EOM1.doc)

- R3-231881, Summary of Offline Discussion on CB: # AIRAN4_ES, Ericsson (Moderator)

- R3-231812, Further discussion for NG-RAN AIML energy saving, NTT DOCOMO

- R3-233232, Further discussion for NG-RAN AIML energy saving, NTT DOCOMO

- R3-232794, Discussion on ES Enhancements for NG-RAN AI/ML, Qualcomm

- R3-233113, AI-ML Network Energy Saving, Ericsson

- R3-233166, Discussion on the EC metric, CATT

- R3-233005, Further discussion on AI/ML based energy saving, ZTE

- R3-232842, (TP for AI/ML BL CR for TS 38.423) Remaining open issues for the Energy Saving use case, Huawei

- R3-232028, SoD on CB: # AIRAN4_ES, Ericsson

# 5. Annex: TP to 38.423 (based on BLCR R3-233756)

<<<<<< Start of CHANGEs >>>>>>

# 8	XnAP procedures

Editor’s note: FFS on the names of new introduced procedures and messages.

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

**************** skip unchanged part *******************

| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| --- | --- | --- | --- |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| IAB Transport Migration Management | IAB TRANSPORT MIGRATION MANAGEMENT REQUEST | IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE | IAB TRANSPORT MIGRATION MANAGEMENT REJECT |
| IAB Transport Migration Modification | IAB TRANSPORT MIGRATION MODIFICATION REQUEST | IAB TRANSPORT MIGRATION MODIFICATION RESPONSE |  |
| IAB Resource Coordination | IAB RESOURCE COORDINATION REQUEST | IAB RESOURCE COORDINATION RESPONSE |  |
| Partial UE Context Transfer | PARTIAL UE CONTEXT TRANSFER | PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE | PARTIAL UE CONTEXT TRANSFER FAILURE |
| Data Acquisition Reporting Initiation | DATA ACQUISITION REQUEST | DATA ACQUISITION RESPONSE | DATA ACQUISITION FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| RAN Multicast Group Paging | RAN MULTICAST GROUP PAGING |
| SCG Failure Information Report | SCG FAILURE INFORMATION REPORT |
| SCG Failure Transfer | SCG FAILURE TRANSFER |
| F1-C Traffic Transfer | F1-C TRAFFIC TRANSFER |
| Retrieve UE Context Confirm | RETRIEVE UE CONTEXT CONFIRM |
| Conditional PSCell Change Cancel | CONDITIONAL PSCELL CHANGE CANCEL |
| Data Acquisition Reporting | DATA ACQUISITION UPDATE |

**************** skip unchanged part *******************

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.4.AA	Data Acquisition Reporting Initiation

#### 8.4.AA.1	General

This procedure is used by an NG-RAN node to request the reporting of AI/ML related information to another NG-RAN node.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be requested using this procedure.

Editor’s Note: FFS content of AL/ML related information.

#### 8.4.AA.2	Successful Operation

Figure 8.4.AA.2-1: Data Acquisition  Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the DATA ACQUISITION REQUEST message to NG-RAN node2 to start AI/ML related information reporting and stop AI/ML related information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested AI/ML related information reporting according to the parameters given in the request in case the Registration Request IE is set to “start”; or

-	shall stop all cells AI/ML related information reporting and terminate the reporting in case the Registration Request IE is set to “stop”; or

-	FFS

If the Registration Request IE is set to “start” in the DATA ACQUISITION REQUEST message and the Report Characteristics IE indicates cell specific AI/ML related information reporting, the Cell To Report List IE shall be included.

If NG-RAN node2 is capable to provide all of the requested information, it shall initiate the AI/ML related information reporting as requested by NG-RAN node1 and respond with the DATA ACQUISITION RESPONSE message.

If NG-RAN node2 is capable to provide some but not all of the requested information, it shall initiate the AI/ML related information reporting for the admitted requested information and include the Node Measurement Initiation Result IE or the Per Cell Measurement Initiation Result IE or both in the DATA ACQUISITION RESPONSE message.

If the Reporting Periodicity IE in the DATA ACQUISITION REQUEST is present, this indicates the periodicity for the reporting of periodic AI/ML related information. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

If the Requested Prediction Time IE in the DATA ACQUISITION REQUEST message is present, it indicates the specific point in time to which the prediction of the information requested applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the DATA ACQUISITION REQUEST indicates the type of objects NG-RAN node2 shall perform measurements or prediction on. NG-RAN node2 shall include in the DATA ACQUISITION UPDATE message:

-	the Predicted Radio Resource Status IE, if the first bit, “Predicted Radio Resource Status” of the Report Characteristics IE included in the DATA ACQUISITION REQUEST message is set to “1”. FFS on the details of Predicted Radio Resource Status IE.

-	the Predicted Number of Active UEs IE, if the second bit, “Predicted Number of Active UEs” of the Report Characteristics IE included in the DATA ACQUISITION REQUEST message is set to “1”;

-	the Predicted RRC Connections IE, if the third bit, “Predicted RRC Connections” of the Report Characteristics IE included in the DATA ACQUISITION REQUEST message is set to “1”.

-	the Average UE Throughput DL IE, if the fourth bit, “Average UE Throughput DL” of the Report Characteristics IE included in the DATA ACQUISITION REQUEST message is set to “1”.

-	the Average UE Throughput UL IE, if the fifth bit, “Average UE Throughput UL” of the Report Characteristics IE included in the DATA ACQUISITION REQUEST message is set to “1”.

-	the Average Packet Delay IE, if the sixth bit, “Average Packet Delay” of the Report Characteristics IE included in the DATA ACQUISITION REQUEST message is set to “1”.

-	the Average Packet Loss IE, if the seventh bit, “Average Packet Loss” of the Report Characteristics IE included in the DATA ACQUISITION REQUEST message is set to “1”.

-    the Energy Cost IE, if the eighth bit, "Energy Cost" of the Report Characteristics IE included in the DATA 	ACQUISITION REQUEST message is set to "1".

#### 8.4.AA.3	Unsuccessful Operation

Figure 8.4.AA.3-1: Data Acquisition Reporting Initiation, unsuccessful operation

If none of the requested AI/ML related information reporting cannot be initiated, NG-RAN node2 shall send the DATA ACQUISITION FAILURE message with an appropriate cause value.

#### 8.4.AA.4	Abnormal Conditions

For the same Measurement ID, if the initiating NG-RAN node1 does not receive either the DATA ACQUISITION RESPONSE message or the DATA ACQUISITION FAILURE message, the NG-RAN node1 may reinitiate the Data Acquisition Reporting Initiation procedure towards the same NG-RAN node, provided that the content of the new DATA ACQUISITION REQUEST message is identical to the content of the previously unacknowledged DATA ACQUISITION REQUEST message.

If the NG-RAN node2 receives a DATA ACQUISITION REQUEST message which includes the Registration Request IE set to “stop” and if the NG-RAN node2 Measurement ID value received in the DATA ACQUISITION REQUEST message is not used, the NG-RAN node2 shall initiate DATA ACQUISITION FAILURE message with an appropriate cause value.

If the Report Characteristics IE bitmap is set to “0” (all bits are set to “0”) in the DATA ACQUISITION REQUEST message then NG-RAN node2 shall initiate a DATA ACQUISITION FAILURE message with an appropriate cause value.

If the NG-RAN node2 receives a DATA ACQUISITION REQUEST message which includes the Registration Request IE set to “start” and the NG-RAN node1 Measurement ID IE corresponding to an existing on-going AI/ML information reporting, then NG-RAN node2 shall initiate a DATA ACQUISITION FAILURE message with an appropriate cause value.

### 8.4.BB	Data Acquisition Reporting

#### 8.4.BB.1	General

This procedure is initiated by an NG-RAN node to report AI/ML related information accepted by the NG-RAN node following a successful Data Acquisition Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be reported using this procedure.

Editor’s Note: FFS content of AL/ML related information.

#### 8.4.BB.2	Successful Operation

Figure 8.4.11.2-1: Data Acquisition Reporting, successful operation

NG-RAN node2 shall report the accepted AI/ML related information in DATA ACQUISITION UPDATE message. The accepted AI/ML related information is the information that was successfully initiated during the preceding Data Acquisition Reporting Initiation procedure.

#### 8.4.BB.3	Unsuccessful Operation

Not applicable.

#### 8.4.BB.4	Abnormal Conditions

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.CC	DATA ACQUISITION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested AI/ML related information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) (FFS on others) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss Eighth Bit = Energy Cost FFS on the coding | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | FFS |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

#### 9.1.3.DD	DATA ACQUISITION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested AI/ML related information, for all or part of the measurement objects included in the reporting is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Measurement Initiation Result |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | ignore |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | EACH | ignore |
| >>Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(128)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Energy Cost. Other bits shall be ignored by the NG-RAN node1. |  |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Per Cell Measurement Initiation Result |  | 0..1 |  | List of measurement objects that failed to be initiated per cell. | YES | ignore |
| >Per Cell Measurement Initiation Result Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | EACH | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell. | – |  |
| >>>Measurement Failure Cause Item |  | 1 .. <maxFailedMeasObjects> |  |  |  |  |
| >>>>Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(128)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss. Other bits shall be ignored by the NG-RAN node1. | – |  |
| >>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxFailedMeasObjects | Maximum number of measurement objects that can fail per measurement. Value is 124. |
| maxFailedMeasPerNode | Maximum number of measurement objects that can fail per node. Value is 124. |

#### 9.1.3.EE	DATA ACQUISITION FAILURE

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that for all of  the requested objects the reporting cannot be initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.FF	DATA ACQUISITION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested AI/ML related information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell AI/ML Info Result (FFS on the name) |  | 0..1 |  |  | YES | ignore |
| >Cell AI/ML Info Result Item(FFS on the name) |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  |  |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>Predicted Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >>Predicted RRC Connections | O |  | 9.2.2.56 |  | – |  |
| UE Associated Info Result |  | 0..1 |  |  |  |  |
| > UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  |  |  |
| >> UE Assistant Identifier (FFS on the name) | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1 |  |  |
| >>UE Performance | O |  | 9.2.3.Y |  |  |  |
| >>UE trajectory | O |  | FFS |  |  |  |
| Energy Cost | O |  | 9.2.3.xx | It represents the actual measurement of the Energy Cost of the NG-RAN node2 |  |  |

**************** skip unchanged part *******************

<<<<<< Next CHANGES >>>>>

#### 9.2.3.xx	Energy Cost

The Energy Cost IE indicates the Energy Cost of a NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Energy Cost | M |  | INTEGER (0..FFS) | Value 0 represents the minimum EC value. Value FFS represents the maximum EC value. |

<<<<<< End of CHANGES >>>>>