3GPP TSG-RAN WG3 Meeting #121                                                                     R3-234308

Toulouse, France, Aug 21st – 25th, 2023

Source:	CATT

Title:	Discussion on Cell DTX/DRX for NES

Agenda Item:	24.2

Document for:	Discussion and Decision

1 Introduction

According to the last RAN3 meeting, a way forward for Cell DTX/DRX has been agreed as below:

Cell DTX/DRX: WA: Support the exchange of the Cell DTX/DRX configuration over Xn. The detail infor of the Cell DTX/DRX configuration is pending to RAN1 and RAN2.

In this contribution, we provide further analysis and proposals on the Cell DTX/DRX for NES.

2 Discussion

The RAN2 discussion on Cell DTX/DRX has not reached a final agreement, and according to the latest email discussion [Post122][307][NES] of RAN2, most companies believed that the Cell DTX/DRX pattern will take UE C-DRX pattern as starting point. Additionally, the last RAN3 meeting has agreed to introduce the Cell DTX / DRX information over Xn interface. Therefore, it is proposed that the detailed IE design continues to wait for the progress of RAN2.

Proposal 1: The detailed IE design for Cell DTX/DRX continues to wait for progress of RAN2.

An important issue to be addressed for RAN3 is which logical entity to determine the Cell DTX/DRX configuration for CU-DU split case? According to RAN2 discussion, the specific configuration of Cell DTX/DRX has impacts on transmission scheduling for connected UE, refer to RAN2 conclusion as below:

Agreements:

1	UE monitors PDCCH for RAR during Cell DTX non-active time. The ra-ResponseWindow could be started as legacy.

2	UE monitors PDCCH for msg4 during Cell DTX non-active time. The ra-ContentionResolutionTimer could be started as legacy.

3	Working assumption:  When the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy.  It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

4	Once gNB recognizes there is an emergency call or public safety related service (e.g. MPS/MCS), the NW should ensure there is no impact to the emergency call (e.g. may deactivate Cell DTX/DRX).  The behavior is captured in stage 2 spec

5	When an DG grant is received, by the gNB during cell DRX/DTX, the UE follows the grant assignment (i.e. like in legacy).  This includes DL HARQ feedback.

It is observed the gNB-DU needs to be aware of the active/non-active period of Cell DTX/DRX for Cell DTX/DRX behaviors.

Observation 1: The gNB-DU needs to be aware of active/non-active period of Cell DTX/DRX.

On the other hand, RAN2 is discussing the Cell DTX/DRX alignment with UE C-DRX, and according to current specification, the UE C-DRX configuration is determined by gNB-DU node. Similarly, since the Cell DTX/DRX has the requirement of aligning UE C-DRX, then it is also related to the transmission scheduling of each connected UE. Thus, the gNB-DU is a suitable logical node for determining the Cell DTX/DRX configuration.

Observation 2: The gNB-DU is a suitable logical node for determining the Cell DTX/DRX configuration parameters.

Proposal 2: It is up to gNB-DU to determine the Cell DTX/DRX configuration for NES.

Based on the conclusion of RAN2, the Cell DTX/DRX pattern is configured via UE specific RRC signaling. Additionally, according to the current specification, the UE C-DRX configuration is included in the DU to CU RRC Information IE of the existing UE context setup/modification response messages. With the same logic, the Cell DTX/DRX configuration parameters can also be included in the DU to CU RRC Information IE to meet the above signaling requirement of RAN2.

Proposal 3: The Cell DTX/DRX configuration parameters can be included in the DU to CU RRC Information IE.

Per RAN2 agreement, Cell DTX/DRX can be activated by UE specific RRC signaling. Whether and when the cell enters NES states can also be determined by gNB-DU. When Cell DTX/DRX is activated, gNB-DU can indicate Cell DTX/DRX activation via Cell DTX/DRX configuration which is contained in the DU to CU RRC Information IE using the UE context modification required procedure, or gNB-DU can indicate Cell DTX/DRX activation to UEs by L1 common signaling.

Proposal 4: The Cell DTX/DRX activation can be included in the DU to CU RRC Information IE.

Furthermore, for UE C-DRX, in current specification, gNB-DU is allowed to actively initiate the corresponding parameter modification procedure using UE CONTEXT MODIFICATION REQUIRED message, and similarly, a corresponding signaling procedure can be reused for Cell DTX/DRX configuration updating. gNB-DU can use the UE Context Modification Required procedure to update Cell DTX/DRX parameters or release Cell DTX/DRX configuration.

Proposal 5: The UE Context Modification Required procedure can be used for updating Cell DTX/DRX configuration.

Finally, the coordination between gNB-DU dan gNB-CU for Cell DTX/DRX needs to be discussed in RAN3. In our view, the gNB-CU can send the assistance information for Cell DTX/DRX to gNB-DU. The assistance information can include the recommended Cell DTX/DRX parameters for gNB-DU to configure. Or the assistance information can also include requirement for NES actions (such as the CCO Assistance Information indicating CCO issues), then the gNB-DU receiving the assistance information can decide to activate Cell DTX/DRX.

Proposal 6: The gNB-CU can send assistance information for Cell DTX/DRX to gNB-DU.

3 Conclusion

In the previous sections we made the following observations and proposals:

Proposal 1: The detailed IE design for Cell DTX/DRX continues to wait for progress of RAN2.

Proposal 2: It is up to gNB-DU to determine the Cell DTX/DRX configuration for NES.

Proposal 3: The Cell DTX/DRX configuration parameters can be included in the DU to CU RRC Information IE.

Proposal 4: The Cell DTX/DRX activation can be included in the DU to CU RRC Information IE.

Proposal 5: The UE Context Modification Required procedure can be used for updating Cell DTX/DRX configuration.

Proposal 6: The gNB-CU can send assistance information for Cell DTX/DRX to gNB-DU.

4 Reference

- Meeting Minutes, RAN3 Chairman, RAN3#119bis-e

- R3-232084, LS on the enhancements to restricting paging in a limited area, to SA2 and RAN2, Nokia, RAN3#119bis-e