3GPP TSG-RAN WG3 Meeting #121	R3-234327

Toulouse, France, 21 - 25 August 2023

Agenda item:	23.2.3

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS 38.455 BL CR) Positioning accuracy enhancements

Document for:	Discussion and Decision

# 1	Introduction

The latest version of the WID on Expanded and Improved NR Positioning was approved by RAN plenary in [1], and includes the following objectives:

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs) - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2]. […] - Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4]. - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements. - Specify measurements that are limited to a single carrier/PFL. […] - Define extensions of signalling, protocol, and procedure for NR positioning enhancement, as needed for the above objectives [RAN3].

In this paper, we evaluate the RAN3 specification impacts of uplink carrier phase positioning and RedCap positioning.

# 2	Discussion

## 2.1	Uplink carrier phase positioning (UL CPP)

UL CPP is a positioning method which makes use of UL Reference Signal Carrier Phase (RSCP) measurements. The TRPs measure the UL RSCP based on UL SRS from the target UE, along with either the gNB Rx-Tx time difference measurement or the RTOA measurement. The TRPs also simultaneously measure the UL RSCP based on UL SRS from the Positioning Reference Unit (PRU). The LMF can then calculate the target UE location based on the phase measurements, the timing measurements, the PRU measurements, and the known location of the TRPs and PRU.

An overview of UL CPP is illustrated in Figure 1, where both the target UE and PRU simultaneously transmit UL SRS. The TRPs then measure the carrier phase of both the target UE and the reference UE and report the measurements to the LMF.

Figure 1. Overview of UL Carrier Phase.

In more detail, the steps of UL CPP according to latest RAN1 agreements are as follows:

Step 1:	The LMF requests the serving gNB of the UE to configure the transmission of the UL SRS resources from the UE within indicated time window(s).

Step 2:	The LMF requests the serving gNB and neighbouring gNBs of the UE to measure the UL SRS resources from the UE within indicated time window(s).

Step 3:	The gNB(s) report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to the LMF.

Regarding step 1, RAN1 has reached the following working assumption:

Working assumption To enable LMF to optionally request the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), support: - Option 1D: Each of the time windows is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of the consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE PeriodicitySRS in “Requested SRS Transmission Characteristics” in TS 38.455. - FFS: the maximum number of the windows

The NRPPa Positioning Information Exchange procedure already enables the LMF to request the serving gNB to configure SRS transmissions for the UE. This is achieved by the LMF sending the POSITIONING INFORMATION REQUEST message including the Requested SRS Transmission Characteristics IE.  However, it is currently not possible for the LMF to indicate time window(s) for the SRS transmissions.  Therefore, the following needs to be added to the POSITIONING INFORMATION REQUEST message:

-	Start of time window (mandatory): subframe number, slot offset, and symbol index with respect to the SFN initialization time.

-	Duration of time window (mandatory): number of consecutive slots/symbols

-	Periodicity of time window (optional): defined similar to the PeriodicitySRS IE in the Requested SRS Transmission Characteristics IE.

Proposal 1:	Introduce a Time Window Information for SRS IE in the POSITIONING INFORMATION REQUEST message to indicate the start of the time window, the duration of the time window, and (optionally) the periodicity of the time window for UL SRS transmission.

Regarding step 2, RAN1 has agreed on the following:

Agreement To enable LMF to request the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources from the UE within indicated time window(s), each time window is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE Measurement Periodicity in MEASUREMENT REQUEST in TS 38.455. - FFS: the maximum number of the windows

The NRPPa Measurement procedure already enables the LMF to request the serving/neighboring gNB(s) to measure the UL SRS from the UE. However, it is currently not possible for the LMF to indicate time window(s) for measuring.  Therefore, the following needs to be added to the MEASUREMENT REQUEST message:

-	Start of time window (mandatory): subframe number, slot offset, and symbol index with respect to the SFN initialization time.

-	Duration of time window (mandatory): number of consecutive slots/symbols

-	Periodicity of time window (optional): defined similar to the Measurement Periodicity IE in the MEASUREMENT REQUEST message.

Proposal 2:	Introduce a Time Window Information for Measurement IE in the MEASUREMENT REQUEST message to indicate the start of the time window, the duration of the time window, and (optionally) the periodicity of the time window to measure UL SRS from the UE.

It is observed that the time window parameters for measuring are the same as the time window parameters for UL SRS transmission. However, the periodicity values of the Measurement Periodicity IE in the MEASUREMENT REQUEST message (for measuring) are not the same as the periodicity values in the PeriodicitySRS IE in the Requested SRS Transmission Characteristics IE (for UL SRS transmission) as shown below:

-	PeriodicitySRS IE: INTEGER (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …)

-	Measurement Periodicity IE: INTEGER (160, 320, 640, 1280, 2560, 5120, 10240, 20480, 40960, 61440, 81920, 368640, 737280, 1843200)

RAN3 can tentatively use the overlapping values (i.e., 160ms to 10240ms) with FFS pending RAN1.

Regarding step 3, RAN1 has agreed on the following:

Agreement NR UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the UL SRS signal for positioning purpose configured for the measurement. A UL RSCP is associated with a specific RF frequency. - FFS: the reference point of the UL RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: The support of MIMO SRS for positioning is transparent to UE Agreement - Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF

Based on the above RAN1 agreements, a new measurement (UL RSCP) needs to be introduced in the NRPPa Measurement procedure.

Proposal 3:	Introduce an “UL-RSCP” codepoint in the TRP Measurement Type IE of the MEASUREMENT REQUEST message, and an UL RSCP IE in the TRP Measurement Result IE. Details of the UL RSCP IE are FFS pending further RAN1 progress.

## 2.2	Positioning for RedCap UEs

RAN1 has been discussing support of both DL frequency hopping and UL frequency hopping. For DL frequency hopping it was agreed that the hopping would occur within one DL PRS resource.

Agreement For positioning for RedCap UEs with DL PRS Rx Hopping, the UE hops within a DL PRS resource - FFS: whether there is specification update needed for RAN1 - FFS: remaining details

From NG-RAN perspective this means that DL frequency hopping for RedCap devices may not require any specification changes, i.e. existing NRPPa may already be sufficient for DL frequency hopping.

Observation 1:	There may not be any RAN3 specification impact to support DL frequency hopping.

Proposal 4:	No need for RAN3 discussion on DL frequency hopping, unless requested by other WGs (e.g., via LS).

There has been discussion in RAN1 on the best way to introduce SRS frequency hopping support to the specifications. One option is to introduce a “virtual” SRS configuration where multiple SRS for positioning resources/resource sets are tied to the “virtual” SRS configuration. Each SRS resource/resource set within the virtual configuration would correspond to one SRS hop. For UL frequency hopping RAN1 has already made the below agreement.

Agreement For RedCap UEs, support SRS for positioning frequency hopping by - Using a configuration separate from the existing BWP configuration - FFS: hopping is configured within a SRS resource or across SRS resources

In Rel-16/17 for UL positioning techniques the serving TRP configures the SRS and then reports this SRS configuration to the LMF who then distributes the configuration to measuring TRPs. This followed the longstanding principle that SRS is configured per BWP. However, for UL frequency hopping RAN1 has now agreed that this principle will be changed. Therefore, RAN3 will need to update its specifications to support this change. More details are still open in RAN1 but at least changes to the SRS Configuration IE will be necessary in NRPPa.

Observation 2:	Adding support for UL frequency hopping for RedCap devices will have RAN3 specification impact, e.g. in NRPPa to update the SRS Configuration.

Proposal 5:	RAN3 to discuss how to support signaling for SRS for positioning frequency hopping using a configuration separate from the existing BWP configuration, pending further progress in RAN1/RAN2.

# 3	Conclusions

In this paper, we evaluated the RAN3 specification impacts of Rel-18 positioning accuracy enhancements and proposed the following:

Related to uplink carrier phase positioning:

Proposal 1:	Introduce a Time Window Information for SRS IE in the POSITIONING INFORMATION REQUEST message to indicate the start of the time window, the duration of the time window, and (optionally) the periodicity of the time window for UL SRS transmission.

Proposal 2:	Introduce a Time Window Information for Measurement IE in the MEASUREMENT REQUEST message to indicate the start of the time window, the duration of the time window, and (optionally) the periodicity of the time window to measure UL SRS from the UE.

Proposal 3:	Introduce an “UL-RSCP” codepoint in the TRP Measurement Type IE of the MEASUREMENT REQUEST message, and an UL RSCP IE in the TRP Measurement Result IE. Details of the UL RSCP IE are FFS pending further RAN1 progress.

Related to positioning for RedCap UE:

Proposal 4:	No need for RAN3 discussion on DL frequency hopping, unless requested by other WGs (e.g., via LS).

Proposal 5:	RAN3 to discuss how to support signaling for SRS for positioning frequency hopping using a configuration separate from the existing BWP configuration, pending further progress in RAN1/RAN2.

# References

- RP-231460 Revised WID on Expanded and Improved NR Positioning, Intel Corporation

- TR 38.859 Study on expanded and improved NR positioning

# Annex	A:	Text Proposal for TS 38.455

Start of modifications

#### 9.1.1.10	POSITIONING INFORMATION REQUEST

This message is sent by the LMF to request positioning information.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Requested SRS Transmission Characteristics | O |  | 9.2.27 |  | YES | ignore |
| UE Reporting Information | O |  | 9.2.70 |  | YES | ignore |
| UE TEG Information Request | O |  | ENUMERATED(onDemand, periodic, stop, …) |  | YES | ignore |
| UE TEG Reporting Periodicity | C-ifUeTegInfoReqPeriodic |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, …) |  | YES | reject |
| Time Window Information for SRS | O |  | 9.2.x1a |  | YES | reject |

| Condition | Explanation |
| --- | --- |
| ifUeTegInfoReqPeriodic | This IE shall be present if the UE TEG Information Request IE is set to the value "periodic". |

next modification

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | reject |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, 30min, and 60min are not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | EACH | reject |
| >TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP) |  | - |  |
| >Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16] | - |  |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) | This IE is ignored when the Measurement Characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.2.68 | This IE is ignored when the Report Characteristics IE is set to “periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.2.81 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4, …) |  | YES | ignore |
| Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |
| Time Window Information for Measurement | O |  | 9.2.x1b |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |
| ifMeasPerExt | This IE shall be present if the Measurement Periodicity IE is set to the value "extended". |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

next modification

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  |  |  |
| >CHOICE Measured Results Value | M |  |  |  |  |  |
| >>UL Angle of Arrival | M |  | 9.2.38 |  | - |  |
| >>UL SRS-RSRP | M |  | INTEGER (0..126) |  | - |  |
| >>UL RTOA | M |  | 9.2.39 |  | - |  |
| >>gNB Rx-Tx Time Difference | M |  | 9.2.40 |  | - |  |
| >>Z-AoA | M |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA | M |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP | M |  | 9.2.72 |  | YES | reject |
| >>UL RSCP | M |  | 9.2.x4 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |

next modification

### 9.2.x1a	Time Window Information of SRS

This IE contains the time window(s) when UL SRS transmission is requested.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Time Window Start | M |  | 9.2.x2 |  |
| Time Window Duration | M |  | 9.2.x3 |  |
| Time Window Type | M |  | ENUMERATED (single, periodic, …) |  |
| Time Window Periodicity | C-ifTimeWindowTypePeriodic |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …) [FFS] | Milli-seconds |
| Number of Periodic Time Windows | C-ifTimeWindowTypePeriodic |  | INTEGER (1..[FFS], …) |  |

| Condition | Explanation |
| --- | --- |
| ifTimeWindowTypePeriodic | This IE shall be present if the Time Window Type IE is set to the value “periodic”. |

### 9.2.x1b	Time Window Information of Measurement

This IE contains the time window(s) when UL SRS measurement is requested.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Time Window Start | M |  | 9.2.x2 |  |
| Time Window Duration | M |  | 9.2.x3 |  |
| Time Window Type | M |  | ENUMERATED (single, periodic, …) |  |
| Time Window Periodicity | C-ifTimeWindowTypePeriodic |  | ENUMERATED (160, 320, 640, 1280, 2560, 5120, 10240, …) [FFS] | Milli-seconds |
| Number of Periodic Time Windows | C-ifTimeWindowTypePeriodic |  | INTEGER (1..[FFS], …) |  |

| Condition | Explanation |
| --- | --- |
| ifTimeWindowTypePeriodic | This IE shall be present if the Time Window Type IE is set to the value “periodic”. |

### 9.2.x2	Time Window Start

This IE indicates the start of the time window with respect to the SFN initialization time.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| System Frame Number | M |  | INTEGER(0..1023) |  |
| Slot Number | M |  | INTEGER(0..79) |  |
| Symbol Index | M |  | INTEGER(0..12) |  |

### 9.2.x3	Time Window Duration

This IE indicates the duration of the time window.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Number of Consecutive Slots | M |  | INTEGER (0..[FFS], …) |  |
| Number of Consecutive Symbols | M |  | INTEGER (0..[FFS], …) |  |

### 9.2.x4	UL RSCP

This IE contains the UL Reference Signal Carrier Phase (RSCP) measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UL RSCP Measurement | M |  | [FFS] |  |
| Additional Path List [FFS] | O |  | [FFS] |  |

End of modifications