3GPP TSG-RAN WG3 Meeting #121	R3-234352

Toulouse, France, August 21st – 25th, 2023

Agenda item:	20.2

Source: 	LG Electronics Inc.

Title: 	Necessary corrections for stage-2 BL CRs (TPs for NR_MT_SDT BL CR for TS 38.300 and TS 38.401)

Document for:	Discussion and Decision

# Introduction

The NR MT-SDT WI is scheduled to be finished at this RAN3-121.

Based on the agreements and progresses so far, we propose some corrections in the stage-2 BL CRs for 38.300 [1] and 38.401 [2].

# Discussion

## Corrections for NR MT-SDT BL CR for 38.300 [1]

The following figure has been captured to support MT-SDT in TS 38.300:

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

## 18.X	MT-SDT with/without UE context relocation

The overall procedure for MT-SDT procedure with/without UE context relocation is illustrated in the figure 18.X-1.

Figure 18.X-1. MT-SDT with/without UE context relocation

1.  DL user data and/or DL NAS signalling are received at the Last Serving gNB for the UE in RRC_INACTIVE state.

2.  The Last Serving gNB may send MT-SDT information to the neighbour gNBs within the RNA, via XnAP RAN PAGING message.

3.  The gNB that receives MT-SDT information within the RNA, takes into account the MT-SDT information received in the XnAP RAN PAGING message to decide whether to trigger MT-SDT Paging. The gNB which ultimately reaches the UE via the Uu Paging becomes the Receiving gNB.

4/5.  The UE may decide to initiate MT-SDT procedure and send an RRCResumeRequest message for MT-SDT to the Receiving gNB.

Editor note: The above step 3/4/5 can be revisited by RAN2 progress.

6. The Receiving gNB identifies the Last Serving gNB using the I-RNTI and retrieves the UE context by means of XnAP Retrieve UE Context procedure.

7.  The following steps are the same as Figure 18.2-1/18.3-1 from step 3, except that the first SDT user data and/or NAS signalling is DL SDT data and/or DL SDT NAS signalling.

NOTE: In case DL non-SDT data or DL non-SDT signalling has arrived between step 2 and step 7, the Last Serving gNB should relocate the UE context to the Receiving gNB and forward the received data to the Receiving gNB.

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

Regarding the above highlighted steps, there are two aspects to be clarified.

Firstly, according to RAN3-119bis agreement:

Agree to reusing existing IE (i.e., SDT Support Request) within the XnAP Retrieve Context Request message when the UE resumes for MT-SDT, and there is no RAN3 standard impact.

Namely, RAN3 concluded the issue on whether and how the receiving gNB indicates MT-SDT initiation (from the UE) to the last serving gNB, which was agreed to re-use the existing SDT Support Request IE defined in Rel-17 MO-SDT during XnAP Retrieve UE Context Retrieval procedure.

In Rel-17 MO-SDT, this aspect was captured in TS 38.300 Section 18.2.1 step 2. However, the above steps 6/7 mirror the MO-SDT procedure from step 3, not from step 2.

To abide by the agreement correctly, these steps 6/7 should be merged and refer the MO-SDT procedure from step 2.

Proposal 1: In 38.300 BL CR, the step 6/7 should be merged and refer the MO-SDT procedure from step 2, in order to correctly reflect the agreement to re-use the existing SDT Support Request IE by the receiving gNB in indicating MT-SDT initiation from the UE to the last serving gNB during XnAP Retrieve UE Context Retrieval procedure.

If Proposal 1 is agreed, then the updated call flow would naturally cover the MO-SDT case, e.g. when MO-SDT is initiated from the UE just before the UE receives MT-SDT paging. RAN2 already agreed that RBs configured for SDT are common for MO-SDT and MT-SDT. And from NW point of view, difference between MO-SDT vs MT-SDT initiation is that the resume request for MO-SDT includes UL data, while the resume request due to MT-SDT paging includes the MT-SDT resume cause (and may not include any UL data). On the other hand, shares the same subsequent procedure of full/partial context relocation. From procedural point of view, there would be no problem as long as the last serving gNB is able to know that SDT is initiated from the UE (which is via the SDT Support Request IE).

The case when MO-SDT procedure is initiated in step 5 could be further clarified. However, this is RAN2 detail, and we already have a place holder for RAN2 to re-visit the steps 3/4/5. We can leave up to RAN2 to correctly reflect the stage-2 behaviors when MO-SDT is initiated instead.

Furthermore, RAN3 agreed that a neighbor gNB, who receives XnAP RAN Paging from the last serving gNB including MT-SDT information, may decide NOT to trigger MT-SDT paging (i.e. between step 2 and step 3). That is, RAN3 agreed to allow a neighbor gNB to trigger the legacy paging instead, even if the last serving gNB requested for MT-SDT paging.

If that happens, then the UE will be paged as in legacy and the legacy resume would follow, meaning that the step 3 and afterwards will follow the legacy procedure, not SDT procedures. This aspect should be properly captured in stage-2 to avoid confusions.

Observation 1: In 38.300 BL CR, between step 2 and step 3, RAN3 agreed that it is possible that a neighboring gNB who receives XnAP RAN Paging including MT-SDT information may decide to trigger legacy paging instead.

Proposal 2: In 38.300 BL CR, a NOTE should be captured that, if the receiving gNB decided not to trigger MT-SDT paging, the step 3 and afterwards follows the legacy procedure, not SDT procedures.

## Corrections for NR MT-SDT BL CR for 38.401 [2]

The following figure has been captured so far to support MT-SDT in TS 38.401:

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

### 8.18.x	MT-SDT

The procedure for mobile terminated small data transmission in RRC Inactive is shown in Figure 8.18.x-1.

Figure 8.18.x-1: Mobile Terminated Small Data Transmission in RRC Inactive state.

1. During the setup or modification of the bearer context as specified in 8.9.2, the gNB-CU-CP requests the gNB-CU-UP to provide MT-SDT information.

2a-0.	The gNB-CU-UP receives DL data for the UE in RRC Inactive on NG-U interface.

2a-1.	The gNB-CU-UP sends DL DATA NOTIFICATION message to the gNB-CU-CP. If determining that DL data packets are only mapped to SDT bearers, as requested in step 1, the gNB-CU-UP includes the MT-SDT information in the DL DATA NOTIFICATION message.

2b. The gNB-CU-CP receives DL NAS signalling over NGAP.

3.	After 2a or 2b, the gNB-CU-CP sends PAGING message to the gNB-DU. The MT-SDT information may be included in the PAGING message.

4.	The gNB-DU sends the Paging message to the UE. In case the MT-SDT information is received in step 3, the gNB-DU includes the MT-SDT indicator in the Paging message.

5. If the UE has been successfully reached, it initiates the RRC connection resume procedure as described in 8.6.2 or 8.9.6.2, or initiates the SDT procedure as described in 8.18.1 or from step 9 in 8.18.2 or from step 1 in 8.18.3 with the following difference:

- In case SDT procedure is initiated, the UE may indicate the RRC Resume Request is for MT-SDT, which may be without UL data.

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

Regarding the MT-SDT information highlighted above, one is provided from CU-UP to CU-CP and the other is provided to DU over F1.

Currently, it is described as if the same information from CU-UP is forwarded to DU, but according to our stage-3 progresses, E1 MT-SDT info is only about data size, whereas F1 MT-SDT info is just an indicator.

Observation 2: In 38.401 BL CR, it is currently described as if the same MT-SDT information from CU-UP is forwarded to DU (via CU-CP), but according to our stage-3 progresses, E1 MT-SDT info is only about data size, whereas F1 MT-SDT info is just an indicator.

To avoid confusions and correctly reflect stage-3, the wording “MT-SDT information” in steps 3 and 4 should be clarified as just an “indicator”.

Proposal 3: In 38.401 BL CR, correct the wording “MT-SDT information” in steps 3 and 4 as “MT-SDT indication”.

The corresponding TP for 38.300 BL CR [1] and 38.401 BL CR [2] can be found in Section 5 and Section 6, respectively.

# Conclusion

In the present contribution we make the following observations:

Observation 1: In 38.300 BL CR, between step 2 and step 3, RAN3 agreed that it is possible that a neighborikng gNB who receives XnAP RAN Paging including MT-SDT information may decide to trigger legacy paging instead.

Observation 2: In 38.401 BL CR, it is currently described as if the same MT-SDT information from CU-UP is forwarded to DU (via CU-CP), but according to our stage-3 progresses, E1 MT-SDT info is only about data size, whereas F1 MT-SDT info is just an indicator.

Based on the discussion in the present contribution and the observations above we propose:

Proposal 1: In 38.300 BL CR, the step 6/7 should be merged and refer the MO-SDT procedure from step 2, in order to correctly reflect the agreement to re-use the existing SDT Support Request IE by the receiving gNB in indicating MT-SDT initiation from the UE to the last serving gNB during XnAP Retrieve UE Context Retrieval procedure.

Proposal 2: In 38.300 BL CR, a NOTE should be captured that, if the receiving gNB decided not to trigger MT-SDT paging, the step 3 and afterwards follows the legacy procedure, not SDT procedures.

Proposal 3: In 38.401 BL CR, correct the wording “MT-SDT information” in steps 3 and 4 as “MT-SDT indication”.

The corresponding TP for 38.300 BL CR [1] and 38.401 BL CR [2] can be found in Section 5 and Section 6, respectively.

# Reference

[1] R3-23378, “(BLCR to 38.300) Introduction on MT-SDT”, ZTE, CATT, Ericsson, China Mobile, China Telecom, Nokia, Nokia Shanghai Bell, Lenovo, Huawei, Google, LG Electronics

[2] R3-233744, “(BLCR to 38.401) Introduction on MT-SDT”, Huawei, ZTE, Nokia, Nokia Shanghai Bell, Ericsson, Intel Corporation, China Telecom, Lenovo, LG Electronics

# TP for NR_MT_SDT for TS 38.300

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

## 18.X	MT-SDT with/without UE context relocation

The overall procedure for MT-SDT procedure with/without UE context relocation is illustrated in the figure 18.X-1.

Figure 18.X-1. MT-SDT with/without UE context relocation

1.  DL user data and/or DL NAS signalling are received at the Last Serving gNB for the UE in RRC_INACTIVE state.

2.  The Last Serving gNB may send MT-SDT information to the neighbour gNBs within the RNA, via XnAP RAN PAGING message.

3.  The gNB that receives MT-SDT information within the RNA, takes into account the MT-SDT information received in the XnAP RAN PAGING message to decide whether to trigger MT-SDT Paging. The gNB which ultimately reaches the UE via the Uu Paging becomes the Receiving gNB.

NOTE: In case that the Receiving gNB decided not to trigger MT-SDT paging, the above step 3 and subsequent steps follow the same as in Figure 9.2.2.4.2 from step 3.

4/5.  The UE may decide to initiate MT-SDT procedure and send an RRCResumeRequest message for MT-SDT to the Receiving gNB.

Editor note: The above step 3/4/5 can be revisited by RAN2 progress.

6/7.  The following steps are the same as Figure 18.2-1/18.3-1 from step 2, except that the first SDT user data and/or NAS signalling is DL SDT data and/or DL SDT NAS signalling.

NOTE: In case DL non-SDT data or DL non-SDT signalling has arrived between step 2 and step 7, the Last Serving gNB should relocate the UE context to the Receiving gNB and forward the received data to the Receiving gNB.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

# TP for NR_MT_SDT for TS 38.401

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.18.x	MT-SDT

The procedure for mobile terminated small data transmission in RRC Inactive is shown in Figure 8.18.x-1.

Figure 8.18.x-1: Mobile Terminated Small Data Transmission in RRC Inactive state.

1. During the setup or modification of the bearer context as specified in 8.9.2, the gNB-CU-CP requests the gNB-CU-UP to provide MT-SDT information.

2a-0.	The gNB-CU-UP receives DL data for the UE in RRC Inactive on NG-U interface.

2a-1.	The gNB-CU-UP sends DL DATA NOTIFICATION message to the gNB-CU-CP. If determining that DL data packets are only mapped to SDT bearers, as requested in step 1, the gNB-CU-UP includes the MT-SDT information in the DL DATA NOTIFICATION message.

2b. The gNB-CU-CP receives DL NAS signalling over NGAP.

3.	After 2a or 2b, the gNB-CU-CP sends PAGING message to the gNB-DU. The MT-SDT indication may be included in the PAGING message.

4.	The gNB-DU sends the Paging message to the UE. In case the MT-SDT indication is received in step 3, the gNB-DU includes the MT-SDT indicator in the Paging message.

5. If the UE has been successfully reached, it initiates the RRC connection resume procedure as described in 8.6.2 or 8.9.6.2, or initiates the SDT procedure as described in 8.18.1 or from step 9 in 8.18.2 or from step 1 in 8.18.3 with the following difference:

- In case SDT procedure is initiated, the UE may indicate the RRC Resume Request is for MT-SDT, which may be without UL data.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////