3GPP TSG-RAN WG3 Meeting #121	R3-234461

Toulouse, France, 21 – 25 August 2023

Agenda Item:	14.4

Source:	ZTE

Title:	(TP for TS 38.423 BL CR) Discussion on support of subsequent CPAC

Document for:	Discussions & Approval

- Introduction

- We made the following progress at the RAN3#120 meeting.

Add a new IE as a sub IE of the Conditional PSCell Change Information Required IE in the S-NODE CHANGE REQUIRED message to indicate that the request is for SCG Selective Activation. It is FFS (up to RAN2 progress) what form the indication will have (an explicit IE or an RRC container). Note: This can be revisited based on RAN2 progress. Add a new IE as a sub IE of the Conditional PSCell Addition Information Request IE in the S-NODE ADDITION REQUEST message to indicate that the request is for SCG Selective Activation. It is FFS (up to RAN2 progress) what form the indication will have (an explicit IE or an RRC container). Note: This can be revisited based on RAN2 progress. Introduce a new indicator for selective activation in the S-NODE MODIFICATION REQUEST message to indicate that the request is for SCG selective activation. FFS on the scenarios. Introduce a new indicator for selective activation in the Xn-U address indication message to indicate that the request is for SCG selective activation. FFS on the scenarios.

This paper further discusses how to support the subsequent CPAC feature.

# 2	Discussion

Since RAN2 has not decided whether the reference SCG configuration is optionally provided to the candidate SN(s), RAN3 should wait for RAN2 progress before further discussing the details of the subsequent CPAC indication introduced in the SN change required message or the SN addition request message.

Proposal 1: RAN3 should wait for RAN2 progress regarding the optionality of the reference SCG configuration before further discussing the details of the subsequent CPAC indication introduced in the SN change required message or the SN addition request message.

At the last meeting, it is still FFS whether to introduce a new indicator for subsequent CPAC in the SN modification request message to indicate that the request is for subsequent CPAC. From our point of view, it would apply to the scenario when the MN decides to configure subsequent CPAC before the execution of CPAC.

Observation 1: Introducing a new indicator for subsequent CPAC would be beneficial for the scenario when the MN decides to configure subsequent CPAC before the execution of CPAC.

Proposal 2: Introduce a new indicator for subsequent CPAC in the SN modification request message to indicate that the request is for subsequent CPAC.

It is noted that RAN2 has the following agreement:

- It is assumed that if the UE need to be able to return to a current SCG by conditional procedure, then the network could explicitly configure a candidate configuration for that cell.

To keep align with the above assumption, it is proposed to convert the following WA into agreement.

WA (up to RAN2’s discussion): RAN3 assumes the last serving (source) PSCell may remain prepared within the prepared cells for Selective Activation.

Proposal 3: RAN3 assumes the last serving (source) PSCell may remain prepared within the prepared cells for Selective Activation. This can be revisited by RAN2 progress.

At the last meeting, we have agreed as below.

For inter-SN SCG selective activation, after CPC execution, the MN needs to notify the source SN and the selected SN of the cell change. FFS how to notify the source SN and the selected SN.

It is still FFS on how to notify the source SN and the selected SN after CPC execution. If the source SN is not configured as a candidate SN, the MN should use the SN release request message to notify the source SN and release it. If the source SN is configured as a candidate SN, the MN should use the Xn-U address indication message to inform the source SN of the CPC execution and start late data forwarding. In addition, a new indicator is also needed to be introduced in the Xn-U address indication message to indicate the CPC execution.

Proposal 4: If the source SN is not configured as a candidate SN, the MN should use the SN release request message to notify the source SN and release it. If the source SN is configured as a candidate SN, the MN should use the Xn-U address indication message to inform the source SN of the CPC execution and start late data forwarding.

Proposal 5: Introduce a new indicator for subsequent CPACP in the Xn-U address indication message to notify the source SN about execution.

As for notifying the selected SN, SN Reconfiguration complete message can be reused.

Proposal 6: The SN reconfiguration complete message can be reused to notify the selected SN of the cell change after CPC execution.

We have agreed to reuse the Xn-U Address Indication message and the Early Status Transfer message to support early data forwarding for SCG Selective Activation. It is still FFS on whether these message needs enhancements and when to use them. From our point of view, the early data forwarding for SCG selective activation should reuse the principle for CPAC. Namely, the source SN forwards the DL user data to the candidate SN via the MN. To achieve this, after receiving the RRCReconfigurationComplete message from the UE, the MN should send the Xn-U Address Indication message to the source SN to provide the data forwarding addresses, the source SN should send the Early Status Transfer message to the MN and the MN should send the Early Status Transfer message to the target SN to start early data forwarding.

Proposal 7: After receiving the RRCReconfigurationComplete message from the UE, the MN should send the Xn-U Address Indication message to the source SN to provide the data forwarding addresses, the source SN should send the Early Status Transfer message to the MN and the MN should send the Early Status Transfer message to the target SN to start early data forwarding.

Some companies proposed to reuse Early Status Transfer message to indicate the candidate SN to discard the already forwarded data for SCG selective activation after CPC execution. From our point of view, the discarding of the forwarded data can be up to implementation when the candidate SN receives the Early Status Transfer message when the second round of early data forwarding is started during subsequent CPC.

Proposal 8: It is up to implementation to discard of the forwarded data for the first round of early data forwarding when the candidate SN receives the Early Status Transfer message when the second round of early data forwarding is started during subsequent CPAC.

As for how to indicate whether the associated the SCG configuration is a delta with respect to the reference SCG configuration in the SN addition request acknowledge message, some companies propose to reuse the existing RRC config indication IE. The existing RRC config indication IE is a per node indication other than per cell, therefore it can not support a per cell indication as specified in the RAN2 agreement.

- R2 understands that A target SN may include an indication in SN Addition Request Ack for each candidate target PSCell, denoting whether the associated SCG configuration is a delta with respect to the reference SCG configuration.

Observation 2: The existing RRC config indication IE is a per node indication other than per cell, therefore it can not support a per cell indication as specified in the RAN2 agreement.

This IE can be reused but needs to be added as a sub IE of the Conditional PSCell Addition Information Acknowledge IE in the SN addition request acknowledge message to achieve a per cell indication.

Proposal 9: Add the RRC config indication IE as a sub IE of the Conditional PSCell Addition Information Acknowledge IE in the SN addition request acknowledge message to indicate whether the associated SCG configuration is a delta with respect to the reference SCG configuration. This can be revisited based on RAN2 progress.

At the last meeting, some companies propose to also introduce new indicators in the UE context setup/modification message and bearer context setup request message. However, from our point of view, the subsequent CPAC feature can still work without introducing new indicators in these messages.

Proposal 10: No need to introduce new indicators in UE context setup message, UE context modification message and bearer context setup request message

# 3	Conclusion

Based on above analysis, we provide the following observations and proposals.

Proposal 1: RAN3 should wait for RAN2 progress regarding the optionality of the reference SCG configuration before further discussing the details of the subsequent CPAC indication introduced in the SN change required message or the SN addition request message.

Observation 1: Introducing a new indicator for subsequent CPAC would be beneficial for the scenario when the MN decides to configure subsequent CPAC before the execution of CPAC.

Proposal 2: Introduce a new indicator for subsequent CPAC in the SN modification request message to indicate that the request is for subsequent CPAC.

Proposal 3: RAN3 assumes the last serving (source) PSCell may remain prepared within the prepared cells for Selective Activation. This can be revisited by RAN2 progress.

Proposal 4: If the source SN is not configured as a candidate SN, the MN should use the SN release request message to notify the source SN and release it. If the source SN is configured as a candidate SN, the MN should use the Xn-U address indication message to inform the source SN of the CPC execution and start late data forwarding.

Proposal 5: Introduce a new indicator for subsequent CPACP in the Xn-U address indication message to notify the source SN about execution.

Proposal 6: The SN reconfiguration complete message can be reused to notify the selected SN of the cell change after CPC execution.

Proposal 7: After receiving the RRCReconfigurationComplete message from the UE, the MN should send the Xn-U Address Indication message to the source SN to provide the data forwarding addresses, the source SN should send the Early Status Transfer message to the MN and the MN should send the Early Status Transfer message to the target SN to start early data forwarding.

Proposal 8: It is up to implementation to discard of the forwarded data for the first round of early data forwarding when the candidate SN receives the Early Status Transfer message when the second round of early data forwarding is started during subsequent CPAC.

Observation 2: The existing RRC config indication IE is a per node indication other than per cell, therefore it can not support a per cell indication as specified in the RAN2 agreement.

Proposal 9: Add the RRC config indication IE as a sub IE of the Conditional PSCell Addition Information Acknowledge IE in the SN addition request acknowledge message to indicate whether the associated SCG configuration is a delta with respect to the reference SCG configuration. This can be revisited based on RAN2 progress.

Proposal 10: No need to introduce new indicators in UE context setup message, UE context modification message and bearer context setup request message

4	Text Proposal for BL CR TS 38.423

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| ////////////// Skip unchanged part ///////////////// |  |  |  |  |  |  |
| Conditional PSCell Addition Information Request | O |  |  |  | YES | reject |
| >Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, ...) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| >Subsequent CPAC Information Request | O |  | 9.2.3.xxx | This information is used for subsequent CPAC. FFS on the details. Editor’s Note: This may be revisited based on RAN2 progress. | YES | ignore |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  | UE Slice Maximum Bit Rate List 9.2.3.167 | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] | YES | reject |
| F1-terminating IAB-donor Indicator | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSNterminated | This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to either request the preparation to modify S-NG-RAN node resources for a specific UE, or to query for the current SCG configuration, or to provide the S-RLF-related information to the S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| PDCP Change Indication | O |  | 9.2.3.74 |  | YES | ignore |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  | YES | ignore |
| SCG Configuration Query | O |  | 9.2.3.27 |  | YES | ignore |
| UE Context Information |  | 0..1 |  |  | YES | reject |
| >UE Security Capabilities | O |  | 9.2.3.49 |  | – |  |
| >S-NG-RAN node Security Key | O |  | 9.2.3.51 |  | – |  |
| >S-NG-RAN node UE Aggregate Maximum Bit Rate | O |  | UE Aggregate Maximum Bit Rate 9.2.3.17 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >Lower Layer presence status change | O |  | 9.2.3.60 |  | – |  |
| >PDU Session Resources To Be Added List |  | 0..1 |  |  | – |  |
| >>PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE is present in a PDU Session Resources To Be Added Item IE, abnormal conditions as specified in clause 8.3.3.4 apply. | – |  |
| >>>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  | – |  |
| >>>PDU Session Resource Setup Info – SN terminated | O |  | 9.2.1.5 |  | – |  |
| >>>PDU Session Resource Setup Info – MN terminated | O |  | 9.2.1.7 |  | – |  |
| >>>PDU Session Expected UE Activity Behaviour | O |  | Expected UE Activity Behaviour 9.2.3.82 | Expected UE Activity Behaviour for the PDU Session. | YES | ignore |
| >PDU Session Resources To Be Modified List |  | 0..1 |  |  | – |  |
| >>PDU Session Resources To Be Modified Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Modification Info – SN terminated IE nor the PDU Session Resource Modification Info – MN terminated IE is present in a PDU Session Resources To Be Modified Item IE, abnormal conditions as specified in clause 8.3.3.4 apply. | – |  |
| >>>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  | – |  |
| >>>PDU Session Resource Modification Info – SN terminated | O |  | 9.2.1.9 |  | – |  |
| >>>PDU Session Resource Modification Info – MN terminated | O |  | 9.2.1.11 |  | – |  |
| >>>S-NSSAI | O |  | 9.2.3.21 |  | YES | reject |
| >>>PDU Session Expected UE Activity Behaviour | O |  | Expected UE Activity Behaviour 9.2.3.82 | Expected UE Activity Behaviour for the PDU Session. | YES | ignore |
| >PDU Session Resources To Be Released List | O |  | PDU session List with Cause 9.2.1.26 |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2. of TS 38.331 [10]. | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. | YES | ignore |
| Requested Split SRBs release | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested to be released. | YES | ignore |
| Desired Activity Notification Level | O |  | 9.2.3.77 |  | YES | ignore |
| Additional DRB IDs | O |  | DRB List 9.2.1.29 | Indicates additional list of DRB IDs that the S-NG-RAN node may use for SN-terminated bearers. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Uplink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Uplink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Uplink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. If the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE is not present, this IE applies to both UL and DL. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Downlink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Downlink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Downlink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. | YES | reject |
| Location Information at S-NODE reporting | O |  | ENUMERATED (pscell, ...) | Indicates that the user’s Location Information at S-NODE is to be provided. | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| PCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| NE-DC TDM Pattern | O |  | 9.2.2.38 |  | YES | ignore |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |
| Requested Fast MCG recovery via SRB3 Release | O |  | ENUMERATED (true, ...) | Indicates that resources for fast MCG recovery via SRB3 are requested to be released. | YES | ignore |
| SN triggered | O |  | ENUMERATED (TRUE ...) |  | YES | ignore |
| Target Node ID | O |  | Global NG-RAN Node ID 9.2.2.3 | Indicates the target node ID of the handover procedure decided by the M-NG-RAN node. | YES | ignore |
| PSCell History Information Retrieve | O |  | ENUMERATED (query, ...) | Indicates that the SN UE history information is requested. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| CHO Information SN Modification | O |  |  |  | YES | ignore |
| >Conditional Reconfiguration | M |  | ENUMERATED (intra-MN-CHO, ...) |  | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| SCG Activation Request | O |  | 9.2.3.154 |  | YES | ignore |
| Conditional PSCell Addition Information Modification Request | O |  |  | This IE may be sent to the target SN. | YES | ignore |
| >Maximum Number of PSCells To Prepare | O |  | INTEGER (1..8, ...) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. |  |  |
| >Subsequent CPAC Information Request | O |  | 9.2.3.xxx | This information is used for subsequent CPAC. FFS on the details. Editor’s Note: This may be revisited based on RAN2 progress. |  |  |
| Conditional PSCell Change Information Update | O |  |  | This IE may be sent to the source SN. | YES | ignore |
| >Multiple Target S-NG-RAN Node List |  | 1 |  |  | – |  |
| >>Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  | – |  |
| >>>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >>>Candidate PSCell List |  | 1 |  |  | – |  |
| >>>>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  | UE Slice Maximum Bit Rate List 9.2.3.167 | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] | YES | ignore |
| Management Based MDT PLMN Modification List | O |  | MDT PLMN Modification List 9.2.3.169 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCell candidates. Value is 8 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.2.11	S-NODE CHANGE REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to trigger the change of the S-NG-RAN node.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| ////////////// Skip unchanged part ///////////////// |  |  |  |  |  |  |
| Conditional PSCell Change Information Required | O |  |  |  | YES | ignore |
| >Multiple Target S-NG-RAN Node List |  | 1 |  |  | – |  |
| >>Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  | – |  |
| >>>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >>>CPC Indicator | M |  | ENUMERATED (CPC-initiation, CPC-modification, CPC-cancellation, ...) |  | – |  |
| >>>Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, …) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >>>Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| >>>S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. |  |  |
| >Subsequent CPAC Information Required | O |  | FFS | This information is used for Subsequent CPAC. FFS on the details. Editor’s Note: This may be revisited based on RAN2 progress. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |

<<<<<<<<<<<<<<<<<<<< Last Change >>>>>>>>>>>>>>>>>>>>

9.2.3.xxx	Subsequent CPAC Information Request

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |

Editor’s note: The name and the encoding of the Subsequent CPAC Information Request IE are FFS

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>