3GPP TSG-RAN WG3 Meeting #121	R3-234606

Toulouse, France, 21 – 25 Aug 2023

Agenda Item:	11.2

Source:	ZTE

Title:	Summary of Offline Discussion on QoE in NR-DC

Document for:	Discussion

# Introduction

This is the offline summary for AI11.3, QoE in NR-DC

# For the Chairman’s Notes

P1: Reuse legacy procedures for the MN-SN coordination in NR-DC to support QoE.

P2: For s-based QoE configuration received by MN

- MN sends the QoE configuration via SRB1

- QoE reports can be sent via SRB4 or SRB5

P3: For management based QoE, there is no need for SN to explicitly indicate to the MN whether it wants to configure the UE directly via SRB3 or via a container and SRB1.

P4: Modify the previous agreement as follows:

In case the SN is interested in configuring a UE with an m-based QoE measurement configuration, the MN can decide and notify the SN whether:

- The MN shall send the configuration information to the UE, or

- The SN should send the configuration to the UE directly, or

- The SN should send the configuration information to the UE via the MN (inside a container)

P5: WA: The transparent reporting for RVQoE over RRC is not supported.

P6: If the MN configures RVQoE, the first RVQoE report shall be delivered to the MN via SRB4.

P7: If the SN configures RVQoE, the first RVQoE report shall be delivered to the SN.

To be continued:

If MN receives m-based QoE configuration from OAM, MN may send the QoE configuration to the selected UEs via SRB1

Does the SN need to include the QoE configuration container in the request message to the MN?

FFS how the SRB5 could be established

Issues not discussed during offline:

Define two different reporting leg indications for QoE and RVQoE?

FFS whether, in a UE in NR-DC, each QoE configuration can have more than one corresponding RVQoE configuration.

The node which sends the first RVQoE configuration to UE is the same node which sends the legacy QoE configuration to UE.

Could the non-RVQoE configuring node directly configures RVQoE to UE, if it is the node that provides bearers for application session?

RRC Transfer procedure can be considered to transfer the RVQoE report between MN and SN.