3GPP TSG-RAN WG3 Meeting #121	R3-234502

Toulouse, France, 21st – 25th Aug, 2023

Agenda Item:	9.1

Source:	ZTE(Moderator)

Title:	Summary of Offline Discussion on CB: # 4_QoEAreaScope

Document for:	Approval

Introduction

CB: # 4_QoEAreaScope

- Check whether there is any stage2 impact?

- Check whether S1AP CRs are needed?

- Check BC way is feasible/preferable or not?

- The order of PLMN ID and TAC?

(moderator - ZTE)

Summary of offline disc R3-234502

For the Chairman’s Notes

Proposal:

Further discuss whether the presence of Area Scope IE should be optional in S1AP and/or X2AP. If yes, consider how to make the change, e.g., any BC way change.

To be continued...

R3-234644 Correction on QoE area scope tabular_36423_r15 agreed

R3-234645 Correction on QoE area scope tabular_36423_r16 agreed

R3-234647 Correction on QoE area scope tabular_36423_r17 agreed

Discussion

Misalignment issue between LTE and NR

During the online session, companies provided their views on whether LTE(including S1AP and X2AP) corrections on the presence of Area Scope IE in LTE QoE are necessary and made no consensus. In this sub-section, moderator suggests companies to provide their understanding on whether the misalignment of the presence of Area Scope IE between LTE QoE and NR QoE has LTE stage2 impact?

Q1: Whether the misalignment issue on the presence of Area Scope IE between LTE QoE and NR QoE has LTE stage2 impact?

| Company | Comment |
| --- | --- |
| Huawei | Needs clarification on what the stage 2 change targeting on. |
| Ericsson | What LTE stage-2 impact do you mean? |

Q2: Please provide your views on whether RAN3  shall correct the presence of the Area Scope IE from M to O in S1AP and/or X2AP? If not, how to fix this misalignment issue, e.g., any BC way correction?

| Company | Comment |
| --- | --- |
| Huawei | Needs more checking and prefer BC way if possible. We currently see three main arguments for changing the presence, which are: - Misalignment between RAN3 and SA5 specification - Handover case, area scope is not mandated - Locationfilter and area scope not exist at the same time. For 1, as we have explained, there do has a misalignment between RAN3 and SA5, but it is not the reason for such change, as in RAN3, we already include PLMN as one type of area scope. For 2, we would like to note some differences in NR and LTE. For both NR and LTE, area scope is used when initiating one QMC measurement, but not later. A UE should continue an ongoing measurement even if it leaves the Area Scope, unless the network indicates to the UE to release the QoE configuration. And the reason to release the QoE configuration is not because of UE leaving the area scope, but OAM ask RAN to release the measurement due to some other reasons. Then one discrepancy in LTE and NR is that in LTE, there is no QoE start indication, so RAN and CN has no idea whether one QoE has started or not. This means, in NR, target has no usage of area scope, as one already started QoE measurement will anyway continue. But in LTE, if target finds UE is out of area scope, then it is up to RAN implementation on whether to release the QoE. So LTE needs the area scope. For 3, this is a valid point, but we would like first reach a consensus in R18 regarding this point. Also one possible solution to avoid NBC change is adding a description that ‘in case of locationfitler is used, list all PLMNs as area scope’, then it means the area scope is actually deactivated. |
| CATT | No change needed |
| Ericsson | OK to change the presence, but there shall be no mentioning of LocationFilter as the reason for change, since the status of LocationFilter is not yet been settled. |

TAI issue in Area Scope IE in QoE

We also briefly touched the TAI issue in the current Area Scope IE in both NR and LTE during the online session. More specifically, there is no tabular for TAI in TS 36.423. The wrong reference shall be corrected. Companies shared two different views on how to correct this issue.

Option1: Introduce TAC + PLMN ID instead of TAI

In contribution [2], [3], and [4], the TAI in tabular has been changed to the PLMN ID and TAC. Detail can check the screenshot below:

Option2: Introduce a new section and tabular for TAI

Add a new subsection which contains a new tabular for TAI(example is shown below). Then, link the reference of the TAI in Area Scope of QMC to this new added section.

- 9.2.2.X TAI This IE is used to uniquely identify a Tracking Area. IE/Group Name; Presence; Range; IE type and reference; Semantics description. PLMN Identity; M; 9.3.3.5. TAC; M; 9.3.3.10.

Generally, either of these options is a BC way to fix the TAI issue without any ASN.1 modification. The reason why option 1 is finally selected by some companies is because of the existing tabular design in MDT field. It is clear that the tabular and ASN.1 design of Area Scope of QMC considers the Area Scope of MDT as baseline. The current ASN.1 structure of the Area Scope of MDT in TS 38.423 is shown below:

The current ASN.1 mapping between MDT and QoE can also be found in Annex part in this report.

Q3:Companies may provide views on how to fix the TAI issue in Area Scope of QMC in TS36.423?

Option1: Introduce TAC + PLMN ID instead of TAI

Option2: Introduce a new section and tabular for TAI

| Company | Option1, option2, or any other view | Comment |
| --- | --- | --- |
| Huawei | Both are fine | The benefit of option 2 is more future proof, but we already agreed option 1 for XnAP, so probably do the same here. No strong view. |
| CATT | Option 1 | Should Align with ASN.1 |
| Ericsson | 1 |  |

Besides, extra reference issue exists in the Area Scope of QMC in TS 36.423. As shown in [2], [3], [4], the reference of the E-CGI, TAC, TAI, PLMN Identity are referred to the related sections in TS 36.413 and shall also be fixed.

Q4: Please provide your views on whether to fix the wrong reference of E-CGI, TAC, PLMN Identity in Area Scope of QMC in TS 36.423.

| Company | Comment |
| --- | --- |
| Huawei | Definitely needed |
| CATT | OK |
| Ericsson | OK |

Any other issue

Companies may provide any other issues in this section.

Q3. Please provide any other views if any.

| Company | Comment |
| --- | --- |

References

- R3-234471	Further discussion on the presence of QoE area scope (ZTE, China Telecom, China Unicom, Ericsson)	discussion

- R3-234472	Correction on QoE area scope tabular reference and presence (ZTE, Huawei, China Telecom, China Unicom, Ericsson)	CR1758r, TS 36.423 v15.13.0, Rel-15, Cat. F

- R3-234473	Correction on QoE area scope tabular reference and presence (ZTE, Huawei, China Telecom, China Unicom, Ericsson)	CR1759r, TS 36.423 v16.11.0, Rel-16, Cat. A

- R3-234474	Correction on QoE area scope tabular reference and presence (ZTE, Huawei, China Telecom, China Unicom, Ericsson)	CR1760r, TS 36.423 v17.5.0, Rel-17, Cat. A

- R3-234475	Correction on presence of the QoE Area Scope IE (ZTE, China Telecom, China Unicom, Ericsson)	CR1917r, TS 36.413 v15.11.0, Rel-15, Cat. F

- R3-234476	Correction on presence of the QoE Area Scope IE (ZTE, China Telecom, China Unicom, Ericsson)	CR1918r, TS 36.413 v16.13.0, Rel-16, Cat. A

- R3-234477	Correction on presence of the QoE Area Scope IE (ZTE, China Telecom, China Unicom, Ericsson)	CR1919r, TS 36.413 v17.5.0, Rel-17, Cat. A

- Annex A  ASN.1 in TS 38.423

#### -- T

TABasedMDT ::= SEQUENCE {

tAListforMDT		TAListforMDT,

iE-Extensions		ProtocolExtensionContainer { {TABasedMDT-ExtIEs} } OPTIONAL,

...

}

TABasedMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TAIBasedMDT ::= SEQUENCE {

tAIListforMDT			TAIListforMDT,

iE-Extensions			ProtocolExtensionContainer { {TAIBasedMDT-ExtIEs} } OPTIONAL,

...

}

TAIBasedMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TAIListforMDT ::= SEQUENCE (SIZE(1..maxnoofTAforMDT)) OF TAIforMDT-Item

TAIforMDT-Item ::= SEQUENCE {

plmn-ID				PLMN-Identity,

tAC						TAC,

iE-Extensions			ProtocolExtensionContainer { {TAIforMDT-Item-ExtIEs} } OPTIONAL,

...

}

TAIforMDT-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TAC ::= OCTET STRING (SIZE (3))

TAINSAGSupportList ::= SEQUENCE (SIZE(1..maxnoofNSAGs)) OF TAINSAGSupportItem

TAINSAGSupportItem ::= SEQUENCE {

nSAG-ID								NSAG-ID,

nSAGSliceSupportList			ExtendedSliceSupportList,

iE-Extensions		ProtocolExtensionContainer { {TAINSAGSupportItem-ExtIEs} } OPTIONAL,

...

}

TAINSAGSupportItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TAISupport-List	::= SEQUENCE (SIZE(1..maxnoofsupportedTACs)) OF TAISupport-Item

TAISupport-Item ::= SEQUENCE {

tac								TAC,

broadcastPLMNs					SEQUENCE (SIZE(1..maxnoofsupportedPLMNs)) OF BroadcastPLMNinTAISupport-Item,

iE-Extensions					ProtocolExtensionContainer { {TAISupport-Item-ExtIEs} } OPTIONAL,

...

}

TAISupport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TAListforMDT ::= SEQUENCE (SIZE(1..maxnoofTAforMDT)) OF TAC

TABasedQMC ::= SEQUENCE {

tAListforQMC		TAListforQMC,

iE-Extensions		ProtocolExtensionContainer { {TABasedQMC-ExtIEs} } OPTIONAL,

...

}

TABasedQMC-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TAListforQMC ::= SEQUENCE (SIZE(1..maxnoofTAforQMC)) OF TAC

TAIBasedQMC ::= SEQUENCE {

tAIListforQMC		TAIListforQMC,

iE-Extensions		ProtocolExtensionContainer { {TAIBasedQMC-ExtIEs} } OPTIONAL,

...

}

TAIBasedQMC-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TAIListforQMC ::= SEQUENCE (SIZE(1..maxnoofTAforQMC)) OF TAI-Item

TAI-Item ::= SEQUENCE {

tAC					TAC,

pLMN-Identity		PLMN-Identity,

iE-Extensions		ProtocolExtensionContainer { {TAI-Item-ExtIEs} } OPTIONAL,

...

}

TAI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

- Annex B ASN.1 in TS 36.423

#### -- T

TABasedMDT::= SEQUENCE {

tAListforMDT		TAListforMDT,

iE-Extensions		ProtocolExtensionContainer { {TABasedMDT-ExtIEs} } OPTIONAL,

...

}

TABasedMDT-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TAC ::= OCTET STRING (SIZE (2))

TAIBasedMDT ::= SEQUENCE {

tAIListforMDT			TAIListforMDT,

iE-Extensions			ProtocolExtensionContainer { {TAIBasedMDT-ExtIEs} } OPTIONAL,

...

}

TAIBasedMDT-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TAIListforMDT ::= SEQUENCE (SIZE(1..maxnoofTAforMDT)) OF TAI-Item

TAI-Item ::= SEQUENCE {

tAC					TAC,

pLMN-Identity		PLMN-Identity,

iE-Extensions		ProtocolExtensionContainer { {TAI-Item-ExtIEs} } OPTIONAL,

...

}

TAI-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TAListforMDT ::= SEQUENCE (SIZE(1..maxnoofTAforMDT)) OF TAC

TABasedQMC ::= SEQUENCE {

tAListforQMC		TAListforQMC,

iE-Extensions		ProtocolExtensionContainer { {TABasedQMC-ExtIEs} } OPTIONAL,

...

}

TABasedQMC-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TAListforQMC ::= SEQUENCE (SIZE(1..maxnoofTAforQMC)) OF TAC

TAIBasedQMC ::= SEQUENCE {

tAIListforQMC		TAIListforQMC,

iE-Extensions		ProtocolExtensionContainer { {TAIBasedQMC-ExtIEs} } OPTIONAL,

...

}

TAIBasedQMC-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TAIListforQMC ::= SEQUENCE (SIZE(1..maxnoofTAforQMC)) OF TAI-Item