3GPP TSG-RAN WG3 #121	R3-234531

Toulouse France, 21-25 August 2023

Agenda Item:	9.2

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on CB: # 8_InactiveTimeOver E1

Document for:	Approval

# Introduction

CB: # 8_InactiveTimeOver E1

- Check the comments above

- Identify the issue

(moderator - Nok)

Summary of offline disc R3-234531

# For the Chairman’s Notes

Based on offline discussion and feedback with involved companies, the following is proposed as way forward:

- Only introduce changes at Source gNB for inactive time reporting

- Changes:

  - Limit reporting from gNB-CU-UP to scenario where inactivity level is UE Level inactivity

  - Granularity of new IE to be in seconds (as in the Inactive Time setting)

- Bring correction from Rel-15 (Cat F), with Rel-16/17 (Cat A)

  - Draft CR to be derived in meeting

# Discussion

Contribution R3-233839 discussed online at RAN3#121 highlighted the issue that in handover scenarios, a gNB is able to over Xn, forward the “inactive” time spent for the UE to the target gNB, which is part of RRC: HandoverPreprationInformation within rrm-Config, which can include ue-InactiveTime IE. However, in case of CP-UP separation there following issues were highlighted:

- (1) Source gNB-CU-CP, is not aware of the “inactive” time as it is measured at gNB-CU-UP and not reported with E1AP. Hence, as consequence cannot signal inactive-time over Xn.

- (2) Target gNB-CU-CP, is not able to take the inactive-time and signal it to gNB-CU-UP even if received over Xn.

The following remarks were made during the online discussion regarding the solution proposed in R3-233839. The moderator proposes to break the discussion in two (a) Source gNB-CU-CP scenario, and (b) Target gNB-CU-CP scenario.

- (1) Source gNB

  - Companies acknowledged the issue and gap in the E1AP specification

  - Whether solution should be limited to UE level inactivity, as since “ue inactive-time” over Xn only applies at UE level inactivity and not DRB or PDU inactivity

  - Whether the IE format proposed in the CR need to be revised

- (2) Target gNB

  - Different views on need for specification changes on E1AP

  - Whether gNB-CU-CP at target can determine how to utilize the received value over Xn without involving gNB-CU-UP at target

## Companies to provide valuable comments for the possible solutions

(1) Questions regarding Source gNB

Q1: Should signaling over E1 at Source gNB be limited to UE inactivity reporting (and not per DRB or PDU inactivity)?

| Company | Yes/No | Comment |
| --- | --- | --- |

Moderator Summary :

Q2: Any comments regarding whether “inactive time” granularity should match same structure as the “ue inactive-time” IE defined in 38.331 RRC specification?

| Company | Yes/No | Comment |
| --- | --- | --- |

Moderator Summary :

Q3: Any other comments/remarks?

| Company | Comment |
| --- | --- |

Moderator Summary :

(2) Questions regarding Target gNB

Q4: Does the Target gNB-CU-CP, when establishing bearer contexts, need to inform gNB-CU-UP the inactive time spent at the Source gNB?

| Company | Yes/No | Comment |
| --- | --- | --- |

Moderator Summary :

Q5: Any other comments/remarks?

| Company | Comment |
| --- | --- |

Moderator Summary :

# Conclusion, Recommendations [if needed]

Based on offline discussion and feedback with involved companies, the following is proposed as way forward:

- Only introduce changes at Source gNB

- Changes:

  - Limit reporting from gNB-CU-UP to scenario where inactivity level is UE Level inactivity

  - Granularity of new IE to be in seconds (as in the Inactive Time setting)

- Bring correction from Rel-15 (Cat F), with Rel-16/17 (Cat A)

  - Draft CR to be derived in meeting

# References

| R3-233839 | Inactive Time Signaling over E1 for Mobility (Nokia, Nokia Shanghai Bell) | CR0068r, TS 37.483 v17.5.0, Rel-17, Cat. F |
| --- | --- | --- |