3GPP TSG-RAN WG3 Meeting #121	R3-234559

# Toulouse, France, August 21st – 25th 2023

Agenda Item:	11.4.

Source:	Ericsson (moderator)

Title:	Offline discussion on QoE AI 11.4.

Document for:	Approval

# For the Chairman notes

Proposal 1-1: RVQoE reporting over F1 can be deactivated per UE.

Proposal 1-2: Decide at the RAN3#121-bis meeting whether the F1AP procedure for deactivation of RVQoE reporting is UE-associated or non-UE-associated.

Proposal 2: QoE measurement continuity from NR to LTE/5GC and vice versa can be supported by the current Area Scope for QMC IE.

Discuss whether and how a source NR node for handover from NR to LTE/5GC can determine for which service type(s) the UE supports QoE measurements in LTE/5GC.

Proposal 3: The scenario where a UE is handed over from NR to LTE/5GC and then back to NR is not considered in the context of continuity of QoE measurements during intra-5GC inter-RAT HO.

Proposal 4: HO from LTE/5GC to NR can be supported without RAN3 impact.

# Company replies

Proposal 1: RVQoE reporting over F1 can be deactivated per UE. Procedure used is FFS.

[SS] We think the procedure should be UE-aasociated in order to maintain the future proofness. (If we would like to deactivete the RVQoE reporting over F1 in a more granular way in the future.)

[Nokia]: We believe we need a step by step approach and not complexify too much F1AP for RVQoE in this release. As discussed offline we could consider as a compromise a procedure to both deactivate and resume RVQoE reporting for the entire DU or possibly some finer granularity (like per cell). The name of such class 2 procedure could be e.g. QoE Report Control Indication. Making the procedure UE-associated will oblige the DU to always perform deactivation per UE. There are examples of non-UE associated procedures that may include a list of UE IDs, so a non-UE associated procedure seems more flexible and could in later release be enhanced to include such list of UE IDs if needed.

[Lenovo]: Fine with the proposal.

[ZTE]:  we were a propponent for the non-UE associated procedure, but okay to go with this per-UE level RVQoE deactivation, soncidering the QoE information transfer from CU to DU is per UE. Why the last FFS part is needed? It was already agreed to use class-2 message, and with the new agreement this meeting, hopefully the procedure could be decided as well.

[Xiaomi] according to offline discussion, we think both per UE deactivation and per DU deactivation are possible, it is possible that DU is not interested in RVQoE information if it has no resource limitation, it can allocate enough resources for all the UEs, RVQoE information is not useful anymore.  We may need to define a procedure that support both cases, Nokia’s compromise procedure is fine to us.

Proposal 2: QoE measurement continuity from NR to LTE/5GC and vice versa can be supported by setting the area scope to PLMN-wide. FFS whether enhancements are needed.

- Moderator: this proposal does not exclude the proposal to extend the area scope with E-UTRA-specific choices. That is the FFS part.

[HW]: area scope is already option, we can simply not bring this IE.

[SS] Maybe not. What is the difference between adding a new PLMN-wide indication and reusing the existing CHOICE branch PLMN list?

[China Unicom] We also support to reuse the current area scope configuration.

[CATT] we also think we use the existing one

[Lenovo]: fine.

[ZTE]: Maybe no.

[Xiaomi]OK

Proposal 3: Turn the following WA to agreement: “WA: From NR to LTE, the source node adapts to the target node on deciding which one of the QoE configuration to keep.”

[HW]: There are still concerns to be addressed before turning it into agreement.

First, only the target node knows whether UE support the QoE measurement in the target RAT, how can source node know that? The source RAT node will not read the UE capabilities of other RATs. There are however cases that UE support QoE measurement of one service type in NR but does not support the QoE measurement of the same service type in LTE. In which case, the source node cannot decide which QoE measurement to keep.

Second, with source node to decide, how to handle the case that UE moves back to NR? If we let target to decide, source NR node can send all the s-based QoE measurements configuration container to target LTE node (which will be stored by target), and the target node can send all these containers to new target NR node again to maintain the QoE measurements for others if UE moves back to NR node.

Third, the main concern to let target decide seems some companies believe there are LTE impacts. However, this is not true. Indeed in legacy LTE cases, the target uses the configuration in RRC Handover Preparation message defined in 36.331 to know what has been configured to UE by the source. But here, for inter-RAT mobility, from NR to LTE, we don’t need to fill this part (don’t need to include the QoE related configuration in ASConfig). Instead, source sends all s-based configuration to target directly, and target decide which one to keep and configure this one to the UE in the handover command, and inform source to release the others. Then we have no LTE impacts

[SS] We support this proposal. If we let the target LTE node to decide, it will add extra function (on deciding which one of the QoE configuration to keep) to the target LTE node, which in our understanding is the extra impact brought to the LTE node.

[China Unicom] We support to turn this WA to agreement for the following reasons:

1. The source can decide which QoE reporting is the most important for the network;

2. There is only one QoE configuration can be included in the AS-Config->sourceOtherConfig-r9->measConfigAppLayer-r15 considering there will be no impact for the 36.331;

Considering Huawei's concern for whether the target can support the QoE configuration, we propose to include all the s-based QoE configuration in the UE Application Layer Measurement Configuration Information information defined in 38.413 and 38.423, then:

1. If the QoE configuration which is adopted by source in AS-Config->sourceOtherConfig-r9->measConfigAppLayer-r15 is not supported by the target, the target can select other s-based QoE configuratrion;

2. When the  UE move back to NR, all the QoE configuration can be maintained.

[CATT] we have same understanding as HW, to China Unicom, if the source include all configuration in HO  and target chose one, it is not align the proposal. . My  supported solution is similar  as china Unicom, i.e. the source send all the configuration to target 5GC/LTE with one indication for most wanted configuration, then the target  select one.

[Lenovo]: Agree. We can discuss the detailed solution next meeting.

[ZTE]: we understands the concern from Huawei about how the source knows the UE capability for LTE QMC collection. Maybe China Unicom's suggestion for including the QoE configuration outside the container can be considered. By the way, do we really discuss the case like NR -> LTE -> NR? RAN2 down selected the cases of LTE/5GC->NR and NR -> LTE/5GC for intra 5GC inter RAT handover.

[Xiaomi]OK

Proposal 4: For HO from LTE/5GC to NR, can be supported is no impacts to RAN3.

【CATT】if from NR to LTE include all configuration, the information should be transferred back from 5GC/LTE to NR.

[Xiaomi]OK