3GPP TSG-RAN3 Meeting #121	R3-234560

Toulouse, France, 21th Aug - 25th Aug 2023

3GPP TSG-RAN WG2#123			R2-2309191

Toulouse, France, Aug 21-25, 2023

Title:	Reply LS on applicability of pre-configured measurement gaps for RedCap UE

Release:	Rel-17

Work Item:	          NR_redcap-Core

Source:	RAN2

To:	RAN3

Cc:	RAN4

Contact Person:

#### Name:	Linhai He

E-mail Address:	linhaihe@qti.qualcomm.com

Attachments:

1. Overall Description:

RAN2 thanks RAN3 for the LS regarding “applicability of pre-configured measurement gaps for RedCap UE”.

RAN2 discussed following RAN3 question:

Based on “gapToAddModList IE” definition in MeasGapConfig, RAN3 would like to ask RAN2 whether it is possible that this IE can be used to configure different Pre-MGs for different BWPs, since the RedCap UE will require a MG whenever it performs a measurement outside its active BWP.

RAN2 Answer:

RAN2 would like to inform RAN3 that due to RAN4 restrictions, only one pre-configured measurement gap is allowed per gap type (i.e, UE type or per FR type), by using “gapToAddModList IE” in MeasGapConfig.

RAN2 would like to inform RAN3 that pre-configured MGs are applicable to both RedCap and non-RedCap UEs.

2. Actions:

To RAN3:

ACTION: 	RAN2 kindly asks RAN3 to take above response into account for RAN3 signalling design work.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2#123bis                   09 – 13 October 2023			Xiamen, China

TSG-RAN WG2#124                       13 – 17 November 2023		Chicago, USA