3GPP TSG-RAN WG3 Meeting #121	R3-234643

Toulouse, France, 21 – 25 Aug, 2023

Title:	LS on RACH enhancement

Response to:

Release:	Rel-18

Work Item:	NR_ENDC_SON_MDT_enh2-Core

Source:	RAN3

To:	RAN2

Cc:	-

Contact person:	Hongzhuo Zhang

Zhanghongzhuo(at)Huawei(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

In the context of RACH optimization, RAN3 discussed how to associate the RA Report(s) retrieved at a gNB with the corresponding RACH configuration, e.g. RACH partitioning configuration, at the time when the UE records the RA Report(s), in case the RACH configuration changed by the time an RA Report is retrieved. Following alternatives were discussed, but there was no consensus in RAN3:

Alt1: Enable the addition in the RA Report of the feature priority of each feature in the feature combination used by the UE at the time RACH access is triggered. This enables the NG-RAN to determine whether any optimizsation is needed with respect to how features with different priorities are combined in the same feature combination associated to a RACH partition.

Alt2: Enable the addition in the RA Report of RACH partition configuration information. This information consists of the start preamble index and the number of preambles in the partition for which the RA Report was generated. This enables the NG-RAN to determine the RACH partition in use.

Alt 3: Enable the addition in the RA Report of the time between RACH access that led to the generation of a RA Report and when RA Report was retrieved. Using this timer, and in case the NG-RAN stores time records of past RA Partitions configurations, feature priorities and feature combinations, the NG-RAN can figure out the RACH configuration, feature priorities and feature combination in use.

RAN3 would like to check with RAN2 on the feasibility and their preference on the above three alternatives.

# 2 	Actions

To RAN2

ACTION: 	RAN3 kindly asks RAN2 to provide feedback on the above question.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#121-bis 	 2023-10-09 - 2023-10-13		Xiamen, CN

RAN3#122  	2023-11-13 -  2023-11-17 		Chicago, US