3GPP TSG-RAN WG3 Meeting #121	R3-234650

Toulouse, France, 21 – 25 Aug, 2023

Agenda Item:	10.2.3

Source:	Huawei

Title:	(TP for SON BLCR for TS38.300): Remaining issues for RACH optimisation

Document for:	other

- Introduction

This document contains a TP for SON BLCR for TS38.300.

Annex 3-TP for SON BLCR for TS 38.300

/*************Start of change*******************/

15.5.3	Support for RACH Optimization

RACH optimization is supported by UE reported information made available at the NG RAN node as specified in TS 38.331 [12] and by PRACH parameters exchange between NG RAN nodes.

The contents of the RA Report comprise of the following:

-	Contention detection indication per RACH attempt;

-	Indexes of the SSBs and number of RACH preambles sent on each tried SSB listed in chronological order of attempts;

-	Indication whether the selected SSB is above or below the configured RSRP threshold per RACH attempt;

-	2-step RACH information as specified in clause 5.7.10.4 of TS 38.331 [12].

SN RA Reports

The UE operating in NR-DC, may also support collection of RA Reports by the SN.

The RA report retrieval and forwarding in case of NGEN-DC is specified in TS 37.340 [21].

/*************End of change*******************/