3GPP TSG-RAN WG3 Meeting #121	R3-234680

Toulouse, France, 21st – 25th August 2023

Agenda Item:	18.2

Source:	ZTE

Title:	(TP for IoT NTN BL CR 36.300) Alignment with NR NTN

Document for:	Approval

1	Introduction

In this contribution, we try to apply the progress of NR NTN to IoT NTN, including the stage 2 progress of HO with time-based trigger condition and UE location verification.

2	Text Proposal to 36.300

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

23.21.4.2	Mobility Management in ECM-CONNECTED

Radio link failure and RRC connection re-establishment are supported in NTN. The principles described in clause 10.1.6 apply unless specified otherwise. The principles described in clause 10.1.2 apply to NTN unless specified otherwise.

To enable mobility in NTN, the network provides target cell NTN payload assistance information needed to access the NTN cell in the handover command.

Conditional handover is supported for BL UEs and UEs in enhanced coverage.

When time-based trigger condition is used, the source eNB should consider the time indicated to the UE to decide when start the early data forwarding to the target eNB.

When a time-based trigger condition is used, the source eNB may signal the corresponding parameters to a single target eNB via the Source eNB to Target eNB Transparent Container in a S1-C based handover, see TS 23.401[17]. The source eNB signals the corresponding CHO configuration to the UE in the RRC Reconfiguration message during handover execution.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

23.21.7	MME(Re-)Selection by eNB

The eNB implements the NAS Node Selection Function specified in TS 36.410 [95].

For a RRC_CONNECTED UE, the eNB ensures that the BL UE or the UE in enhanced coverage connects to an MME that serves the country in which the UE is located. If the eNB detects that a BL UE or a UE in enhanced coverage is in a different country to that served by the serving MME, it should perform an S1 handover to change to an appropriate MME or initiate an UE Context Release Request procedure towards the serving MME (in which case the MME may decide to detach the UE).

For an RRC_CONNECTED NB-IoT UE, the eNB ensures that the NB-IoT UE connects to an MME that serves the country in which the UE is located. If the eNB detects that the UE is in a different country to that served by the serving MME, it should initiate a UE Context Release Request procedure towards the serving MME (in which case the MME may decide to detach the UE).

For the purpose of selecting an appropriate MME, the EPS may verify the UE location according to TS 23.401 [17] and TS 36.305 [51] after the UE has attached to the network.

NOTE:	UE location verification for MME selection should not be necessary if NTN cell(s) do not extend across countries.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>