3GPP TSG-RAN WG3 Meeting #121	R3-234698

Toulouse, France, 21 – 25 Aug, 2023

Title: 	(TP for MBS BL CR for 38.401) Update of MBS RAN sharing solution

Source: 	CATT, ZTE, Huawei, Nokia, Nokia shanghai Bell, Ericsson, Qualcomm, CMCC, CBN

Agenda item:	15.2

Document Type:	other

1. Introduction

This contribution provides TP to MBS 38.401 BL CR based on the agreement reached in RAN3#121meeting.

F1-U/NG-U tunnel establishments

CU-CP makes decision on whether to establish NG-U tunnel.

For MOCN scenario, only one set of shared F1-U tunnels is established and kept for MOCN scenario as long as there is one PLMN keeping the MBS service.

For multiple cell-ID broadcast scenario, the entity controlling logical DUs decides how many F1-U tunnels to be set up. The decision of CU-CP on establishment of NG-U tunnel takes the feedback of DU on establishment of a set of F1-U tunnels into account.

FFS whether the set of F1-U tunnel(s) and NG-U tunnel are always one to one mapping or not.

One F1AP message vs multiple F1AP messages

Adopt the option of Multiple F1AP contexts/messages, one per PLMN, for MOCN scenario.

E1AP impact

Associated session ID and MBS service area should be introduced in E1AP for shared NG-U termination scenario (regardless of whether we have one or multiple broadcast bearer context on E1 for RAN sharing).

Multiple E1AP context/message, one per PLMN, for MOCN scenario.

# 2	Text Proposal

------------Start of the First Change-----------------

7.7	Support for NR MBS

The Support of NR MBS in non-split gNB case is specified in TS 38.300 [2].

7.7.1	Support of dynamic PTP and PTM switching

For UEs in RRC_CONNECTED, NG-RAN supports dynamic switch between PTP and PTM for MBS as specified in TS 38.300 [2].

In case of split gNB architecture, for a MRB with common PDCP involving both PTP (RLC leg) and PTM (RLC leg), upon receiving the MBS data from the gNB-CU via a shared F1-U tunnel, the gNB-DU makes decision of using PTP (RLC leg) or PTM (RLC leg) or both.

For UEs in RRC_INACTIVE only PTM is supported as specified in TS 38.300 [2].

7.7.x	Support of resource efficiency for RAN Sharing

7.7.x.1	General

Editor’s Note:	TBD

7.7.x.2	Support of resource efficiency for MOCN

F1-AP supports resource efficiency in RAN sharing for MOCN.

Information enabling identifying broadcast MBS sessions for which RAN resources could be shared by the involved gNB-DUs and the gNB-CU-UP is provided by the gNB-CU-CP. Only one set of shared F1-U resources is established and kept as long as there is one PLMN keeping the Multicast service.

Editor’s Note:	Whether specific text for location dependent MBS sessions is necessary is FFS.

7.7.x.3	Support of resource efficiency for RAN Sharing with multiple cell-ID broadcast

F1AP supports resource efficiency in RAN Sharing with multiple cell-ID broadcast.

gNB-DUs sharing the same physical cell resources receive via F1-C information enabling identifying broadcast MBS sessions providing identical content.

Applying resource efficiency for RAN Sharing with multiple cell-ID broadcast

-	resolve different QoS requirements received from the participating 5GCs in an implementation specific way.

-	F1-U resources are established towards either all involved gNB-CUs or only some of them which is decided by the entity controlling the involved gNB-DUs sharing the same physical cell resources.

-	the gNB-CU-CP takes into account the decision F1-U resources have been established or not to decide whether to establish NG-U resources.

Editor’s Note:	Whether specific text for location dependent MBS sessions is necessary is FFS.

Editor’s Note:	Whether there is a one-one relationship between one set of F1-U tunnels and one NG-U tunnel is FFS.

------------End of the Change-----------------