3GPP TSG-RAN WG3 Meeting #121	R3-234486

# Toulouse, France, 21 - 25 August

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | Draft | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of PDU session split |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, CMCC, China Telecom, China Unicom, Nokia, Nokia Shanghai Bell, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-07-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During PDU session split, if the MN decides to setup a new tunnel at the SN side upon receiving a request for PDU session split from 5GC, this may take a long time since Uu measurement triggering and reporting, SN addition and RRC reconfiguration may need to be performed. After the AMF sends out the PDU session Resource Setup Request or PDU session Resource Modify Request, it expects to receive the corresponding response in a given time, otherwise it may consider the procedure is not performed successfully. The waiting time for the AMF to receive a response message for establishing a split PDU session requested by SMF is the same as the waiting time without a request from SMF to establish split PDU session because the AMF is agnostic to the request from SMF. The waiting time in the AMF may then be too short when MN needs to trigger Uu measurement triggering and reporting. The behaviour of MN is undetermined in this scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add notes to indicate that if the MN determines that it cannot take a decision to split the PDU session during the PDU session resource setup/modify procedure time (e.g. due to need of measurement triggering and reporting) it shall execute the PDU session resource setup/modify procedure without considering the split PDU session request from SMF and may trigger the RAN initiated QoS flow offloading procedure after the completion of the PDU session resource setup/modify procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5GC may not receive the response message form the MN in time and may consider the procedure is not successfully performed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.14.1, 10.14.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Revision of R3-234486, capture revision comments received during RAN3 #121 meeting |  |  |  |  |  |  |  |  |

## 10.14	PDU Session Split at UPF

### 10.14.1	PDU Session Split at UPF during PDU session resource setup

When a new PDU session needs to be established, the 5GC may provide two UL TEID addresses during PDU Session Resource Setup in order to allow for PDU session split. The MN may perform the SN Addition or the MN-initiated SN Modification procedure. If the MN decides to split the PDU session, the MN provides two DL TEID addresses and also the QoS flows associated with each tunnel.

Figure 10.14.1-1: PDU Session Split at UPF during PDU session resource setup

- The 5GC provides two UL TEID addresses during PDU Session Resource Setup, to be applied as the first UL tunnel on the NG-U interface and the additional NG-U tunnel in case the MN decides to split the PDU session.

If the MN determines that it cannot take a decision to split the PDU session during the PDU session resource setup procedure time (e.g. due to need of measurement triggering and reporting) it shall execute the PDU session resource setup procedure without considering the split PDU session request from SMF and may trigger the RAN initiated QoS flow offloading procedure (see section 10.14.3) after the completion of the PDU session resource setup procedure.

2.	The MN decides to setup two tunnels. The MN uses the SN Addition procedure (as described in 10.2.2) or the MN-initiated SN Modification procedure (as described in 10.3.2) up to step 6.

3.	The MN provides a DL TEID address to be applied as the first and an additional DL tunnel address on the NG-U interface. The MN also provides which QoS flows are associated with which tunnel.

### 10.14.2	PDU Session Split at UPF during PDU session resource modify (5GC initiated)

The 5GC may provide an additional UL TEID address during PDU Session Resource Modify in order to allow the MN to split the PDU session. The MN may perform the SN Addition or the MN-initiated SN Modification procedure. If the MN decides to split the PDU session, the MN provides a DL TEID address to be applied as the additional DL tunnel address and the QoS flows associated with that tunnel.

Figure 10.14.2-1: PDU Session Split at UPF during PDU session resource modify

1.	The 5GC provides an additional UL TEID address during PDU Session Resource Modify, to be applied as the additional NG-U tunnel in case the MN decides to split the PDU session.

If the MN determines that it cannot take a decision to split the PDU session during the PDU session resource modify procedure time (e.g. due to need of measurement triggering and reporting) it shall execute the PDU session resource modify procedure without considering the split PDU session request from SMF and may trigger the RAN initiated QoS flow offloading procedure (see section 10.14.3) after the completion of the PDU session resource modify procedure.

2.	The MN decides to setup two tunnels. If the new tunnel is to be setup at the SN, the MN uses the SN Addition procedure (as described in 10.2.2) or the MN-initiated SN Modification procedure (as described in 10.3.2) up to step 6, or up to step 8 if a QoS flow is moved to the SN and data forwarding applies.

3.	The MN provides a DL TEID address to be applied as the additional DL tunnel address on the NG-U interface and the QoS flows associated with that tunnel.