3GPP TSG-RAN WG3 Meeting #121	R3-234796

Toulouse, France, 21 – 25 Aug, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0281 | rev | 5 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BLCR to 38.401) Introduction of NR MBS enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Ericsson, Lenovo, ZTE, CATT, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_enh-Core |  |  |  |  |  | Date: |  |  | 2023-09-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of NR MBS enhancements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Capture RAN Sharing aspects including the support of resource efficiency for RAN Sharing with multiple cell-ID broadcast. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR MBS enhancements cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.x (new), 7.7.x.1 (new), 7.7.x.2 (new), 7.7.x.3 (new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.300 CR ... TS 38.470 CR0111 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial version: Capture agreed TP R3-231032. Rev1: resubmission based on latest version of specification. Rev2: merge agreed TP R3-232057. Rev3: resubmission based on latest version of spec. Rev4: add co-source companies. Rev5: merge agreed TPs R3-234765 and R3-234704. |  |  |  |  |  |  |  |  |

------------Start of the First Change-----------------

## 7.7	Support for NR MBS

The Support of NR MBS in non-split gNB case is specified in TS 38.300 [2].

### 7.7.1	Support of dynamic PTP and PTM switching

For UEs in RRC_CONNECTED, NG-RAN supports dynamic switch between PTP and PTM for MBS as specified in TS 38.300 [2].

In case of split gNB architecture, for a MRB with common PDCP involving both PTP (RLC leg) and PTM (RLC leg), upon receiving the MBS data from the gNB-CU via a shared F1-U tunnel, the gNB-DU makes decision of using PTP (RLC leg) or PTM (RLC leg) or both.

For UEs in RRC_INACTIVE only PTM is supported as specified in TS 38.300 [2].

### 7.7.x	Support of resource efficiency for RAN Sharing

#### 7.7.x.1	General

Editor’s Note:	TBD

#### 7.7.x.2	Support of resource efficiency for MOCN

Information enabling identifying broadcast MBS sessions for which RAN resources could be shared by the involved gNB-DUs and the gNB-CU-UP is provided by the gNB-CU-CP. Only one set of shared F1-U resources is established and kept as long as there is one PLMN keeping the Multicast service.

Editor’s Note:	Whether specific text for location dependent MBS sessions is necessary is FFS.

#### 7.7.x.3	Support of resource efficiency for RAN Sharing with multiple cell-ID broadcast

gNB-DUs sharing the same physical cell resources receive via F1-C information enabling identifying broadcast MBS sessions providing identical content.

Applying resource efficiency for RAN Sharing with multiple cell-ID broadcast

-	resolve different QoS requirements received from the participating 5GCs in an implementation specific way.

-	F1-U resources are established towards either all involved gNB-CUs or only some of them which is decided by the entity controlling the involved gNB-DUs sharing the same physical cell resources.

-	the gNB-CU-CP takes into account the decision F1-U resources have been established or not to decide whether to establish NG-U resources.

Editor’s Note:	Whether specific text for location dependent MBS sessions is necessary is FFS.

Editor’s Note:	Whether there is a one-one relationship between one set of F1-U tunnels and one NG-U tunnel is FFS.

### 7.7.y	Support of Multicast reception for UEs in RRC_INACTIVE

F1AP supports enabling of multicast reception for UEs in RRC_INACTIVE.

The gNB-CU decides whether multicast reception in RRC_INACTIVE is applied.

Editor’s Note:	FFS whether additional assistance information is sent by gNB-DU to gNB-CU to help gNB-CU's decision making.

------------End of the Change-----------------