3GPP TSG-RAN WG3 Meeting #121	R3-234837

Toulouse, France, 21– 25 August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | -- | rev | -- | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BL CR to 38.300) for XR Enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc, Ericsson, Nokia, Nokia Shanghai Bell, Huawei, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2023-09-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add stage-2 changes to support NR XR enhancement |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes are: - Add Stage-2 changes based on RAN3#121 NR XR enhancement related agreements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NR XR enhancement cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.x.1 (new), 16.x.2 (new), 16.x.3 (new), 16.x.4 (new), 16.x.5 (new), 16.x.6 (new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Next Modified Subclause (new)

## 16.X	eXtended Reality Services

### 16.X.1	General

The following functions will be supported by NG-RAN for handling eXtended Reality (XR) services.

-	PDU Set QoS handling.

- 	Power saving enhancements.

Editor’s note: Other NG-RAN functionality details to be updated based on further discussion.

### 16.X.2	Awareness

XR-Awareness relies on QoS flows, PDU Sets, Data Bursts and traffic assistance information (see TS 23.501 [3]). Optional PDU Set QoS Parameters may be provided by the SMF to the gNB as part of the QoS profile of the QoS flow.

SMF provides following PDU Set QoS Parameters to NG-RAN as part of the NG-AP PDU Session Resource Management procedure. The NG-RAN node shall, if supported, store the PDU Set QoS Parameters and take into account the received PDU Set QoS Parameters, as specified in TS 23.501 [3].

- PDU Set Delay Budget (PSDB): as defined in TS 23.501 [3], upper bound for the duration between the reception time of the first PDU (at the UPF for DL, at the UE for UL) and the time when all PDUs of a PDU Set have been successfully received (at the UE in DL, at the UPF in UL). A QoS Flow is associated with only one PSDB, and when available, it applies to both DL and UL and supersedes the PDB of the QoS flow.

- PDU Set Error Rate (PSER): as defined in TS 23.501 [3], upper bound for a rate of non-congestion related PDU Set losses between RAN and the UE. A QoS Flow is associated with only one PSER, and when available, it applies to both DL and UL and supersedes the PER of the QoS flow.

NOTE 1:	In this release, a PDU set is considered as successfully delivered only when all PDUs of a PDU Set are delivered successfully.

- PDU Set Integrated Handling Information (PSIHI): indicates whether all PDUs of the PDU Set are needed for the usage of PDU Set by application layer, as defined in TS 23.501 [3].

NOTE 2:	The PDU Set QoS parameters are common for all PDU Sets within a QoS flow.

During the Xn-handover procedure, source NG-RAN node will send the stored PDU Set QoS Parameters as part of the QoS profile of the QoS flow to the target NG-RAN node.

In addition, the UPF can identify PDUs that belong to PDU Sets, and may determine the following PDU Set Information which it sends to the gNB in the GTP-U header:

-	PDU Set Sequence Number;

-	Indication of End PDU of the PDU Set;

-	PDU Sequence Number within a PDU Set;

-	PDU Set Size in bytes;

-	PDU Set Importance (PSI), which identifies the relative importance of a PDU Set compared to other PDU Sets within the same QoS Flow.

### 16.X.3	Power Saving

5GC may provide XR traffic assistance information to NG-RAN through NG-AP TSC Assistance Information (TSCAI) as specified in clause 5.37.8 of TS 23.501[3]. This assistance information will be used by NG-RAN node to configure connected mode DRX scheme to enable UE power saving for efficient handling of XR traffic.

-	UL and/or DL Periodicity

-	N6 Jitter Information associated with the DL Periodicity

-	Indication of End of Data Burst in the GTP-U header of the last PDU in downlink.

Editor's Notes: Additional details will be updated based on further discussion.

### 16.X.4	Capacity

Editor's Notes: FFS: how NG-RAN node performs DL PDU Set discarding when congestion is detected and what parameters NG-RAN node takes into account.

### 16.X.5	NG-RAN Congestion Reporting and ECN marking for L4S

Editor's Notes: FFS: How NG-RAN based ECN marking for L4S in UPF will be handled and How NG-RAN provides congestion information to UPF for 5GC Information Exposure to AF.

### 16.X.6	Non-Homogeneous support of PDU set handling in NG-RAN

Editor’s Notes: FFS: How PDU set QoS parameters will be handled during HO from source NG-RAN node not supporting PDU Set handling to target NG-RAN node supporting PDU Set QoS handling.

End of Changes