3GPP TSG-RAN WG3 Meeting #122	          R3-238130

Chicago, USA, Nov 13 - Nov 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.425 | CR | 0148 | rev | 5 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for NR XR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, Nokia, Nokia Shanghai Bell, Ericsson, ZTE, Qualcomm Inc, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce the support for NR XR features. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RAN3#121: - Add the definitions and abbreviations for NR XR features. RAN3#121bis: - Add the Uplink Information Flag, Downlink Information Flag, Uplink Information and Downlink Information in PDU Type 2. RAN3#122： - Remove the definitions and abbreviations for PDU Set QoS parameters. - Add the procedure text of transferring UL/DL Congestion Information in the ASSISTANCE INFORMATION DATA frame. - Add the UL/DL Congestion Information Indicator and UL/DL Congestion Information in the ASSISTANCE INFORMATION DATA frame. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NR XR features will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3.1, 5.5.2.3, 5.5.3.x1 (new), 5.5.3.x2 (new), 5.5.3.x3 (new), 5.5.3.x4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 37.483 CR 0078 TS 38.413 CR 1025 TS 38.415 CR 0037 TS 38.423 CR 1091 TS 38.473 CR 1219 TS 38.410 CR 0125 TS 38.420 CR 0040 TS 38.470 CR 0125 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev-0: Capture the agreed changes from R3-233657 in RAN3#121. Rev-1: Resubmission to RAN3 #121bis. Rev-2: Capture the agreed changes from R3-235841 in RAN3#121bis. Rev-3: Resubmission to RAN3 #122. Rev-4: Revision of R3-237098 Rev-5: Capture the agreed changes from R3-237848 and R3-237973 in RAN3#122. |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Corresponding node: a node interacting with a node hosting PDCP for flow control. In an IAB network, this is the IAB-DU serving the UE for the corresponding data radio bearer.

eNB-CP: as defined in TS 36.401 [9].

eNB-UP: as defined in TS 36.401 [9].

Master node: as defined in TS 37.340 [3].

Secondary node: as defined in TS 37.340 [3].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

EN-DC	E-UTRA-NR Dual Connectivity

IAB	Integrated Access and Backhaul

MBS	Multicast/Broadcast Service

MR-DC	Multi-RAT Dual Connectivity

MRB	MBS Radio Bearer

PTM	Point To Multipoint

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 5.4.3	Transfer of Assistance Information

#### 5.4.3.1	Successful operation

NOTE 1:	In this section, PDCP duplication and delay measurement related information are not applicable to E-UTRA PDCP.

The purpose of the Transfer of Assistance Information procedure is to provide assistance information to the node hosting the NR PDCP entity. Such information may be taken into consideration by the node hosting the NR PDCP entity for UP management and optimisation procedures.

An NR user plane protocol instance making use of the Transfer of Assistance Information procedure is associated to a single data radio bearer only.

The Transfer of Assistance Information procedure may be invoked if

-	the corresponding node decides to send the Radio Quality Assistance Information and/or the PDCP duplication activation suggestion to the node hosting the NR PDCP entity for the concerned data radio bearer or,

-	the corresponding node decides to send the Radio Quality Assistance Information to the node hosting the NR PDCP entity for the concerned RLC entity.

The Transfer of Assistance Information procedure may be invoked if the corresponding node is configured to perform the QoS monitoring and to send the QoS monitoring results to the node hosting the NR PDCP entity for the concerned data radio bearer.

The ASSISTANCE INFORMATION DATA frame may include one or more Radio Quality Assistance Information. The information shall consist of the information indicated in the Assistance Information Type.

The ASSISTANCE INFORMATION DATA shall be sent, if supported, when the corresponding node receives a DL USER DATA PDU including the Assistance Information Report Polling Flag set to 1.

The ASSISTANCE INFORMATION DATA frame may include the PDCP Duplication Activation Suggestion, which informs the node hosting the NR PDCP entity of the suggestion from the corresponding node on whether to activate or not activate DL PDCP duplication. The node hosting the NR PDCP entity may take this information into account to take a decision on whether to activate or not activate PDCP duplication.

The ASSISTANCE INFORMATION DATA frame may include the UL Delay or/and DL Delay measured by the corresponding node. The node hosting the NR PDCP entity may take this information into account to calculate the whole UL or/and DL delay of RAN.

The ASSISTANCE INFORMATION DATA frame may include UL Congestion Information and/or DL Congestion information measured by the corresponding node. The node hosting the NR PDCP entity shall, if supported, take this information into account to perform ECN marking in the NG-RAN node, or to further send it to the UPF for ECN marking or information exposure as specified in TS 23.501 [8].

#### Figure 5.4.3.1-1: Successful Transfer of Assistance Information Data

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

5.5.2.3	ASSISTANCE INFORMATION DATA (PDU Type 2)

This frame format is defined to allow the node hosting the PDCP entity to receive assistance information.

The following shows the respective ASSISTANCE INFORMATION DATA frame.

NOTE 1:	All information elements defined in Figure 5.5.2.3-1 are also applicable to E-UTRA PDCP unless specified otherwise in section 5.5.3.

| Bits |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 |  | 3 |  | 2 | 1 | 0 |  |
| PDU Type (=2) |  |  |  | PDCP Dupl. Ind. |  | Assistance Info. Ind. |  | UL Delay Ind. | DL Delay Ind. | 1 |
| Spare |  |  |  |  |  |  | UL Congestion Information Ind. | DL Congestion Information Ind. | PDCP Duplication Activation Suggestion | 1 |
| Number of Assistance Information Fields |  |  |  |  |  |  |  |  |  | 0 or 1 |
| Assistance Information Type |  |  |  |  |  |  |  |  |  | 0 or (2*Number of Assistance Info Fields + sum of Number of octets for Radio Quality Assistance Information Fields) |
| Number of octets for Radio Quality Assistance Information Fields |  |  |  |  |  |  |  |  |  |  |
| Radio Quality Assistance Information |  |  |  |  |  |  |  |  |  |  |
| UL Delay DU Result |  |  |  |  |  |  |  |  |  | 0 or 4 |
| DL Delay DU Result |  |  |  |  |  |  |  |  |  | 0 or 4 |
| UL Congestion Information |  |  |  |  |  |  |  |  |  | 0 or 2 |
| DL Congestion Information |  |  |  |  |  |  |  |  |  | 0 or 2 |

Figure 5.5.2.3-1: ASSISTANCE INFORMATION DATA (PDU Type 2) Format

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 5.5.3.x1	UL Congestion Information Indicator

Description: This parameter indicates the presence of UL Congestion Information.

Value range: {0= UL Congestion Information not present, 1= UL Congestion Information present}.

Field length: 1 bit.

#### 5.5.3.x2	DL Congestion Information Indicator

Description: This parameter indicates the presence of DL Congestion Information.

Value range: {0= DL Congestion Information not present, 1= DL Congestion Information present}.

Field length: 1 bit.

#### 5.5.3.x3	UL Congestion Information

Description: For the cases of ECN marking Request, this field indicates the percentage of UL IP packets up to two decimal points that should be ECN marked for a DRB.

For the case of Congestion Information Request, this field should be interpreted as a percentage of congestion level in UL up to two decimal points for a DRB.

As an example, value 9574 corresponds to a percentage of 95.74%.

Value range: {0..10000}.

Field length: 2 octets.

#### 5.5.3.x4	DL Congestion Information

Description: For the cases of ECN marking Request, this field indicates the percentage of DL IP packets up to two decimal points that should be ECN marked for a DRB.

For the case of Congestion Information Request, this field should be interpreted as a percentage of congestion level in DL up to two decimal points for a DRB.

As an example, value 9574 corresponds to a percentage of 95.74%.

Value range: {0..10000}.

Field length: 2 octets.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>