3GPP TSG-RAN WG3 Meeting #122	R3-237082

# Chicago, USA, 13 – 17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.305 | CR | Draft | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BL CR to TS 38.305) Support of NR Positioning Enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, CATT, Huawei, Ericsson, Xiaomi, ZTE, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-10-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CR introduces functionality for support of NR positioning enhancements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add description of procedures for cross-cell SRS configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing stage 2 for support of NR positioning enhancements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | RAN3#121bis (post): Inclusion of agreed TP (R3-235812) RAN3#122 (pre): Resubmission |  |  |  |  |  |  |  |  |

Start of change

## 7.x	Procedures for Cross-Cell SRS Configuration

### 7.x.1	General

The Cross-Cell SRS Configuration procedure is used by the network to configure the cross-cell SRS configuration for the UE, including the Area Specific SRS Resource Allocation Procedure and Area Specific SRS Resource Update Procedure.

Editor’s Note: The name of the procedures and the general description could be updated or refined.

### 7.x.2	Area Specific SRS Resource Allocation Procedure

Figure 7.x.2-1 shows the Area Specific SRS Resource Allocation Procedure.

Figure 7.x.2-1: Area Specific SRS Resource Allocation Procedure

1.	LMF initiates NRPPa Positioning Information Request towards the serving gNB, including LPHAP Assistance Information in the Requested SRS Transmission Characteristics.

2.	The serving gNB allocates the area specific SRS configuration, and moves the UE back to RRC_INACTIVE by sending RRCRelease message, which includes the area specific SRS configuration, and corresponding validity area.

3.	The serving gNB responds the NRPPa Positioning Information Response to the LMF, including the LPHAP Validity Area Cells.

4.	The LMF notifies the gNBs within LPHAP validity area cells to reserve the SRS configuration.

Editor’s Note: More details could be further updated according to the future discussion.

### 7.x.3	Area Specific SRS Resource Update Procedure

Figure 7.x.3-1 shows the Area Specific SRS Resource Update Procedure.

Figure 7.x.3-1: Area Specific SRS Resource Update Procedure

0.	The UE in RRC_INACTIVE is configured with an area specific SRS configuration.

1.	When UE moves out of the configured validity area, it request for new SRS resource.

Editor’s Note: FFS on the UE behaviour, taking RAN2 agreement into account.

2.	The gNB which receives the request from UE triggers the Retrieve UE Context procedure, and indicate new SRS is requested.

Editor’s Note: FFS on whether the new cause value or indicator is needed in the Retrieve UE Context Request.

3.	The last serving gNB notifies the LMF the UE is moves out of the validity area by providing the new Cell ID where the UE resumes from.

Editor’s Note: FFS on which node to send Positioning Information Update to the LMF.

4.	The last serving gNB relocate the full UE context to the new gNB.

5.	The receiving gNB triggers the Path Switch Request procedure towards the AMF.

6.	AMF response with the Path Switch Request Acknowledge.

7.	The LMF indicates the receiving gNB to allocate new SRS resource for the UE. If LPHAP assistance info is included in the Requested SRS Transmission Characteristics, the Area Specific SRS Resource Allocation Procedure as specified in section 7.x.2 is applied. Or else, the legacy SRS allocation procedure is applied.

8.	The receiving gNB indicates the last serving gNB to release the UE context.

Editor’s Note: More details could be further updated according to the future discussion.

End of change