3GPP TSG-RAN WG3 Meeting #122	R3-237180

Chicago, IL, U.S.A., November 13th – 17th 2023

Agenda Item:	11.3.

Source:	Ericsson

Title:	(TP for QoE BL CR for TS 38.423, TS 37.340, and TS 38.300): QoE and RVQoE Measurements and Reporting in NR-DC Scenarios

Document for:	Agreement

# Introduction

In this paper we further discuss the support for QoE and RVQoE measurements and reporting in NR-DC scenarios, based on the agreements and TBCs from the RAN3#121-bis and earlier meetings. TPs for TS 38.423, TS 37.340 and TS 38.300 are provided in Annexes A, B and C.

# Discussion

The following issues are discussed:

- QMC handling upon mobility in NR-DC.

- Enhancements of the MN-SN coordination procedure

## QMC handling upon mobility in NR-DC

The following RAN3#121-bis agreements are related to the coordination between MN and SN in case of mobility:

To support inter MN HO scenarios, the new MN needs to be provided with all the information that the old MN has about the SN configured QoE measurements exchanged during co-ordination procedure.

To support SN change scenario, the co-ordination procedure can be used to deliver the necessary information about SN configured QoE measurements in old SN to MN, and MN can transfer this information to new SN.

The key issue is how to handle the transfer of QMC-related information. We think that the following general principles should be followed:

- MN change: Upon MN change, the QMC-related information known by the source MN needs to be transferred from the source MN to the target MN.

- SN change: Upon SN change, the QMC-related information known by the source SN needs to be transferred from the source SN to the target SN.

  - This is the information contained in the Rel-17 QMC Configuration Information IE, i.e., the information that is, since Rel-17, passed to the target during HO in single connectivity.

- Reuse of existing procedures and IEs: reuse as much as possible IEs and procedures that are already specified for NR-DC before the support for QoE in NR-DC was introduced.

The relevant scenarios are discussed below.

Scenario A: MN change without SN change

In this scenario, the source MN needs to send to the target MN:

- The information about the MN-owned configurations. This is already supported today, since the XnAP HANDOVER REQUEST already contains the QMC Configuration Information IE. However, the source MN also needs to indicate to the target MN the reporting paths used for MN-owned QoE and RVQoE configurations.

- The MN’s information about the SN-owned configurations (e.g., SN-related QMC Information at MN IE) – this is the information that the source MN may have learned during the coordination with the SN. There is absolutely no need for the MN in a non-mobility scenario to know more about the SN-owned configurations than the MN in a mobility scenario.

Specification impact:

- A new IE inside the QMC Configuration Information IE, to indicate the QoE and RVQoE reporting paths.

- A new IE in the HANDOVER REQUEST message containing the MN’s information about the SN-owned configurations (e.g., SN-related QMC Information at MN IE): the QoE reference, the current reporting path for QoE and for RVQoE, and the current RVQoE configuration.

Proposal 1-1: Include in the QMC Configuration Information IE the information about the current reporting path for QoE and RVQoE.

Proposal 1-2: Include in the XnAP HANDOVER REQUEST message to transfer from the source MN to the target MN the SN-related QMC Information at MN IE, containing the following information about the SN-owned QMC configurations: 1) the QoE reference, 2) the current reporting path for QoE, 3) the current reporting path for RVQoE, 4) the current RVQoE configuration.

Scenario B: MN change with SN change

This scenario is a superset of Scenario A since it also includes the SN change. Hence, in addition to Scenario A, the source SN also needs to send to the target SN via the MN the SN-originated information about the SN-owned configurations (e.g., Source SN to Target SN QMC Information IE). This information may be conveyed by means of the existing XnAP QMC Configuration Information IE, transferred between two gNBs in case of Rel-17 mobility. Regarding the messages used to convey this information, we note that, clause 10.7.2 of TS 37.340, for the case of MN change with SN change states that, before the HANDOVER REQUEST is sent from the source MN to the target MN, “the source MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration”. In other words, as per legacy specifications, the old MN So, reusing the legacy approach, i.e.:

- First the source MN retrieves from the source SN the Source SN to Target SN QMC Information IE, via the MN-initiated SN Modification procedure.

- Then the source MN sends the Source SN to Target SN QMC Information IE to the target MN in the XnAP HANDOVER REQUEST (together with the SN-related QMC Information at MN IE, as per Proposal 1-2), after which the target MN sends the Source SN to Target SN QMC Information IE to the target SN in the S-NODE ADDITION REQUEST message.

Specification impact:

- Inclusion, in the S-NODE MODIFICATION REQUEST message, the MN’s request asking the source SN to provide the XnAP QMC Configuration Information IE (carrying the Source SN to Target SN QMC Information), and the inclusion of this information into the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message.

- Inclusion of the existing XnAP QMC Configuration Information IE (carrying the Source SN to Target SN QMC Information) into the existing XnAP HANDOVER REQUEST and S-NODE ADDITION REQUEST messages.

Based on the above, the following proposals are needed:

Proposal 1-3: In case of MN change with SN change, the source MN can trigger MN-initiated SN Modification procedure to retrieve the Source SN to Target SN QMC Information IE to be forwarded to the target SN.

Proposal 1-4: Include the Source SN to Target SN QMC Information IE into the XnAP HANDOVER REQUEST and S-NODE ADDITION REQUEST messages.

During the RAN3#121-bis meeting, some companies argued that the Source SN to Target SN QMC Information IE s should be provided to the MN continuously, i.e., even if it is not apparent whether mobility will happen. This means that the Source SN to Target SN QMC Information IE would need to be included into the QMC Coordination Request IE. We think that this is not needed, i.e., it is sufficient to pass this information only upon mobility, but we are open to discuss the proposal.

Proposal 1-5: Discuss whether to include the Source SN to Target SN QMC Information IE into the QMC Coordination Request IE.

Scenario C: SN-initiated SN change

In this scenario, the source SN needs to transfer the Source SN to Target SN QMC Information IE to the target SN (via the MN). This information can be included into the S-NODE CHANGE REQUIRED message towards the MN, which then forwards the information to the target SN in the S-NODE ADDITION REQUEST message. Same as Scenario B, the XnAP QMC Configuration Information IE, could be reused for this purpose (bearing a different name).

Specification impact:

- Inclusion of the existing XnAP QMC Configuration Information IE (carrying Source SN to Target SN QMC Information) into the existing S-NODE CHANGE REQUIRED message.

- The part of Proposal 1-4 pertaining to the S-NODE ADDITION REQUEST message.

Proposal 1-6: Include the Source SN to Target SN QMC Information IE into the S-NODE CHANGE REQUIRED message.

Scenario D: MN-initiated SN change

Also in this scenario, the source SN needs to transfer the Source SN to Target SN QMC Information IE to the target SN (via the MN). Just like in Scenario B:

- First the source MN retrieves from the source SN the Source SN to Target SN QMC Information IE, via the MN-initiated SN Modification procedure (as per Proposal 1-3).

- Then the source MN sends this information to the target SN via the SN Addition procedure.

Specification impact:

- Inclusion, in the S-NODE MODIFICATION REQUEST message, the MN’s request asking SN to provide the XnAP QMC Configuration Information IE (carrying the Source SN to Target SN QMC Information), and the inclusion of this information into the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message.

- The part of Proposal 1-4 pertaining to the S-NODE ADDITION REQUEST message.

Proposal 1-7: In case of MN initiated SN change, the source MN can trigger the MN-initiated SN Modification procedure to retrieve the Source SN to Target SN QMC Information IE from the source SN.

## Enhancements of the MN-SN coordination procedure

A RAN3#119-bis agreement states:

If the SN is asked by the MN to forward to the MCE the QoE reports pertaining to a measurement configured by the MN, the MN should indicate to the SN the QoE Reference, the MCE IP Address and the RRC ID.

If the MN is asked by the SN to forward to the MCE the QoE reports pertaining to a measurement configured by the SN, the SN should indicate to the MN the QoE Reference and the MCE IP Address.

The current coordination procedure between MN and SN supports the first agreement (MN can send to the SN the QoE Reference, the MCE IP Address and the Measurement Configuration Application Layer ID), while the second agreement is only partially supported, because the SN can send to the MN the QoE Reference, but not the MCE IP Address. This part should be added to the coordination procedure between MN and SN.

Proposal 2-1: Add in the QMC Coordination Request IE (in the SN-to-MN part) an optional IE containing the MCE IP Address.

Another necessary enhancement relates to the following scenario where the MN has configured the UE and later determines that the SN carries the session (or that also the SN carries the session). As already captured in the BL CR, the MN (in the QMC Coordination Request IE) can send to the SN the current RVQoE configuration. In the response message, the SN can indicate a RVQoE configuration preference that is different from the one sent to the UE. At this point, the MN can modify it, and if it does so, a new RVQoE configuration becomes relevant, and the SN needs to be informed about it. Note that, since the configuration is in this case owned by the MN, it is up to the MN whether to update the RVQoE configuration according to the SN’s preferences. In any case, the SN needs to be informed about MN’s decision.

Observation: For MN-owned RVQoE configurations, if the SN expresses the RVQoE configuration preference to the MN, the MN needs to notify the SN whether the RVQoE configuration was modified according to the SN’s preferences.

Based on the above, we conclude that the MN should notify the SN that the UE has been successfully configured with the new RVQoE configuration by using the S-NODE RECONFIGURATION COMPLETE message (S-NG-RAN node Reconfiguration Completion class-2 procedure). This needs to be captured in the QoE BL CR for TS 38.423.

Proposal 2-2: For MN-initiated QMC coordination, enhance the existing S-NODE RECONFIGURATION COMPLETE XnAP message to notify the SN whether the RVQoE configuration has been modified.

Note that, in the opposite scenario, where the SN owns the RVQoE configuration, the RVQoE reporting goes via SRB4, and MN realizes that it is carrying the session, the MN should be able to express its RVQoE configuration preference to the SN. Hence, the RAN Visible QoE Configuration Preference IE needs to be included into the MN to SN QMC Coordination Request List IE.

Proposal 2-3: Include the RAN Visible QoE Configuration Preference IE into the MN to SN QMC Coordination Request List IE.

## TPs for QoE BL CRs for TS 38.423, TS 37.340, and TS 38.300

TPs for QoE BL CRs for TS 38.423, TS 37.340, and TS 38.300 are provided in Annexes A, B and C, respectively.

Proposal 3: Agree the TPs for QoE BL CR for TS 38.423, TS 37.340, and TS 38.300 provided in Annexes A, B and C.

# Conclusion

This paper discusses the support for QoE and RVQoE measurements and reporting in NR-DC scenarios. The following is proposed:

Proposal 1-1: Include in the QMC Configuration Information IE the information about the current reporting path for QoE and RVQoE.

Proposal 1-2: Include in the XnAP HANDOVER REQUEST message to transfer from the source MN to the target MN the SN-related QMC Information at MN IE, containing the following information about the SN-owned QMC configurations: 1) the QoE reference, 2) the current reporting path for QoE, 3) the current reporting path for RVQoE, 4) the current RVQoE configuration.

Proposal 1-3: In case of MN change with SN change, the source MN can trigger MN-initiated SN Modification procedure to retrieve the Source SN to Target SN QMC Information IE to be forwarded to the target SN.

Proposal 1-4: Include the Source SN to Target SN QMC Information IE into the XnAP HANDOVER REQUEST and S-NODE ADDITION REQUEST messages.

Proposal 1-5: Discuss whether to include the Source SN to Target SN QMC Information IE into the QMC Coordination Request IE.

Proposal 1-6: Include the Source SN to Target SN QMC Information IE into the S-NODE CHANGE REQUIRED message.

Proposal 1-7: In case of MN initiated SN change, the source MN can trigger the MN-initiated SN Modification procedure to retrieve the Source SN to Target SN QMC Information IE from the source SN.

Proposal 2-1: Add in the QMC Coordination Request IE (in the SN-to-MN part) an optional IE containing the MCE IP Address.

Observation: For MN-owned RVQoE configurations, if the SN expresses the RVQoE configuration preference to the MN, the MN needs to notify the SN whether the RVQoE configuration was modified according to the SN’s preferences.

Proposal 2-2: For MN-initiated QMC coordination, enhance the existing S-NODE RECONFIGURATION COMPLETE XnAP message to notify the SN whether the RVQoE configuration has been modified.

Proposal 2-3: Include the RAN Visible QoE Configuration Preference IE into the MN to SN QMC Coordination Request List IE.

Proposal 3: Agree the TPs for QoE BL CR for TS 38.423, TS 37.340, and TS 38.300 provided in Annexes A, B and C.

# Annex A: TP for QoE BL CR for TS 38.423

-------------------------------------------Start of changes-------------------------------------------

## 8.2	Basic mobility procedures

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

If the QMC Configuration Information IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, take it into account for QoE measurements handling, as described in TS 38.300 [9].

If the SN-related QMC Information at MN IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it for QoE measurements handling, as specified in TS 38.300 [9].

If the Source SN to Target SN QMC Information IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it for QoE measurements handling, as specified in TS 38.300 [9].

If the UE Slice-Maximum Bit Rate List IE is contained in HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received UE Slice Maximum Bit Rate List in the UE context, and use the received UE Slice Maximum Bit Rate value for each S-NSSAI for the concerned UE as specified in TS 23.501 [7].

Interaction with SN Status Transfer procedure:

If the UE Context Kept Indicator IE set to "True" and the DRBs transferred to MN IE are included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, include the uplink/downlink PDCP SN and HFN status received from the S-NG-RAN node in the SN Status Transfer procedure towards the target NG-RAN node, as specified in TS 37.340 [8].

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

## 8.3	Procedures for Dual Connectivity

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

If the S-NG-RAN node UE Slice Maximum Bit Rate IE for a specific S-NSSAI is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store and use the received S-NG-RAN node UE Slice Maximum Bit Rate for all PDU sessions associated with the S-NSSAI for the concerned UE as defined in TS 23.501 [7].

If the QMC Coordination Request IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node may use it as specified in TS 37.340 [8], and shall, if supported, include the QMC Coordination Response IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

If the Source SN to Target SN QMC Information IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, use it for QoE measurements handling, as specified in TS 37.340 [8].

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits at least one PDU session resource, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE ADDITION REQUEST ACKNOWLEDGE message to the M-NG-RAN node except for a request for conditional configuration. The reception of the S-NODE RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall if TXnDCoverall is running.

Interaction with the Activity Notification procedure

Upon receiving an S-NODE ADDITION REQUEST message containing the Desired Activity Notification Level IE, the S-NG-RAN node shall, if supported, use this information to decide whether to trigger subsequent Activation Notification procedures according to the requested notification level.

### 8.3.2	S-NG-RAN node Reconfiguration Completion

#### 8.3.2.1	General

The purpose of the S-NG-RAN node Reconfiguration Completion procedure is to provide information to the S-NG-RAN node whether the requested configuration was successfully applied by the UE, or whether the requested modification of the RAN Visible QoE configuration was successfully applied by the UE.

The procedure uses UE-associated signalling.

#### 8.3.2.2	Successful Operation

Figure 8.3.2.2-1: S-NG-RAN node Reconfiguration Complete procedure, successful operation.

The M-NG-RAN node initiates the procedure by sending the S-NODE RECONFIGURATION COMPLETE message to the S-NG-RAN node.

The S-NODE RECONFIGURATION COMPLETE message may contain information that

-	either the UE has successfully applied the configuration requested by the S-NG-RAN node. The M-NG-RAN node may also provide configuration information in the M-NG-RAN node to S-NG-RAN node Container IE.

-	or the configuration requested by the S-NG-RAN node has been rejected. The M-NG-RAN node shall provide information with sufficient precision in the included Cause IE to enable the S-NG-RAN node to know the reason for an unsuccessful reconfiguration. The M-NG-RAN node may also provide configuration information in the M-NG-RAN node to S-NG-RAN node Container IE.

-    or the requested modification of RAN visible QoE configuration was successfully applied by the UE.

Upon reception of the S-NODE RECONFIGURATION COMPLETE message the S-NG-RAN node shall stop the timer TXnDCoverall if TXnDCoverall is running.

### 8.3.3	M-NG-RAN node initiated S-NG-RAN node Modification Preparation

#### 8.3.3.1	General

This procedure is used to enable an M-NG-RAN node to request an S-NG-RAN node to either modify the UE context at the S-NG-RAN node or to query the current SCG configuration for supporting delta signalling in M-NG-RAN node initiated S-NG-RAN node change, or to provide the S-RLF-related information to the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: M-NG-RAN node initiated S-NG-RAN node Modification Preparation, successful operation

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

If the Management Based MDT PLMN Modification List IE is contained in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, overwrite any previously stored Management Based MDT PLMN List information in the UE context and use the received information to determine subsequent selection of the UE for management based MDT defined in TS 32.422 [23].

If the QMC Coordination Request IE is contained in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node may use it as specified in TS 37.340 [8], and shall, if supported, include the QMC Coordination Response IE in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message.

If the Source SN to Target SN QMC Information Inquiry IE is contained in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, include the Source SN to Target SN QMC Information IE in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message.

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits a modification of the UE context requiring the M-NG-RAN node to report about the success of the RRC connection reconfiguration procedure, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message to the M-NG-RAN node except for a request for conditional configuration. The reception of the S-NG-RAN node RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall if TXnDCoverall is running.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

### 8.3.5	S-NG-RAN node initiated S-NG-RAN node Change

#### 8.3.5.1	General

This procedure is used by the S-NG-RAN node to trigger the change of the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.5.2	Successful Operation

Figure 8.3.5.2-1: S-NG-RAN node initiated S-NG-RAN node Change, successful operation.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

If the Multiple Target S-NG-RAN Node List IE is included in the S-NODE CHANGE REQUIRED message, if multiple Target S-NG-RAN nodes are prepared, the M-NG-RAN node may include the Additional List of PDU Session Resource Change Confirm Info – SN Terminated IE in the S-NODE CHANGE CONFIRM message to provide different data forwarding addresses for different Target S-NG-RAN nodes.

If the Source SN to Target SN QMC Information IE is contained in the S-NODE CHANGE REQUIRED message, the M-NG-RAN node shall, if supported, use it for QoE measurements handling, as specified in TS 37.340 [8].

Interaction with M-NG-RAN node initiated S-NG-RAN node Release:

If the M-NG-RAN node receives the S-NODE CHANGE REQUIRED message indicating releasing target S-NG-RAN node(s) and cancelling all prepared PSCells in the target S-NG-RAN node(s), the M-NG-RAN shall, if supported, trigger the M-NG-RAN node initiated S-NG-RAN node release procedure to the target S-NG-RAN node(s) and cancel all the prepared PSCells at the target S-NG-RAN node(s).

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

-------------------------------------------Next change-------------------------------------------

## 9.1	Message Functional Definition and Content

### 9.1.1	Messages for Basic Mobility Procedures

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |  |  |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| SN-related QMC Information at MN | O |  | 9.2.3.x3 | This IE contains the information that the M-NG-RAN node has about the QMC configurations at the S-NG-RAN node. | YES | ignore |
| Source SN to Target SN QMC Information | O |  | 9.2.3.156 | This IE contains SN-related QMC configuration information to be forwarded to the target S-NG-RAN node. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

-------------------------------------------Next change-------------------------------------------

### 9.1.2	Messages for Dual Connectivity Procedures

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |  |  |
| QMC Coordination Request | O |  | 9.2.3.x1 |  | YES | ignore |
| Source SN to Target SN QMC Information | O |  | 9.2.3.156 | This IE contains SN related QMC Configuration Information to be forwarded to the target S-NG-RAN node. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSNterminated | This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. |

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

#### 9.1.2.4	S-NODE RECONFIGURATION COMPLETE

This message is sent by the M-NG-RAN node to the S-NG-RAN node to indicate whether the configuration requested by the S-NG-RAN node was applied by the UE, or whether the requested modification of the RAN Visible QoE configuration for the UE was successful.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name |  | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID |  | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID |  | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Response Information |  | M |  |  |  | YES | ignore |
| >CHOICE Response Type |  | M |  |  |  | – |  |
| >>Configuration successfully applied |  |  |  |  |  | – |  |
| >>>M-NG-RAN node to S-NG-RAN node Container |  | O |  | OCTET STRING | Includes the RRCReconfigurationComplete message as defined in subclause 6.2.2 of TS 38.331 [10] or the RRCConnectionReconfigurationComplete message as defined in subclause 6.2.2 of TS 36.331 [14]. | – |  |
| >>Configuration rejected by the M-NG-RAN node |  |  |  |  |  | – |  |
| >>>Cause |  | M |  | 9.2.3.2 |  | – |  |
| >>>M-NG-RAN node to S-NG-RAN node Container |  | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in as defined in subclause 11.2.2 of TS 38.331 [10]. | – |  |
| RAN Visible Configuration Preference Response | O |  | ENUMERATED (true, ...) | Indicates whether the RAN Visible QoE configuration preference of the S-NG-RAN node was applied by the UE. | - | ignore |  |

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

#### 9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to either request the preparation to modify S-NG-RAN node resources for a specific UE, or to query for the current SCG configuration, or to provide the S-RLF-related information to the S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |  |  |
| Management Based MDT PLMN Modification List | O |  | MDT PLMN Modification List 9.2.3.169 |  | YES | ignore |
| QMC Coordination Request | O |  | 9.2.3.x1 |  | YES | ignore |
| Source SN to Target SN QMC Information Inquiry | O |  | BOOLEAN |  | YES | ignore |

#### 9.1.2.6	S-NODE MODIFICATION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node’s request to modify the S-NG-RAN node resources for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | ignore |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |  |  |
| Conditional PSCell Addition Information Modification Acknowledge | O |  |  | This IE may be sent from the target SN. | YES | ignore |
| >Candidate PSCell List |  | 1 |  |  | – |  |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| Source SN to Target SN QMC Information | O |  | 9.2.3.157 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCell candidates. Value is 8 |

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

#### 9.1.2.11	S-NODE CHANGE REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to trigger the change of the S-NG-RAN node.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 | This IE shall be ignored if the Conditional PSCell Change Information Required IE is present. | YES | reject |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |  |  |
| >>>S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. |  |  |
| Source SN to Target SN QMC Information | O |  | 9.2.3.156 | This IE contains SN related QMC Configuration Information to be forwarded to the target S-NG-RAN node. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

### 9.2.3	General IE definitions

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

#### 9.2.3.157	UE Application Layer Measurement Configuration Information

This IE defines the information about the QoE Measurement Collection (QMC) configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are coming with the QMC activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3 bytes Octet String. |
| Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, ...) | This IE indicates the identity of the application layer measurement configuration, and corresponds to information provided in the MeasConfigAppLayerId IE as defined in TS 38.331 [10]. |
| Service Type | M |  | ENUMERATED (QMC for DASH streaming, QMC for MTSI, QMC for VR, ...) | This IE indicates the service type of QoE measurements. |
| QoE Measurement Status | O |  | ENUMERATED (ongoing, ...) | Indicates whether the QoE measurement has started. |
| Container for Application Layer Measurement Configuration | O |  | OCTET STRING (SIZE(1..8000)) | Contains the signalling based QoE measurement configuration, see Annex L in TS 26.247 [47], clause 16.5 in TS 26.114 [53] and clause 9 in TS 26.118 [54]. |
| CHOICE MDT Alignment Information | O |  |  | Indicates the MDT measurements with which alignment is required. |
| >S-based MDT |  |  |  |  |
| >>NG-RAN Trace ID | M |  | 9.2.3.97 | Indicates the signalling-based MDT measurements with which alignment is required. |
| Measurement Collection Entity IP Address | O |  | Transport Layer Address 9.2.3.29 | The IP address of the entity receiving the QoE measurement report. |
| CHOICE Area Scope of QMC | O |  |  |  |
| >Cell based |  |  |  |  |
| >>Cell ID List for QMC |  | 1 .. <maxnoofCellIDforQMC> |  |  |
| >>>Global NG-RAN Cell Identity | M |  | 9.2.2.27 | The included NG-RAN Cell Identity IE can only indicate the NR Cell Identity. |
| >TA based |  |  |  |  |
| >>TA List for QMC |  | 1 .. <maxnoofTAforQMC> |  |  |
| >>>TAC | M |  | 9.2.2.5 | The TAI is derived using the current serving PLMN. |
| >TAI based |  |  |  |  |
| >>TAI List for QMC |  | 1 .. <maxnoofTAforQMC> |  |  |
| >>>PLMN Identity | M |  | 9.2.2.4 |  |
| >>>TAC | M |  | 9.2.2.5 |  |
| >PLMN based |  |  |  |  |
| >>PLMN List for QMC |  | 1 .. <maxnoofPLMNforQMC> |  |  |
| >>>PLMN Identity | M |  | 9.2.2.4 |  |
| S-NSSAI List | O | 0..1 |  |  |
| >S-NSSAI Item |  | 1 .. <maxnoofSNSSAIforQMC> |  |  |
| >>S-NSSAI | M |  | S-NSSAI 9.2.3.21 |  |
| Available RAN Visible QoE Metrics | O |  | 9.2.3.158 | Present in case of signalling-based QoE. |
| MBS Communication Service Type | O |  | ENUMERATED (multicast, broadcast, …) | This IE indicates for which type of MBS communication service the QoE measurement configuration pertains to. |
| Current QoE and RVQoE Reporting Paths | O | 9.2.3.x4 | ENUMERATED (SRB4, SRB5, …) | This IE indicates the SRBs currently used for QoE and RVQoE reporting. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforQMC | Maximum no. of Cell IDs comprising the QMC scope. Value is 32. |
| maxnoofTAforQMC | Maximum no. of TA comprising the QMC scope. Value is 8. |
| maxnoofPLMNforQMC | Maximum no. of PLMNs in the PLMN list for QMC scope. Value is 16. |
| maxnoofSNSSAIforQMC | Maximum no. of S-NSSAIs comprising the QMC scope. Value is 16. |

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

-------------------------------------------Next change-------------------------------------------

#### 9.2.3.x1	QMC Coordination Request

This IE contains the information that the S-NG-RAN node needs to provide to the M-NG-RAN node, or the information that the M-NG-RAN node needs to provide to the S-NG-RAN node, for managing configuration and reporting of one or more QoE and/or RAN visible QoE measurements.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MN to SN QMC Coordination Request List |  | 0..1 |  |  |
| >MN to SN QMC Coordination Request Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) |  |
| >>Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, ...) |  |
| >>Measurement Collection Entity IP Address | O |  | Transport Layer Address 9.2.3.29 | The IP address of the entity receiving the QoE measurement report. |
| >>QoE Reporting Path Request | O |  | ENUMERATED (SRB4, SRB5, …) | This IE indicates the preferred SRB for receiving the QoE reports. |
| >>RAN Visible QoE Reporting Path Request | O |  | ENUMERATED (SRB4, SRB5, …) | This IE indicates the preferred SRB for receiving the RAN Visible QoE reports. |
| >>Further RAN Visible QoE Interest Inquiry | O |  | BOOLEAN | This IE is used to request from the S-NG-RAN node to indicate whether it is interested in receiving further RVQoE reports. |
| >>Further RAN Visible QoE Reporting Path | O |  | BOOLEAN | This IE indicates the preferred SRB for receiving further RAN Visible QoE reports. |
| >>Current RAN Visible QoE Configuration | O |  | 9.2.3.x5 | This IE is to indicate the current RAN Visible QoE configuration and inquire about the RAN Visible QoE Configuration preference of the S-NG-RAN node. |
| >>RAN Visible QoE Configuration Preference | O |  | 9.2.3.x5 | The RAN Visible QoE Configuration preference of the M-NG-RAN node. |
| SN to MN QMC Coordination Request List |  | 0..1 |  |  |
| >SN to MN QMC Coordination Request Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) |  |
| >>Measurement Collection Entity IP Address | O |  | Transport Layer Address 9.2.3.29 | The IP address of the entity receiving the QoE measurement report. |
| >>QoE Reporting Path Request | O |  | ENUMERATED (SRB4, SRB5, ...) | This IE indicates the preferred SRB for receiving the QoE reports. |
| >>RAN Visible QoE Reporting Path Request | O |  | ENUMERATED (SRB4, SRB5, ...) | This IE indicates the preferred SRB for receiving the RAN Visible QoE reports. |
| >>Further RAN Visible QoE Interest Inquiry | O |  | BOOLEAN | This IE is used to request from the M-NG-RAN node to indicate whether it is interested in receiving further RVQoE reports. |
| >>Further RAN Visible QoE Reporting Path | O |  | BOOLEAN | This IE indicates the preferred SRB for receiving further RAN Visible QoE reports. |
| >>Current RAN Visible QoE Configuration | O |  | 9.2.3.x5 | This IE is to indicate the current RAN Visible QoE configuration and inquire about the RAN Visible QoE Configuration preference of the M-NG-RAN node. |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

#### 9.2.3.x2	QMC Coordination Response

Editor’s Note: The messages used to convey this IE are FFS.

This IE contains the information that the M-NG-RAN node needs to provide to the S-NG-RAN node, or the information that the S-NG-RAN node needs to provide to the S-NG-RAN node in response to the QMC Coordination Request.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MN to SN QMC Coordination Response List |  | 0..1 |  |  |
| >MN to SN QMC Coordination Response Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) |  |
| >>Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, ...) |  |
| >>QoE Configuration Sending Path | O |  | ENUMERATED (MN, SN, …) |  |
| >>QoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  |
| >>RVQoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  |
| >>Further RAN Visible QoE Interest Response | O |  | ENUMERATED (interested, not-interested, ...) | This IE is to indicate whether the M-NG-RAN node is interested in receiving further RAN Visible QoE reports. |
| >>Further RAN Visible QoE Reporting Path | O |  | ENUMERATED (SRB4, SRB5, ...) | This IE is to indicate the preferred path for further RAN Visible QoE reporting. |
| >>RAN Visible QoE Configuration Preference | O |  | 9.2.3.x5 | The RAN Visible QoE Configuration preference of the M-NG-RAN node. |
| SN to MN QMC Coordination Response List |  | 0..1 |  |  |
| >SN to MN QMC Coordination Response Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) |  |
| >>QoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  |
| >>RVQoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  |
| >>Further RAN Visible QoE Interest Response | O |  | ENUMERATED (interested, not-interested, ...) | This IE is to indicate whether the S-NG-RAN node is interested in receiving further RAN Visible QoE reports. |
| >>Further RAN Visible QoE Reporting Path | O |  | ENUMERATED (SRB4, SRB5, ...) | This IE is to indicate the preferred path for further RAN Visible QoE reporting. |
| >>RAN Visible QoE Configuration Preference | O |  | 9.2.3.x5 | The RAN Visible QoE Configuration preference of the S-NG-RAN node. |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

#### 9.2.3.x3	SN-related QMC Information at MN

This IE contains the information that the M-NG-RAN node has about the QoE configurations at the S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QoE Reference | M |  | OCTET STRING (SIZE(6)) |  |
| QoE Reporting Path | O |  | ENUMERATED (SRB4, SRB5, …) | This IE indicates the SRB for receiving the QoE reports. |
| RVQoE Reporting Path | O |  | ENUMERATED (SRB4, SRB5, …) | This IE indicates the SRB for receiving the RAN Visible QoE reports. |
| Current RAN Visible QoE Configuration | O |  | 9.2.3.x5 | This IE indicates the current RAN Visible QoE configuration. |

#### 9.2.3.x4 	Current QoE and RVQoE Reporting Paths

This IE indicates the SRB currently used for receiving the QoE reports and RAN visible QoE reports.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QoE Reporting Path | O |  | ENUMERATED (SRB4, SRB5, …) | This IE indicates the SRB currently used for receiving the QoE reports. |
| RVQoE Reporting Path | O |  | ENUMERATED (SRB4, SRB5, …) | This IE indicates the SRB currently used for receiving the RAN Visible QoE reports. |

#### 9.2.3.x5	RAN Visible QoE Configuration

This IE provides information of RAN visible QoE configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Available RAN Visible QoE Metrics | O |  | 9.2.3.158 |  |
| Reporting Periodicity | O |  | ENUMERATED (ms120, ms240, ms480, ms640, ms1024) |  |

-------------------------------------------End of changes-------------------------------------------

# Annex B: TP for QoE BL CR for TS 37.340

-------------------------------------------Start of changes-------------------------------------------

## 10.5	Secondary Node Change (MN/SN initiated)

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

### 10.5.2	MR-DC with 5GC

MN initiated SN Change

The MN initiated SN change procedure is used to transfer a UE context from the source SN to a target SN and to change the SCG configuration in UE from one SN to another. This procedure can also be used to support coordination between MN and SN for managing the configuration and reporting of QoE measurements and/or RAN visible QoE measurements in NR-DC.

The Secondary Node Change procedure always involves signalling over MCG SRB towards the UE.

Figure 10.5.2-1: SN change procedure - MN initiated

Figure 10.5.2-1 shows an example signalling flow for the SN Change initiated by the MN:

1/2.	The MN initiates the SN change by requesting the target SN to allocate resources for the UE by means of the SN Addition procedure. The MN may include measurement results related to the target SN. If data forwarding is needed, the target SN provides data forwarding addresses to the MN. The target SN includes the indication of the full or delta RRC configuration. In case of coordination between MN and SN on QoE and RAN visible QoE measurement configuration and reporting, the SN Addition Request message may contain the QMC Coordination Request IE and the Source SN to Target SN QMC Information IE.

In case of coordination on QoE and RAN visible QoE measurement configuration and reporting, the SN Addition Request Acknowledge message may contain the QMC Coordination Response IE.

NOTE 1:	The MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration and the source SN-related QMC configuration information, and to allow provision of data forwarding related information before step 1.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

SN initiated SN Change

The SN initiated SN change procedure is used to transfer a UE context from the source SN to a target SN and to change the SCG configuration in UE from one SN to another. This procedure can also be used to support coordination between MN and SN for managing the configuration and reporting of QoE measurements and/or RAN visible QoE measurements in NR-DC.

Figure 10.5.2-2: SN change procedure - SN initiated

Figure 10.5.2-2 shows an example signalling flow for the SN Change initiated by the SN:

1.	The source SN initiates the SN change procedure by sending the SN Change Required message, which contains a candidate target node ID and may include the SCG configuration (to support delta configuration) and measurement results related to the target SN. In case of coordination between MN and SN on QoE and RAN visible QoE measurement configuration and reporting, the SN Change Required message may contain the Source SN to Target SN QMC Information IE.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

-------------------------------------------End of changes-------------------------------------------

# Annex C: TP for QoE BL CR for TS 38.300

-------------------------------------------Start of changes-------------------------------------------

## 21.3	QoE Measurement Continuity for Mobility

QoE measurement collection continuity for intra-system intra-RAT handover is supported, with the Area Scope parameters configured by the OAM, where the network is responsible for keeping track of whether the UE is inside or outside the area scope. A UE continues an ongoing QoE measurement even if it leaves the area scope, unless the network indicates to the UE to release the application layer measurement configuration.

For handover, the source gNB may transmit the information related to one or more application layer measurement configurations of the UE to the target gNB via XnAP or NGAP. For signalling-based QoE, the service type indication, QoE reference, and, optionally, the MCE IP address, measurement configuration application layer ID, MDT alignment information, area scope, slice support list for QMC, available RAN visible QoE metrics and measurement status are passed to the target gNB. For management-based QoE, the service type indication, measurement configuration application layer ID, the MCE IP address and QoE measurement status are passed to the target gNB. For RRC_INACTIVE state mobility, QoE measurement configuration(s) of a specific UE can be retrieved from the gNB hosting the UE context when it resumes to the RRC_CONNECTED state.

For signalling-based QoE, at handover to a target gNB that supports QoE measurement collection, the target gNB decides which of the application layer measurement configurations should be kept or released, e.g., based on application layer measurement configuration information received from the source gNB in Xn/NG signalling.

During mobility for UEs in NR-DC, the coordination procedure ensures that the target gNB(s) is (are) provided with all the information about the configured QoE/RVQoE measurements that the corresponding source gNB(s) has (have).

For QoE sessions pertaining to data flows received via MBS broadcast, QoE measurement collection may continue during the RRC_INACTIVE and RRC_IDLE states, and the measurement results, if collected, may be provided to the network when the UE returns to the RRC_CONNECTED state.

Upon UE’s transition from the RRC_IDLE to the RRC_CONNECTED state, the gNB serving the UE should ensure that it does not release an already configured signaling based QoE measurement configuration for the sake of configuring a new management-based QoE measurement configuration.

When the UE resumes the connection with a gNB that does not support QoE, the UE releases all application layer measurement configurations.

-------------------------------------------End of changes-------------------------------------------