3GPP TSG-RAN WG3 Meeting #122	R3-237308

Chicago, USA, 13 - 17 November 2023

Agenda Item:	14.3

Source:	ZTE

Title:	(TP for CHO with NR-DC to TS 38.423, TS37.340): Left issue on CHO with multiple SCG

Document for:	Approval

- Introduction

In the RAN3 #121bis meeting, RAN3 has made a lot of agreeements as below.

Include PSCell ID in the HO SUCCESS message The existing IEs (e.g. Handover Command container and data forwarding addresses) in Handover Request Ack. can be used for CHO-only. Enhance the CONDITIONAL HANDOVER CANCEL message to cancel/modify a particular PCell-PSCell pair prepared. In case of add additional PCell-PSCell configuration by the T-MN or the T-SN, whether to support adding a new PCell-PSCell configuration after the HO preparation is completed will not to be continued in R18. WA (depending on further gain-effort analysis): The information the source MN needs to know via Handover Request ACK: the direct path availability of S-SN<-> T-SN The introduced IEs for avoiding multiple data forwarding path, if needed, are applicable to direct data forwarding at the DC to DC handover and CHO with DC.

However, there are also some open issues left, seen in [R3-235767], as below.

List of issues to be discussed at next meeting - When only CHO with candidate SCG is prepared (there is no CHO only), how to handle the existing mandatory IE? - In case of CP-UP separation in the T-SN, there are two options to decide the same data forwarding tunnels: 1) T-SN-CU-CP decide the same tunnel - T-SN-CU-UP decide the same tunnel If 1), no signaling enhancement is needed. If 2), MN ID (maybe source or target) or other indicator in the bearer setup request, bearer context modification request and/or the bearer context release command may be needed. - Identification of parallel CPAC procedure - Measurement configuration or CHO with CPAC - New CHO Information SN Addition Acknowledge IE in SN ADD REQ ACK, which includes a list of the prepared PSCell IDs

This paper addresses the above left issues based on the above progress.

- Discussion

  - Issue1: How to handle the existing mandatory IE

When only CHO with candidate SCG is prepared (there is no CHO only), how to handle the existing mandatory IE?

The following tabular is abstracted from XnAP BLCR [R3-235969].

#### 9.1.1.2 HANDOVER REQUEST ACKNOWLEDGE This message is sent by the target NG-RAN node to inform the source NG-RAN node about the prepared resources at the target. Direction: target NG-RAN node  source NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; reject. Source NG-RAN node UE XnAP ID; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the source NG-RAN node; YES; ignore. Target NG-RAN node UE XnAP ID; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the target NG-RAN node; YES; ignore. PDU Session Resources Admitted List; M; 9.2.1.2; Ignored, if the CHO-only reconfiguration IE is included and set to "no-cho-only-config". (FFS); YES; ignore. PDU Session Resources Not Admitted List; O; 9.2.1.3; YES; ignore. Target NG-RAN node To Source NG-RAN node Transparent Container; M; OCTET STRING; Either includes the HandoverCommand message as defined in subclause 10.2.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB or shall be ignored if the CHO-only reconfiguration IE is included and set to "no-cho-only-config". (FFS); YES; ignore. UE Context Kept Indicator; O; 9.2.3.68; YES; ignore. Criticality Diagnostics; O; 9.2.3.3; YES; ignore. DRBs transferred to MN; O; DRB List 9.2.1.29; In case of DC, indicates that SN Status is needed for the listed DRBs from the S-NG-RAN node.; YES; ignore. DAPS Response Information; O; 9.2.1.34; YES; reject. Conditional Handover Information Acknowledge; O; YES; reject. >Requested Target Cell ID; M; Target Cell Global ID 9.2.3.25; Target cell indicated in the corresponding HANDOVER REQUEST message; –. >Maximum Number of CHO Preparations; O; 9.2.3.101; –. >CHO-only reconfiguration (FFS); O; ENUMERATED (no-cho-only-config, …); YES; reject. >Multiple Target S-NG-RAN Node List; 0..1; YES; reject. >>Multiple Target S-NG-RAN Node Item; 1 .. <maxnoofTargetSNs>; –. >>>Target S-NG-RAN node ID; M; Global NG-RAN Node ID 9.2.2.3; –. >>>PDU Session Resources Admitted List; M; 9.2.1.2; –. >>>Candidate PSCell List; 1; –. >>>>Candidate PSCell Item; 1 .. <maxnoofPSCellCandidate>; –. >>>>>PSCell ID; M; NR CGI 9.2.2.7; –. >>>>>Target NG-RAN node To Source NG-RAN node Transparent Container; M; OCTET STRING FFS; Includes the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB.; –. >>>>>Execution Condition Information; O; FFS; –. MBS Session Information Response List; O; 9.2.1.38; YES; ignore. RRC Config Indication; O; 9.2.3.72; YES; ignore. Range bound; Explanation. maxnoofTargetSNs; Maximum number of the target S-NG-RAN nodes. Value is FFS.. maxnoofPSCellCandidate; Maximum number of the candidate PSCells. Value is FFS..

In this tabular, the existing IE “Maximum Number of CHO Preparations” is used to indicate number of CHO without SCG, CHO with SCG and CHO with candidate SCGs.

In this tabular, the new IE structure “Multiple Target S-NG-RAN Node List” is used to configure CHO with candidate SCGs.

If the value of maxnoofTargetSNs multiplied by maxnoofPSCellCandidate is less than the value of Maximum Number of CHO Preparations, the left value is used for CHO without SCG and CHO with SCG. In this case, the existing mandatory IE can be reused.

Otherwise, if the value of maxnoofTargetSNs multiplied by maxnoofPSCellCandidate is equal to the value of Maximum Number of CHO Preparations, CHO only is not configured. In this case, the existing mandatory IE shall be ignored.

Proposal 1: Remove the new IE “CHO-only reconfiguration”.

Proposal 2: Add “Ignored, if the value of maxnoofTargetSNs multiplied by maxnoofPSCellCandidate is equal to the value of Maximum Number of CHO Preparations.” into the semantics description of the existing IEs.

  - Issue 2: Which node to assign the same data forwarding tunnel

In case of CP-UP separation in the T-SN, there are two options to decide the same data forwarding tunnels:

1)  T-SN-CU-CP decide the same tunnel

2)  T-SN-CU-UP decide the same tunnel

If 1), no signaling enhancement is needed.

If 2), MN ID (maybe source or target) or other indicator in the bearer setup request, bearer context modification request and/or the bearer context release command may be needed.

In legacy, it is the CU-UP to assign the data forwarding tunnel. For the same reason, it is the T-SN-CU-UP decide the same tunnel, refer to [R3-235160].

Proposal 3: It is CU-UP to assign the same data forwarding tunnel.

Proposal 4: Add target MN id into the E1AP: BEARER CONTEXT SETUP REQUEST message and BEARER CONTEXT RELEASE COMMAND message.

  - Issue 3: Other left issues

“Execution Condition Information”

In R17 CHO, the execution condition is included in the IE “arget NG-RAN node To Source NG-RAN node Transparent Container”. For the same reason, this method can be also used for R18 CHO with candidate SCGs.

Proposal 5: Remove the IE “Execution Condition Information”.

Based on the RAN2 agreement, the maximum number of CHO only and CHO with candidate SCG is 8.

Proposal 6: maxnoofTargetSNs is 8, maxnoofPSCellCandidate is 8

In the last RAN3 meeting, we have agreed that

For CHO with multiple SCGs among different T-SNs, include list of info of multiple SCGs in Handover Request Ack message, e.g., PDU Session admission results, data forwarding addresses and list of prepared PSCells for each prepared T-SN. This agreement can be revisited after RAN2 agreement.

The above agreement has been captured in stage 2 spec, but it has not captured in stage 3 spec.

Proposal 7: Capture above agreement into TS37.340 BLCR.

- Conclusion

Based on the above analysis, we provide the following proposals.

Proposal 1: Remove the new IE “CHO-only reconfiguration”.

Proposal 2: Add “Ignored, if the value of maxnoofTargetSNs multiplied by maxnoofPSCellCandidate is equal to the value of Maximum Number of CHO Preparations.” into the semantics description of the existing IEs.

Proposal 3: It is CU-UP to assign the same data forwarding tunnel.

Proposal 4: Add target MN id into the E1AP: BEARER CONTEXT SETUP REQUEST message and BEARER CONTEXT RELEASE COMMAND message.

Proposal 5: Remove the IE “Execution Condition Information”.

Proposal 6: maxnoofTargetSNs is 8, maxnoofPSCellCandidate is 8

- Text Proposal to 38.423

-------------------------<<<<<< START CHANGE >>>>>>--------------------------------------------------

9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target NG-RAN node to inform the source NG-RAN node about the prepared resources at the target.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List | M |  | 9.2.1.2 | Ignored, if the value of maxnoofTargetSNs multiplied by maxnoofPSCellCandidate is equal to the value of Maximum Number of CHO Preparations. | YES | ignore |
| PDU Session Resources Not Admitted List | O |  | 9.2.1.3 |  | YES | ignore |
| Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING | Either includes the HandoverCommand message as defined in subclause 10.2.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB or shall be ignored if the value of maxnoofTargetSNs multiplied by maxnoofPSCellCandidate is equal to the value of Maximum Number of CHO Preparations. | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| DRBs transferred to MN | O |  | DRB List 9.2.1.29 | In case of DC, indicates that SN Status is needed for the listed DRBs from the S-NG-RAN node. | YES | ignore |
| DAPS Response Information | O |  | 9.2.1.34 |  | YES | reject |
| Conditional Handover Information Acknowledge | O |  |  |  | YES | reject |
| >Requested Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 | Target cell indicated in the corresponding HANDOVER REQUEST message | – |  |
| >Maximum Number of CHO Preparations | O |  | 9.2.3.101 |  | – |  |
| >Multiple Target S-NG-RAN Node List |  | 0..1 |  |  | YES | reject |
| >>Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  | – |  |
| >>>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >>>PDU Session Resources Admitted List | M |  | 9.2.1.2 |  | – |  |
| >>>Candidate PSCell List |  | 1 |  |  | – |  |
| >>>>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>>>>Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING | Includes the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| MBS Session Information Response List | O |  | 9.2.1.38 |  | YES | ignore |
| RRC Config Indication | O |  | 9.2.3.72 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTargetSNs | Maximum number of the target S-NG-RAN nodes. Value is 8. |
| maxnoofPSCellCandidate | Maximum number of the candidate PSCells. Value is 8 |

-------------------------<<<<<< END CHANGE >>>>>>--------------------------------------------------

- Text Proposal to 37.340

-------------------------<<<<<< START CHANGE >>>>>>--------------------------------------------------

### 10.19.2	MR-DC with 5GC

The Conditional Handover with Secondary Node procedure is used for configuration and execution of CHO with SN. This procedure includes the cases where the SN is kept, changed or added. If the SN is kept, the UE context at the SN is kept. If the SN is changed, the UE context at the source SN is moved to the target SN.

Figure 10.19.2-1: Conditional Handover with Secondary Node procedure

Figure 10.19.2-1 shows an example signaling flow for Conditional Handover with Secondary Node.

NOTE 1:	For a CHO without SN change, the source SN and the target SN shown in Figure 10.19.2-1 are the same node.

NOTE 2:	For a CHO with SN addition, the source SN and steps involving the source SN in Figure 10.19.2-1 are ignored.

1.	The source MN starts the conditional handover procedure by initiating the Xn Handover Preparation procedure including MCG configuration and, if the UE is configured with an SCG, SCG configuration. The source MN includes the (source) SN UE XnAP ID, SN ID, the UE context in the (source) SN and the Conditional Handover Information Request IE in the Handover Request message.

NOTE 3:	In case of the CHO with/without SN change, the source MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration, if configured, before step 1.

NOTE 3a0: In case of CHO with candidate SCG(s), the source MN provides the maximum number of conditional reconfigurations that the candidate MN can prepare for the UE in the Handover Request message.

2.	If the candidate MN decides to keep the UE context in the SN, the candidate MN sends the SN Addition Request message to the SN including the SN UE XnAP ID as a reference to the UE context in the SN that was established by the source MN. If the candidate MN decides to change the SN allowing delta configuration, the candidate MN sends the SN Addition Request message to the candidate SN including the UE context in the source SN that was established by the source MN. Otherwise, the candidate MN may send the SN Addition Request message to the candidate SN including neither the SN UE XnAP ID nor the UE context in the source SN that was established by the source MN. Within the SN Addition Request message, the candidate MN also includes the CHO related information, i.e., the source MN ID and the MN UE XnAP ID in the source MN, in order to indicate that the SN Addition Preparation procedure is triggered in relation to a CHO and to enable the SN to identify requests related to the same UE.

NOTE 3a1: In case of CHO with candidate SCG(s), the candidate MN provides the maximum number of PSCells that the candidate SN can prepare for the UE in the SN Addition Request message.

NOTE 3a:	The target MN and other potential target MNs may trigger the SN Addition Preparation procedure to the same (target) SN.

NOTE 3b:	The source MN may initiate additional Xn Handover Preparation procedures towards the same or other target MNs. Based on each Xn Handover Preparation procedure, each target MN may decide to trigger SN Addition Preparation procedure.

3.	The (candidate) SN replies with the SN Addition Request Acknowledge message. The (candidate) SN may include the indication of the full or delta RRC configuration.

NOTE 4:	In CHO with SCG configuration, it is up to the candidate MN implementation to make sure that the CG-Config provided from the (candidate) SN can be used in all CHO preparations.

NOTE 4a0:	In CHO with (multiple) SCG(s) configurations, the (candidate) SN assigns the same data forwarding addresses for multiple data forwarding requests from different target MNs. If available, the (candidate) SN indicates to the target MN direct data forwarding path availability with the source SN and/or source MN.

3a.	For the SN terminated bearers using MCG resources, the candidate MN provides Xn-U DL TNL address information in the Xn-U Address Indication message.

4.	The candidate MN includes within the Handover Request Acknowledge message the MN RRC reconfiguration message to be sent to the UE in order to perform the conditional handover, and may also provide forwarding addresses to the source MN. If PDU session split is performed in the target side during handover procedure, more than one data forwarding addresses corresponding to each node are included in the Handover Request Acknowledge message. The candidate MN indicates to the source MN that the UE context in the SN is kept if the candidate MN and the SN decided to keep the UE context in the SN in step 2 and step 3.

For CHO with multiple SCGs among different T-SNs, the candidate MN includes a list of one or more T-SNs in Handover Request Acknowledge message, e.g., PDU Session admission results, data forwarding addresses and list of prepared PSCells for each prepared T-SN.

4a.	The source MN sends the Xn-U Address Indication message to the (source) SN. This Xn-U Address Indication message notifies conditional handover to the (source) SN, which may decide to perform, if applicable, early data forwarding for SN-terminated bearers, together with the sending of an Early Status Transfer message to the source MN.

NOTE 4a:	Separate Xn-U Address Indication procedures may be initiated to provide different forwarding addresses of the prepared conditional handovers. In this case, it is up to the source MN and SN implementations to make sure that the Early Status Transfer message(s) from the source SN, if any, is forwarded to the right target MN. The Xn-U Address Indication procedure may further be initiated to indicate to the (source) SN to stop already initiated early data forwarding for some SN-terminated bearers, if they are no longer subject to data forwarding due to the modification or cancellation of the prepared conditional handovers.

5.	The source MN sends an RRC reconfiguration message to the UE, including the CHO configuration, i.e. a list of RRC reconfiguration* messages and associated execution conditions, in which each RRC reconfiguration* message contains an MCG configuration and possibly an SCG configuration in the RRC reconfiguration** message received from the candidate SN in step 3.

6.	The UE applies the RRC reconfiguration message received in step 5, stores the CHO configuration and replies to the MN with an RRC reconfiguration complete message.

7/8. The UE maintains connection with the source MN and, if the UE is configured with a PSCell, with the source PSCell, after receiving CHO configuration, and starts evaluating the CHO execution condition for the candidate cell(s). If at least one CHO candidate cell satisfies the corresponding CHO execution condition, the UE detaches from the source MN, applies the stored corresponding configuration for that selected candidate cell, synchronises to that candidate cell and completes the RRC handover procedure by sending RRC reconfiguration complete* message to the target MN. If the stored configuration for the selected candidate cell includes an SCG configuration, the UE includes an embedded SN RRCReconfigurationComplete** message for the target SN. The UE releases stored CHO configurations after successful completion of RRC handover procedure.

NOTE 5:	In case the target SN includes the indication of the full RRC configuration, the MN performs release of the SN terminated radio bearer configuration and release and add of the NR SCG configuration part towards the UE.

9.	If configured with bearers requiring SCG radio resources, the UE synchronizes to the (target) SN.

NOTE 6:	The order the UE performs Random Access towards the MN (step 7) and performs the Random Access procedure towards the (target) SN (step 9) is not defined.

10.	If the RRC connection reconfiguration procedure was successful, the target MN informs the (target) SN via SN Reconfiguration Complete message.

11.	The target MN sends the Handover Success message to the source MN to inform that the UE has successfully accessed the target cell. In CHO with candidate SCG(s), the target PSCell ID may also be included in the Handover Success message.

12a/b.	The source MN sends SN Release Request message to the (source) SN including a Cause indicating MCG mobility. The source MN indicates to the (source) SN that the UE context in SN is kept, if it receives the indication from the target MN. The (source) SN acknowledges the release request.

12c.	The source MN sends XN-U Address Indication message to the (source) SN to transfer data forwarding information. More than one data forwarding addresses may be provided if the PDU session is split in the target side.

12d. The source MN sends the Handover Cancel message toward the other signalling connections or other candidate MNs, if any, to cancel CHO for the UE.

12e/f. If the target MN is configured with other candidate PCell(s) associated with other candidate SN(s) than the target SN, the target MN sends the SN Release Request message(s) to the corresponding candidate SN(s). Other candidate MN(s) send(s) the SN Release Request message(s) to other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

13a. The (source) SN sends the Secondary RAT Data Usage Report message to the source MN and includes the data volumes delivered to and received from the UE over the NR/E-UTRA radio as described in clause 10.11.2.

NOTE 7:	The order the source SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN/target SN is not defined. The SN may send the report when the transmission of the related QoS is stopped.

13b. The source MN sends the Secondary RAT Data Usage Report message to AMF to provide information on the used NR/E-UTRA resource.

14.	For bearers using RLC AM, the source MN sends the SN Status Transfer message to the target MN, including, if needed, SN Status received from the source SN. The target MN forwards the SN Status to the target SN, if needed.

15.	If applicable, data forwarding takes place from the source side (i.e. source MN or source SN). If the SN is kept, data forwarding may be omitted for the SN terminated bearers or QoS flows kept in the SN.

16-19.	The target MN initiates the Path Switch procedure. If the target MN includes multiple DL TEIDs for one PDU session in the Path Switch Request message, multiple UL TEID of the UPF for the PDU session should be included in the Path Switch Ack message in case there is TEID update in UPF.

NOTE 8:	If new UL TEIDs of the UPF for SN are included, the target MN performs MN initiated SN Modification procedure to provide them to the SN.

20.	The target MN initiates the UE Context Release procedure towards the source MN.

21.	Upon reception of the UE Context Release message from source MN, the (source) SN releases C-plane related resources associated to the UE context towards the source MN. Any ongoing data forwarding may continue. The SN shall not release the UE context associated with the target MN if the UE contest kept indication was included in the SN Release Request message in step 12a.

-------------------------<<<<<< END CHANGE >>>>>>--------------------------------------------------