3GPP TSG-RAN WG3 #122                                                    R3-237585

Chicago, USA, 13th – 17th Nov, 2023

Agenda Item:	10.2.1

Source:	CATT

Title:	(TP for SON 37.340, 38.423 and 36.423) Discussion on SON enhancement for SHR and SPR

Document for:	other

Introduction

In last RAN3 meeting, we made general discuss and achieved some agreements [1].

In the document, we provide some analysis on SON enhancements for SHR and SPR according to the new split topic.

Discussion

2.1 Inter-RAT SHR

As for correlation of NR SHR and LTE RLF Report, the following agreements have been achieved in last RAN2 meeting and LS in R3-237146 is also sent to RAN3 to inform the following agreement:

Agreements:

1	The target C-RNTI is included in inter-RAT SHR to enable the correlation of the SHR and RLF report.

In RAN3#121, the following agreement has been achieved:

If RAN2 agrees to use Target C-RNTI in order to support correlation, Target C-RNTI should be included in the Xn HANDOVER REPORT.

So, source NG-RAN can correlate SHR with RLF by target C-RNTI.

Proposal 1: RAN2 agreed to include target C-RNTI in inter-RAT SHR and RAN3 also agreed to introduce target C-RNTI in the Xn HANDOVER REPORT in RAN3#121 meeting, so, the issue on correlation of NR SHR and LTE RLF Report may be closed.

2.2 SPR for NR-DC

2.2.1 The objective of T304 SPR trigger

For SPR triggered by T312/T310, we have agreed the objective is to optimize PSCell change configuration and lower layer issues of source PSCell (e.g., optimize T310/T312 timer values) as below:

In RAN3 #119bis meeting:

If the trigger is T312/T310, the objective of SPR is to

- optimize PSCell change configuration and associated mobility thresholds

- optimize lower layer issues of source PSCell (e.g., optimize T310/T312 timer values)

Further, T310/T312 related SPR triggers can also be optimized to ensure UE doesn’t unnecessarily collect SPR or only rarely collects SPR.

As for the SPR triggered by T304, the open issue is as below:

The objective of T304 SPR trigger is to optimize RACH access issues in target SN.

If objective of T304 SPR trigger is also to optimize the mobility configuration in the initiating node, is there any conflicting optimization?

We think SPR can be used to optimize both RACH access issues in target SN and mobility configurations in the initiating node.

If the initiating node triggers PSCell change too early, UE cannot access to target cell smoothly and then SPR would be triggered by T304. In this case, PSCell change configuration should be optimized while the RACH resource configured by target is good. Hence the objective of SPR triggered by T304 also includes the optimization on mobility configurations.

Definitely, unsuitable RACH resource may also trigger SPR with T304 cause.

Proposal 2: The objective of SPR triggered by T304 shall include the optimization on RACH access issues in target SN and mobility configurations in the initiating node.

We think there is no issue on conflicting optimizations in the target SN and the initiating node.

For the initiating node, mobility configurations can be optimized based on measurement result of service cell and neighbor cells. If service cell does not good i.e., the cause T310/312 present while target cell is better than any other neighbor cells in measurement result during PSCell change execution, too late PSCell failure type would be detected. If service cell is good while target cell is weak during PSCell change execution, too early PSCell failure type would be detected.

In case of SPR is triggered by T304, the MN may always send SPR to target SN and target SN make the final decision on whether perform optimization on RACH issue.

Some companies believe source node optimize PSCell change configuration to decrease T304 running time may increase T310/312 running time. We illustrate the scenario as the figure below:

UE moves from PSCell A to PSCell B in the direction indicated by the arrow.

PSCell change occurs at the time point 1. At that time the signal of PSCell B is not good and SPR may be triggered by T304. PSCell B can know the reason is bad signal of target PSCell B from the measurement result in SPR, so, RACH resource should not be optimized. PSCell A should optimize PSCell change configuration to trigger PSCell change at time point 2 when UE is still in the courage of PSCell A, so, T310/T312 running time would not be increased.

So we think there is no conflicting optimizations issue in the target SN and the initiating node based on measurement result.

Proposal 3: there is no conflicting optimizations issue in the target SN and the initiating node. The optimization in initiating node can be performed base on measurement result of service cell, target cell and neighbour cells.

Also, for the issue on conflicting optimizations, we have to point out that objective of T310/T312 SPR trigger also includes two parts as below:

If the trigger is T312/T310, the objective of SPR is to

- optimize PSCell change configuration and associated mobility thresholds

- optimize lower layer issues of source PSCell (e.g., optimize T310/T312 timer values)

For MN initiated PSCell change, Old MN may first optimize PSCell change configuration based on measurement result after fetching SPR. MN may always send SPR to source SN and source SN make the final decision on whether perform lower layer issues optimization. We think there is no conflicting optimization which is the same for T304 SPR trigger.

Proposal 4: Both T304 SPR trigger and T310/T312 SPR trigger may involve two RAN nodes to perform optimization. We do not identify conflicting optimization issue for both cases.

As for spec impacts, after MN retrieves SPR trigged by T304, the SPR should be forwarded to both target SN and the initiating node. Each node can decide by itself whether the optimization is needed and interaction between them is not needed.

Proposal 5: It is proposed to forward SPR to both the target SN and the initiating node in case of T304 trigger being met.

2.2.2 How to configure SPR Trigger

For MN initiated PSCell change, there are some open issues on how MN configures T310/T312 SPR thresholds as below:

The SN can propose its preferred T310/T312 SPR thresholds to MN?

T310/T312 timer values can be provided as assistance information from SN to MN?

In last RAN2 meeting, two SPR configurations have been agreed as below:

Agreements:

2	UE should be allowed to store two SPR configurations configured by MN and SN respectively. UE only monitors the SPR configuration configured by the node that initiated the PSCell change.

3	The NW indicates that a PSCell change is MN-initiated or SN-initiated if UE support SPR, and UE includes this information in the SPR.

For SN initiated PSCell change, it is SN to set the T310/T312 SPR thresholds. While for MN initiated PSCell change, it should be MN to set the T310/T312 SPR thresholds. The related conclusion is also captured in RAN2 38.331 running CR as below:

thresholdPercentageT310-SCG This field indicates the threshold for the ratio in percentage between the elapsed T310 timer associated to the source PSCell and the configured value of the T310 timer. Value p40 corresponds to 40%, value p60 corresponds to 60% and so on. This field is set in the otherConfig configured by the source PSCell of the PSCell change, or in the otherConfig configured by the PCell for the PSCell change.

thresholdPercentageT312-SCG This field indicates the threshold for the ratio in percentage between the elapsed T312 timer associated to the measurement identity of the target PSCell and the configured value of the T312 timer. Value p20 corresponds to 20%, value p40 corresponds to 40% and so on. This field is set in the otherConfig configured by the source PSCell of the PSCell change, or in the otherConfig configured by the PCell for the PSCell change.

For SN initiated PSCell change, T310/T312 Threshold is included in CG-Config message which is supposed to be not known by MN. For MN initiated PSCell change, MN has to configure T310/T312 Threshold, and if SN can propose its preferred T310/T312 SPR threshold explicitly to MN, it may be helpful for MN to select a suitable T310/T312 SPR threshold. .

Proposal 6: It is proposed to provide SN preferred T310/T312 SPR thresholds from SN to MN.

Similar as P6, providing T310/T312 timer value from SN to MN is also helpful for MN to select a suitable T310/T312 SPR threshold for MN initiated PSCell change. In case of a big T310/T312 timer value configured by SN for certain UE, MN may configure a little T310/T312 SPR threshold to decrease the possibility of generating unnecessary SPR.

Proposal 7: It is proposed to provide T310/T312 timer value from SN to MN in order for MN to configure a suitable T310/T312 SPR threshold.

2.2.2 SPR available indication via SRB3

In last RAN2 meeting, RAN2 discussed whether to indicate SPR available indication via SRB3 which is captured in RAN2 38.331 running CR as below:

1>	else if the RRCReconfiguration message was received via SRB3 (UE in NR-DC):

2>	if the RRCReconfiguration message was received within DLInformationTransferMRDC:

3>	if the RRCReconfiguration message was received within the nr-SCG within mrdc-SecondaryCellGroup (NR SCG RRC Reconfiguration):

4>	if the scg-State is not included in the RRCReconfiguration message containing the RRCReconfiguration message:

5>	if reconfigurationWithSync was included in spCellConfig in nr-SCG:

6>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

6>	if the UE was configured with successPSCell-Config by the PCell or by the source PSCell:

7>	perform the actions for the successful PSCell change report determination as specified in clause 5.7.10.X, upon successfully completing the Random Access procedure triggered for the reconfigurationWithSync in spCellConfig of the SCG;

5>	else:

6>	the procedure ends;

4>	else:

5>	perform SCG deactivation as specified in 5.3.5.13b;

5>	the procedure ends;

3>	else:

4>	if the RRCReconfiguration does not include the mrdc-SecondaryCellGroupConfig:

5>	if the RRCReconfiguration includes the scg-State:

6>	perform SCG deactivation as specified in 5.3.5.13b;

4>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

2>	else:

3>	if the RRCReconfiguration includes the reconfigurationWithSync in spCellConfig for the SCG; and

3>	if the UE was configured with successPSCell-Config by the source PSCell:

4>	perform the actions for the successful PSCell change report determination as specified in clause 5.7.10.X, upon successfully completing the Random Access procedure triggered for the reconfigurationWithSync in spCellConfig of the SCG;

Editor´s note: Whether the UE indicates availability of the SPR to the SN when applying the RRC Reconfiguration via SRB3.

If UE directly indicate successPSCell-InfoAvailable in the RRCReconfigurationComplete message to SN, XN interface may be impact because only MN can retrieve SPR, SN should inform MN about availabilityof SPR via Xn interface.

So, RAN3 should pay attention to the result of issue and then decide whether to update XN interface accordingly.

Proposal 8: the result of the issue on whether UE indicates availability of the SPR to the SN may have impact on XN interface. RAN3 may need to update XN interface message accordingly.

2.3 corrections on BLCR 38.423

In 38.423 BL CR in R3-235959, the SN Mobility Information IE is included in S-NODE ADDITION REQUEST ACKNOWLEDGE message as below:

- 9.1.2.2	S-NODE ADDITION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node about the S-NG-RAN node addition preparation.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Skip unrelated IE |  |  |  |  |  |  |
| Conditional PSCell Addition Information Acknowledge | O |  |  |  | YES | ignore |
| >Candidate PSCell List |  | 1 |  |  | – |  |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to PSCell change; T-SN provides it in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |

The Semantics description of SN Mobility Information should include the case of CPA. We propose to the following description.

| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to PSCell change or PSCell addition; T-SN provides it in order to enable later analysis of the conditions that led to wrong PSCell change or PSCell addition. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Proposal 9: It is proposed for RAN3 to discuss the above modification in S-NODE ADDITION REQUEST ACKNOWLEDGE message.

The following Semantics description of SN Mobility Information should be revised as below because it is S-SN provide SN Mobility Information to MN via SN Release Request Acknowledge In case of MN initiated PSCell change.

9.1.2.15	S-NODE RELEASE REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to the M-NG-RAN node to confirm the request to release S-NG-RAN node resources.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| PDU sessions To Be Released List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources To Be Released List – SN terminated | O |  | PDU Session List with data forwarding request info 9.2.1.24 |  | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| SCG UE History Information | O |  | 9.2.3.151 |  | YES | ignore |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to PSCell change; S-SN provides it in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |

Proposal 10: It is proposed for RAN3 to discuss the above modification in S-NODE RELEASE REQUEST ACKNOWLEDGE message.

For the maximum number of Successful PSCell Change Reports in ACCESS AND MOBILITY INDICATION message, we propose to follow the number of SHR, i.e. 64.

Proposal 11: It is proposed that the maximum number of Successful PSCell Change Reports in ACCESS AND MOBILITY INDICATION message is 64.

In 38.423/36.423, the Semantics description of RACH is pending to RAN2 progress:

| >>PSCell List Container | O |  | OCTET STRING | [FFS – Name to be decided by RAN2] IE as defined in subclause [FFS – to be decided by RAN2] in TS 36.331 [9]. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

In RAN2 running CR, the name is defended as RACH-Report-r18.

UEInformationResponse-v1710-IEs ::= SEQUENCE {

coarseLocationInfo-r17				OCTET STRING					OPTIONAL,

nonCriticalExtension				UEInformationResponse-v18xx-IEs	OPTIONAL

}

UEInformationResponse-v18xx-IEs ::= SEQUENCE {

rach-Report-r18					    RACH-Report-r18				OPTIONAL,

nonCriticalExtension				SEQUENCE {}					    OPTIONAL

}

Proposal 12: Remove the FFS in 38.423 and 36.423.  RACH-Report-r18 IE as defined in subclause 6.2.2 in TS 36.331.

Conclusions

SHR:

Proposal 1: RAN2 agreed to include target C-RNTI in inter-RAT SHR and RAN3 also agreed to introduce target C-RNTI in the Xn HANDOVER REPORT in RAN3#121 meeting, so, the issue on correlation of NR SHR and LTE RLF Report may be closed.

SPR for NR-DC:

Proposal 2: The objective of SPR triggered by T304 shall include the optimization on RACH access issues in target SN and mobility configurations in the initiating node.

Proposal 3: there is no conflicting optimizations issue in the target SN and the initiating node. The optimization in initiating node can be performed base on measurement result of service cell, target cell and neighbour cells.

Proposal 4: Both T304 SPR trigger and T310/T312 SPR trigger may involve two RAN nodes to perform optimization. We do not identify conflicting optimization issue for both cases.

Proposal 5: It is proposed to forward SPR to both the target SN and the initiating node in case of T304 trigger being met.

Proposal 6: It is proposed to provide SN preferred T310/T312 SPR thresholds from SN to MN.

Proposal 7: It is proposed to provide T310/T312 timer value from SN to MN in order for MN to configure a suitable T310/T312 SPR threshold.

Proposal 8: the result of the issue on whether UE indicates availability of the SPR to the SN may have impact on XN interface. RAN3 may need to update XN interface message accordingly.

Proposal 9: It is proposed for RAN3 to discuss the above modification in S-NODE ADDITION REQUEST ACKNOWLEDGE message.

Proposal 10: It is proposed for RAN3 to discuss the above modification in S-NODE RELEASE REQUEST ACKNOWLEDGE message.

Proposal 11: It is proposed that the maximum number of Successful PSCell Change Reports in ACCESS AND MOBILITY INDICATION message is 64.

Proposal 12: Remove the FFS in 38.423 and 36.423.  RACH-Report-r18 IE as defined in subclause 6.2.2 in TS 36.331.

# 4. Reference

[1] RAN3#121bis, Chair notes

# 5. TP for 37.340

10.18.B	Successful PSCell Change Report

The objective of Successful PSCell change Report (SPR) is to detect sub-optimal successful PSCell change/CPC or successful PSCell addition/CPA.

For analysis of such sub-optimal successful PSCell change/CPC and successful PSCell addition/CPA, the UE may collect SPR based on the triggers configured by the initiating node, if received, and makes the SPR available to the network as specified in TS 38.331 [4].

For PSCell addition/CPA and PSCell change/CPC (MN or SN initiated), the target SN always decides the T304 trigger for SPR and performs root cause analysis.

For SN-initiated PSCell change/CPC, the source SN decides the T310/T312 triggers for SPR and is responsible for SPR related optimizations e.g., to optimize PSCell change/CPC configuration or associated mobility thresholds or adjust T310/T312 timer values.

For MN-initiated PSCell change/CPC, the MN decides the T310/T312 triggers for SPR. MN may optimize PSCell change/CPC configuration or associated mobility thresholds or both. Source SN may optimize lower layer issues e.g., adjust T310/T312 timer values.

The SPR can be fetched from the UE by the MN only while the UE is still connected to the MN, or by a node different from the MN that sent the SPR configuration to the UE if the UE is not connected to the MN anymore. In case the SPR is retrieved in a node different from the MN that sent the SPR configuration to the UE, the SPR is first forwarded to that MN, which then forwards it to the respective SN(s) which should perform the SPR optimization.

If SPR is triggered by T310/T312, SPR may be sent to source SN for either SN-initiated PSCell change or MN-initiated PSCell change case. If SPR is triggered by T304, SPR may be sent to target SN for either SN-initiated PSCell change or MN-initiated PSCell change case, and/or the node initialled PSCell change.

# 6. TP for 38.423

- 9.1.2.2	S-NODE ADDITION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node about the S-NG-RAN node addition preparation.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| PDU Session Resources Admitted To Be Added List |  | 1 |  |  | YES | ignore |
| >PDU Session Resources Admitted To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Response Info – SN terminated IE nor the PDU Session Resource Setup Response Info – MN terminated IE is present in a PDU Session Resources Admitted to be Added Item IE, abnormal conditions as specified in clause 8.3.1.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Setup Response Info – SN terminated | O |  | 9.2.1.6 |  | – |  |
| >>PDU Session Resource Setup Response Info – MN terminated | O |  | 9.2.1.8 |  | – |  |
| PDU Session Resources Not Admitted List | O |  |  |  | YES | ignore |
| >PDU Session Resources Not Admitted List – SN terminated | O |  | PDU Session Resources Not Admitted List 9.2.1.3 |  | – |  |
| >PDU Session Resources Not Admitted List – MN terminated | O |  | PDU Session Resources Not Admitted List 9.2.1.3 |  | – |  |
| S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message or the CG-CandidateList message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | reject |
| Admitted Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs | YES | reject |
| RRC Config Indication | O |  | 9.2.3.72 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Location Information at S-NODE | O |  | Target Cell Global ID 9.2.3.25 | Contains information to support localisation of the UE | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| Available fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates the fast MCG recovery via SRB3 is enabled. | YES | ignore |
| Direct Forwarding Path Availability | O |  | ENUMERATED (direct path available, …) | Indicates direct forwarding path is available between the target S-NG-RAN node and source NG-RAN node for intra-system handover or between the target S-NG-RAN node and the source SN. | YES | ignore |
| SCG Activation Status | O |  | 9.2.3.155 |  | YES | ignore |
| Conditional PSCell Addition Information Acknowledge | O |  |  |  | YES | ignore |
| >Candidate PSCell List |  | 1 |  |  | – |  |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to PSCell change or PSCell addition; T-SN provides it in order to enable later analysis of the conditions that led to wrong PSCell change or PSCell addition. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is 8 |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

- 9.1.2.15	S-NODE RELEASE REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to the M-NG-RAN node to confirm the request to release S-NG-RAN node resources.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| PDU sessions To Be Released List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources To Be Released List – SN terminated | O |  | PDU Session List with data forwarding request info 9.2.1.24 |  | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| SCG UE History Information | O |  | 9.2.3.151 |  | YES | ignore |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to PSCell change; S-SN provides it in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

- 9.1.3.25	ACCESS AND MOBILITY INDICATION

This message is sent by NG-RAN node1 to transfer access and mobility related information to NG-RAN node2.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report List Item |  | 1 .. < maxnoofRAReports > |  |  | EACH | ignore |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList IE as defined in subclause 6.2.2 in TS 38.331 [10]. | YES | ignore |
| >>UE Assistant Identifier | O |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | YES | ignore |
| >>PSCell List Container | O |  | OCTET STRING | RACH-Report-r18 IE as defined in subclause 6.2.2 in TS 36.331 [14]. | YES | ignore |
| Successful HO Report List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report List Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | – |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [10]. | – |  |
| Successful PSCell Change Report List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report List Item |  | 1 .. <maxnoofSuccessfulPSCellChangeReports> |  |  | – |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in subclause 6.2.2 in TS 38.331 [10]. | – |  |
| >>SN Mobility Information |  |  | BIT STRING (SIZE (32)) | Mobility Information in the PSCell of the source SN in case this message is sent from the MN to the source SN; Mobility Information in the PSCell of the target SN in case this message is sent from the MN to the target SN. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports, the maximum value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

- 9.3.7	Constant definitions

maxnoofSuccessfulPSCellChangeReports		INTEGER ::= 64

# 6. TP for 36.423

- 9.1.2.50	ACCESS AND MOBILITY INDICATION

This message is sent by the eNB to the eNB/en-gNB to transfer access and mobility related information.

Direction: eNB  en-gNB, eNB1  eNB2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NR RA Report List |  | 0..1 |  |  | YES | ignore |
| >NR RA Report List Item |  | 1 .. <maxnoofRAReports> |  |  | EACH | ignore |
| >>NR RA Report Container | M |  | OCTET STRING | RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [10]. | – |  |
| >>UE Assistant Identifier | O |  | en-gNB UE X2AP ID 9.2.100 |  | – |  |
| >>PSCell List Container | O |  | OCTET STRING | RACH-Report-r18 IE as defined in subclause 6.2.2 in TS 36.331 [9]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |