3GPP TSG-RAN WG3 #122	               R3-237598

13th – 17th Nov 2023 Chicago, USA

Agenda item:		26.2

Source:				CATT, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell

Title:	TP for 38.300 on Introduction of 5G Timing Resiliency and URLLC enhancements

Document for:		Approval

1		Introduction

This TP is to specify stage 2 to support the objectives in the WID on NR Timing Resiliency and URLLC enhancements.

Including the below items:

-	Support the 5GS network timing synchronization status and reporting.

-	Support adapting downstream and upstream scheduling based on RAN feedback for low latency communication.

-	Support Interworking with TSN network deployed in the transport network

2	TP for 38.300

----------------------------Change Begins-------------------------------------------------------------------------

16.8	Support for Time Sensitive Communications

Time Sensitive Communications (TSC), as defined in TS 23.501 [3], is a communication service that supports deterministic communication and/or isochronous communication with high reliability and availability. Examples of such services are the ones in the area of Industrial Internet of Things, e.g. related to cyber-physical control applications as described in TS 22.104 [39].

To support strict synchronization accuracy requirements of TSC applications, the gNB may signal 5G system time reference information to the UE using unicast or broadcast RRC signalling with a granularity of 10 ns. Uncertainty parameter may be included in reference time information to indicate its accuracy. The UE may indicate to the gNB a preference to be provisioned with reference time information using UE Assistance Information procedure. Propagation delay compensation (PDC) mechanisms may be applied based on RTT or TA, and can be performed at the UE or gNB side. When performed at UE side, the PDC mechanisms are controlled via RRC signalling by the gNB.

The RTT-based PDC mechanism is achieved by using Rx-Tx time difference measurements of a single pair of configured TRS/PRS and SRS. The following figure describes the signalling procedures of UE-side RTT-based PDC:

Figure 16.8-1: Signalling Procedure of UE-side RTT-based PDC

1.	The gNB provides measurement configurations to the UE;

2a/b.	The gNB transmits TRS or PRS to the UE for measurements, and the UE transmits SRS to the gNB for measurement;

3a/b.	Both the UE and the gNB perform Rx-Tx time difference measurements;

4.	The gNB provides its Rx-Tx time difference measurement to the UE;

5.	The UE performs PDC based on Rx-Tx time difference measurements from itself and the gNB.

The following figure describes the signalling procedures of gNB-side RTT-based PDC:

Figure 16.8-2: Signalling Procedure of gNB-side RTT-based PDC

1.	The gNB provides measurement configurations to the UE;

2a/b.	The gNB transmits TRS or PRS to the UE for measurements, and the UE transmits SRS to the gNB for measurement;

3a/b.	Both the UE and the gNB perform Rx-Tx time difference measurements;

4.	The UE reports its Rx-Tx time difference measurement to the gNB;

5.	The gNB performs PDC based on Rx-Tx time difference measurements from itself and the UE.

The gNB may also receive TSC Assistance Information (TSCAI), see TS 23.501 [3], from the Core Network, e.g. during QoS flow establishment, or from another gNB during handover. TSCAI contains additional information about the traffic flow such as burst arrival time, burst periodicity, and survival time. TSCAI knowledge may be leveraged in the gNB's scheduler to more efficiently schedule periodic, deterministic traffic flows either via Configured Grants, Semi-Persistent Scheduling or with dynamic grants, and/or to improve the associated link reliability to meet the survival time requirement (see TS 22.104 [39]).

To support uplink periodic traffics of services with survival time requirement, configured grant resources can be used such that the mapping relation between the service and the configured grant is known to both gNB and UE, thus allowing the gNB to use configured grant retransmission scheduling (addressed by CS-RNTI) to trigger survival time state entry for the corresponding DRB. Upon survival time state entry, all RLC entities configured for the DRB are activated by the UE for duplication to prevent failure of subsequent messages and hence fulfilling the survival time requirement.  If CA or DC duplication for the DRB is already activated, the DRB should enter survival time state when any retransmission grant for any of its active LCHs is received.

16.8.x1	Network timing synchronization monitoring

16.8.x1.0	General

While time synchronization service is offered by the 5GS, the network timing synchronization status of the gNB may change. The gNB detects timing synchronization degradation or improvement locally and informs the consumer of the information as follows:

-	TSCTSF may receive information about timing synchronization status from the gNB via the AMF based on node-level reporting configuration.

-	UE may receive clock quality information from the gNB based on UE-level clock quality control information.

16.8.x1.1	Network timing synchronization monitoring towards CN

The 5GC initiates RAN Timing Synchronisation Status (TSS) Reporting procedure to obtain the change of network timing synchronization status of gNBs.

Based on NG-RAN’s capabilities of reporting timing synchronisaation status, NG-RAN accepts or rejects the request from AMF. NG-RAN can be pre-configured with thresholds for attributes on timing synchronisaation status reporting via OAM. When the thresholds are met or exceeded, events will be triggered and NG-RAN reports Timing Synchronization Status to AMF. For detailed procedure on the Timing Synchronization Status reporting, refer to TS 38.401 Clause 8.x.1.

16.8.x1.2	Network timing synchronization monitoring towards UE

Editor’s Note: RAN3 and RAN2 scope.

16.8.x2	RAN feedback for adaptation of Burst Arrival Time and Periodicity

The TSCAI received by gNB from 5GC may contain a BAT Window or the Capability for BAT adaptation for a QoS Flow during QoS flow establishment or modification. Based on this information, the gNB can determine a BAT offset in order to align the arrival of the traffic bursts with the next expected transmission opportunity over the air interface in uplink and downlink, and the gNB shall provid the BAT offset to 5GC as specified in TS 23.501 [3].

The TSCAI may also contain a Periodicity Range for a QoS Flow. Based on this information, the gNB can determine an adjusted Periodicity in order to align the periodicity of the traffic bursts with the expected time interval between subsequent transmission opportunities over the air interface in uplink and downlink, and the gNB provides the djusted periodicity along with above specified BAT offset to 5GC as specified in TS 23.501 [3].

----------------------------Skipped unchange part-------------------------------------------------------------------------

16.y	Support for TSN enabled Transport Network

The SMF collocated with Centralized User Configuration (CUC) may interwork with the Centralized Network Configuration (CNC) in the transport network when the 5GS supports interworking with TSN deployed in the transport network,

The NG-RAN may receive a get-request included in a TL-Container from SMF/CUC which requests to report Access Network Talker/Listener (AN-TL) information during PDU session (FFS on QoS flow level or PDU session level) setup or modification. And the NG-RAN/AN-TL responds a get-response within TL-Container including the defined elements associated with the corresponding tunnel in the NG-RAN as specified in TS 23.501 [3].

The NG-RAN/AN-TL may also receive a set-request included in a TL-Container from SMF/CUC during QoS flow establishment or modification. The NG-RAN/AN-TL uses the provided configuration for the traffic in the QoS Flow as specified in TS 23.501 [3].

------------------------Change Ends----------------------------------------------------------------------------