3GPP TSG-RAN WG3 #122							R3-237785

Chicago, IL, USA, November 13th – 17th, 2023

Agenda Item:	10.2.1

Source:	Samsung (moderator)

Title:	Summary of Offline Discussion: CB: # SONMDT1_SHRSPR

Document for:	Discussion

Introduction

This is the offline summary for the following comeback:

CB: # SONMDT1_SHRSPR

- Further discuss the above open issue

-Provide TPs if agreeable

(moderator - SS)

Summary of offline disc R3-237785

For the Chairman’s Notes

TPs to be agreed:

SHR:

TP for SON BLCR for 38.413:  R3-237930 rev of R3-237606 (ZTE)

TP for SON BLCR for 38.423:  R3-237962 (new) (CATT)

TP for SON BLCR for 38.300:  R3-237944 rev of R3-237350 (Samsung)

SPR:

TP for SON BLCR for 38.423:  R3-237931 rev of R3-237511 (Ericsson)

Proposal for agreement:

SHR:

Target C-RNTI should be included in the Xn HANDOVER REPORT.

Target C-RNTI should be included in the NG HANDOVER REPORT.

Include timeSinceFailure in HO Report message (Xn and NG).

SPR:

Add a new IE in S-NODE MODIFICATION REQUIRED message to inform the MN that an SPR is available at the UE.

Phase 2

Whether the objective of T304 SPR trigger is also to optimize the mobility configurations in the source node?

The proponent propose to add the following text to stage 2:

In case of T304 trigger, the initialing node may also performs root cause analysis for mobility optimization.

The overall paragraph is as below:

For PSCell addition/CPA and PSCell change/CPC (MN or SN initiated), the target SN always decides the T304 trigger for SPR and performs root cause analysis. In case of T304 trigger, the initialing node may also performs root cause analysis for mobility optimization.

Q1: What’s your company view whether to have the above sentence in stage 2?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Ericsson | Yes | It is already possible for MN-initiated PSCell change as MN will always receive the SPR. It would not harm to also make it possible for SN-initiated PSCell Change. |
| CATT | Yes |  |
| Nokia | NO, but providing an alternative proposal. | As we stated during the offline discussion the only T304 SPR is not sufficient for proper mobility threshold optimization by initiating node. We propose to reformulate it to the following form: The initialing node may use the information contained in T304 SPR as an assistance information to perform also root cause analysis for mobility optimization. |
| Lenovo | No | - Unnecessary optimization in the initiating node may result in too late PSCell change/addition. - We should follow previous agreements that when SPR is triggered due to T304, the SPR would only be forwarded to target SN directly for RACH optimization. - Forwarding mechanism for SHR and SPR should be aligned, we are confused why we need the initialing node to perform root cause analysis for mobility optimization in case of T304 SPR trigger, but not need the source node to do mobility optimization in case of T304 SHR trigger. |
| Huwei | Yes | We think it is beneficial to allow this. Any negative impact is due to bad implementation. Anyone using this mobility optimization knows the risks |
| Samsung | No | We share the same view as Lenovo. In additional to what Lenovo comments, the other reasons: - There is no agreement in RAN3 to support near too early, near too late, near wrong cell handover scenario. It is not reasonable to discuss solutions before agreement on the scenario. - In Rel-17, the forwarding mechanism is not clear for SHR. In Rel-18, we make the forwarding mechanism clear for both SHR and SPR. The text not only break the previous agreement but also may bring a lot of mess for future discussions. |

Moderator summary:

Three companies are fine with the sentence, two companies don’t agree. One company provided an alternative. No agreement.

SN informs the UE (e.g. with MN involvement) that PSCell Change is SN-initiated, when MN is not aware of SN SPR configuration?

During the offline discussion on Wednesday, companied think the status in RAN2 need to be checked.

Is the MN/SN initiated flag always sent to the UE if UE support SPR? (check RAN2)

If yes, nothing is needed.

If no, further discuss whether there is any issue

Let’s check companies understanding after the internal check.

Q2: What’s your company’s understanding on the issue?

| Company | Comment |
| --- | --- |
| Ericsson | We’ve checked with our RAN2 delegates, the MN should only send MN/SN initiated indication when SPR has been configured. |
| CATT | OtherConfig-v18xy ::= SEQUENCE { successPSCell-Config-r18 SetupRelease {SuccessPSCell-Config-r18} OPTIONAL, -- Need M sn-InitiatedPSCellChange-r18 ENUMERATED {true} OPTIONAL --Need M Same understanding as E/// |
| Lenovo | We are not sure about the issue and intention. |

Q3: Based on above understanding, do you think further work needed in RAN3 e.g. specification impact or LS to RAN2?

| Company | Comment |
| --- | --- |
| Ericsson | SN does not always know that SPR has been configured by SN. So RAN2 needs to do something. Whether it is triggered by LS or directly in RAN2 via running CR does not matter |

Moderator summary:

No further work is needed in RAN3 for the time being.

Discussion

SHR

Correlation of SHR and RLF:

Target C-RNTI should be included in the Xn HANDOVER REPORT. Define as Mandatory IE.

Target C-RNTI should be included in the NG HANDOVER REPORT. Define as Mandatory IE.

After checking the specifications, the moderator think it is needed.

In NG Handover Report, RLF Report is optional. NG Handover Report can be used for RRC Reestablishment without RLF report case.

The question raised online is whether the target node can know the source node information for routing via 5GC. The target node can know the source CGI from the UE history information.

- 9.3.3.39	HO Report

This IE contains the HO report to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Handover Report Type | M |  | ENUMERATED (HO too early, HO to wrong cell, Inter-system ping-pong, …) |  |
| Handover Cause | M |  | Cause 9.3.1.2 | Indicates handover cause employed for handover from source cell |
| Source Cell CGI | M |  | NG-RAN CGI 9.3.1.73 | NG-RAN CGI of the source cell for handover procedure |
| Target Cell CGI | M |  | NG-RAN CGI 9.3.1.73 | NG-RAN CGI of the target cell for handover procedure. If the Handover Report Type is set to “Inter-system ping-pong”, it contains the target cell of the inter system handover from the other system to NG-RAN node cell |
| Re-establishment Cell CGI | C- ifHandoverReportType HoToWrongCell |  | NG-RAN CGI 9.3.1.73 | NG-RAN CGI of the cell where UE attempted re-establishment or where the UE successfully re-connected after the failure |
| Source Cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the source NG-RAN node |
| Target Cell in E-UTRAN | C- ifHandoverReportType Intersystempingpong |  | E-UTRA CGI 9.3.1.9 | E-UTRA CGI of the E-UTRAN target cell for handover procedure |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information provided in the HANDOVER REQUEST message from the source NG-RAN node |
| UE RLF Report Container | O |  | 9.3.3.41 | The UE RLF Report Container IE received in the FAILURE INDICATION message. |

Include timeSinceFailure in HO Report message (Xn and NG). Mandatory IE.

SHR: time from Handover command to SHR reporting

timeConnFailure-r16                  INTEGER (0..1023)

TimeSinceFailure-r16 ::= INTEGER (0..172800)

SPR

- Either or both the preferred T310/T312 SPR thresholds or T310/T312 timer values needs to be provided as assistance information from the source SN to the MN?

- Whether the objective of T304 SPR trigger is also to optimize the mobility configurations in the source node?

Check whether there is any stage2 text is acceptable?

Three options:

Option 1: no need to send message to S-SN

Option 2: Stage 2: The MN always send the message to the S-SN for SN initiated PSCell change/CPC

Option 3: the MN firstly check with T-SN whether there is RACH issue, if the problem is NOT in the T-SN, the MN send message to the S-SN.

We already has the following agreement on the forwarding:

Third node -> MN -> the node which generate the threshold which configure the threshold.

Text from stage 2 BLCR:

For PSCell addition/CPA and PSCell change/CPC (MN or SN initiated), the target SN always decides the T304 trigger for SPR and performs root cause analysis. In case of T304 trigger, the initialing node may also performs root cause analysis for mobility optimization.

Check further whether the above sentence for stage 2 can be agreed.

- SN informs the UE (e.g. with MN involvement) that PSCell Change is SN-initiated, when MN is not aware of SN SPR configuration?

Is the MN/SN initiated flag always sent to the UE if UE support SPR? (check RAN2)

If yes, nothing is needed.

If no, further discuss whether there is any issue

Considering that RAN2 has agreement: RAN2 agreed that the available flag can be sent via SRB3. The following can be agreed.

Add a new IE in S-NODE MODIFICATION REQUIRED message to inform the MN that an SPR is available at the UE.