3GPP TSG-RAN WG3 Meeting #123-bis	R3-241783

Changsha, China, 15 – 19 April 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1391 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Early TA acquisition Source DU may receive early TA value list for some candidate cells. In subsequent-LTM, UE will continue keeping the LTM candidate configuration after completing cell switching. If source DU does not transfer the stored early TA value list, UE will be expected to get early TA value again. Therefore, for avoidance of unnecessary early TA acquisition by the target DU, the source DU may provide the stored early TA values of candidate cells using LTM Cell Change Notification message to the target DU via CU. - RACH-less indication According to RAN2 discussion, if the LTM is performed without RACH procedure, the target cell may need to perform dynamic scheduling and assigns a UL grant to UE to transmit the first UL message after LTM without RACH. Therefore, it is beneficial to let target DU know if a triggered LTM is with or without RACH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For inter-DU LTM, add the valid TA values of other candidate cells in DU-CU CELL SWITCH NOTIFICATION message and CU-DU CELL SWITCH NOTIFICATION message. - Add RACH-less indication in the DU-CU CELL SWITCH NOTIFICATION and CU-DU CELL SWITCH NOTIFICATION messages. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has no impact with the previous version of the specification (same release) because it only impacts the LTM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Rel-18 LTM will not work properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.9.2, 8.3.10.2, 9.2.2.15, 9.2.2.16, 9.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.401 CR 0380 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<< START OF CHANGES>>>>>>

8.3.9	DU-CU Cell Switch Notification

8.3.9.1	General

The purpose of the DU-CU Cell Switch Notification procedure is to enable the gNB-DU to inform the gNB-CU about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the gNB-DU to the gNB-CU. The procedure uses UE-associated signalling.

8.3.9.2	Successful Operation

Figure 8.3.9.2-1: DU-CU Cell Switch Notification procedure. Successful operation.

The gNB-DU initiates the procedure by sending a DU-CU CELL SWITCH NOTIFICATION message.

Upon reception of the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, consider that a cell switch command was sent to the UE where the target cell is indicated by the included Cell ID IE.

If the LTM Cell Switch Information IE is included in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, forward it to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

If the Valid TA Information List IE is contained in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, consider this as the valid TA values for the LTM candidate cell(s).

If the Rachless Indication IE is contained in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, consider the LTM is triggered without random access procedure towards the target cell.

8.3.9.3	Unsuccessful Operation

Not applicable.

8.3.9.4	Abnormal Conditions

Not applicable.

<<<<<< NEXT CHANGE >>>>>>

8.3.10	CU-DU Cell Switch Notification

8.3.10.1	General

The purpose of the CU-DU Cell Switch Notification procedure is to enable the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the gNB-CU to the gNB-DU. The procedure uses UE-associated signalling.

8.3.10.2	Successful Operation

Figure 8.3.10.2-1: CU-DU Cell Switch Notification procedure. Successful operation.

The gNB-CU initiates the procedure by sending a CU-DU CELL SWITCH NOTIFICATION message.

Upon reception of the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, consider that a cell switch command was sent to the UE where the target cell is indicated by the included Cell ID IE.

If the LTM Cell Switch Information IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4].

If the Valid TA Information List IE is contained in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, consider this as the valid TA values for the LTM candidate cell(s).

If the Rachless Indication IE is contained in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, consider the LTM is triggered without random access procedure towards the target cell.

8.3.10.3	Unsuccessful Operation

Not applicable.

8.3.10.4	Abnormal Conditions

Not applicable.

<<<<<< NEXT CHANGE >>>>>>

9.2.2.15	DU-CU CELL SWITCH NOTIFICATION

This message is sent by the gNB-DU to inform the gNB-CU about the initiation of the cell switch  command to the UE.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | O |  | INTEGER (0..127) | Corresponds to the TCI-StateId IE as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | INTEGER (0..63) | Corresponds to the TCI-UL-StateId IE as defined in TS 38.331 [8]. |  |  |
| Valid TA Information List |  | 0..1 |  |  | YES | ignore |
| >Valid TA Information Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Time Alignment Timer | O |  | ENUMERATED (ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, …) | Indicates how long the TA value can be considered valid. |  |  |
| Rachless Indication | O |  | ENUMERATED (true, ...) | Indicates the LTM is triggered without random access towards the target cell. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

<<<<<< NEXT CHANGE >>>>>>

9.2.2.16	CU-DU CELL SWITCH NOTIFICATION

This message is sent by the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DLTCI State ID | O |  | INTEGER (0..127) | Corresponds to the TCI-StateId IE as defined in TS 38.331 [8]. | - | - |
| >UL TCI State ID | O |  | INTEGER (0..63) | Corresponds to the TCI-UL-StateId IE as defined in TS 38.331 [8]. | - | - |
| Valid TA Information List |  | 0..1 |  |  | YES | ignore |
| >Valid TA Information Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Time Alignment Timer | O |  | ENUMERATED (ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, …) | Indicates how long the TA value can be considered valid. |  |  |
| Rachless Indication | O |  | ENUMERATED (true, ...) | Indicates the LTM is triggered without random access towards the target cell. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

<<<<<< NEXT CHANGE >>>>>>

9.4.4	PDU Definitions

< skip unchanged part>

SRSPreconfiguration-List,

Broadcast-MRBs-Transport-Request-Item,

ValidTAInformation-List,

RachlessIndication

< skip unchanged part>

id-SRSInformation,

id-ValidityAreaSpecificSRSInformation,

id-ValidTAInformation-List,

id-RachlessIndication,

maxCellingNBDU,

maxnoofCandidateSpCells,

< skip unchanged part>

-- **************************************************************

--

##### -- DU-CU Cell Switch Notification

--

-- **************************************************************

DUCUCellSwitchNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ DUCUCellSwitchNotificationIEs}},

...

}

DUCUCellSwitchNotificationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-NRCGI							CRITICALITY reject	TYPE NRCGI							PRESENCE mandatory	}|

{ ID id-LTMCellSwitchInformation	 	CRITICALITY ignore	TYPE LTMCellSwitchInformation		PRESENCE optional		}|

{ ID id-ValidTAInformation-List			CRITICALITY ignore	TYPE ValidTAInformation-List		PRESENCE optional		}|

{ ID id-RachlessIndication				CRITICALITY ignore	TYPE RachlessIndication				PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- CU-DU CELL SWITCH NOTIFICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- CU-DU Cell Switch Notification

--

-- **************************************************************

CUDUCellSwitchNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ CUDUCellSwitchNotificationIEs}},

...

}

CUDUCellSwitchNotificationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-NRCGI							CRITICALITY reject	TYPE NRCGI							PRESENCE mandatory	}|

{ ID id-LTMCellSwitchInformation	 	CRITICALITY ignore	TYPE LTMCellSwitchInformation		PRESENCE optional		}|

{ ID id-ValidTAInformation-List			CRITICALITY ignore	TYPE ValidTAInformation-List		PRESENCE optional		}|

{ ID id-RachlessIndication				CRITICALITY ignore	TYPE RachlessIndication				PRESENCE optional		},

...

}

< skip unchanged part>

<<<<<< NEXT CHANGE >>>>>>

9.4.5	Information Element Definitions

< skip unchanged part>

RachlessIndication ::= ENUMERATED {true, ...}

ValidTAInformation-List ::= SEQUENCE (SIZE(1.. maxnoofLTMCells)) OF ValidTAInformationItem

ValidTAInformationItem	::= SEQUENCE {

nRCGI					NRCGI,

tAValue					TAValue,

tATimer					ENUMERATED {ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, …}		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { ValidTAInformationItem-ExtIEs} }	OPTIONAL,

...

}

ValidTAInformationItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

< skip unchanged part>

<<<<<< NEXT CHANGE >>>>>>

9.4.7	Constant Definitions

< skip unchanged part>

id-SRSInformation									ProtocolIE-ID ::= 832

id-ValidityAreaSpecificSRSInformation				ProtocolIE-ID ::= 833

id-ValidTAInformation-List							ProtocolIE-ID ::= xxx

END

-- ASN1STOP

<<<<<< END OF CHANGE >>>>