3GPP TSG-RAN3 Meeting #123	R3-240823

- Athens, Greece, 26th Feb – 1st Mar 2024

Agenda Item:	9.3.1

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion for CB: # 8_2RxXRUE

Document for:	Discussion

# Introduction

CB: # 8_2RxXRUE

- Check the CRs in R3-240822 and R3-240139

- N2-NGAP indication for ‘2Rx non-REDCAP XR devices’ from gNB to Core Network based on progress from RAN2?

(moderator - Nok)

Summary of offline disc R3-240823

# For the Chair’s Notes

# Discussion

## SPID

Q1: Please share your view on draft Stage-2 CR in the uploaded draft R3-240822 and R3-240139

| Company | Comment |
| --- | --- |
| Nokia | TS38.300 CR: ok TS36.300 CR: we think it may be better to clarify the new SPID value is not applicable to eNB/EPC, but we are ok if majority prefer no 36.300 CR. |
| ZTE | Agree with the 38.300 CR Share the same view as Nokia for the 36.300 CR |
| Deutsche Telekom | Same views as Nokia on 38.300 and having a CR for 36.300. |
| Ericsson | Share the same view as Nokia. Prefer to refer to RFSP rather than SPID for NR case. |
| Apple | TS 38.300 CR is OK, just a few minor editorial suggestions: “The SPID=x defined below only applies for 5GC.” “The selection provides information that subscriber is an XR device and is permitted to utilise a minimum of two Rx antenna ports for the NR bands where four Rx antenna ports are baseline as defined in TS 38.101-1 [88].” No strong view on the TS 36.300 CR, OK to follow the majority. |
| Huawei | TS38.300 CR: ok, I provide minor updates for the revision. TS36.300 CR: not needed. LTE not support XR, EPC and 5GC are separate system, it does not make sense to restrict the value in EPC. And, if the value among the reference values is not specified in 36.300, it means this will not be used for EPC naturally, so no change is needed. Values 1- 128 - Operator specific SPID values; Values 129 - 256 - Reference values. |
| Vodafone | TS38.300 CR should not say that SPID=252 only applies for 5GC. The eNB needs to know the SPID=252 too. Please consider the following scenario: The UE says that it is a 2Rx-XR device. But the SIM does not have an appropriate subscription and therefore the core network does not send SPID=252. So, the subscription doesn’t match with the UE type. Hence the operator might want not to give XR service or even no 5G to that user but some form of restricted access (e.g. select frequency band, send to 4G,…). Then it is logical that the 4G eNB should consider not handing this device over to 5G. TS36.300 CR: Add SPID=252 with reference to TS38.300. |
| Qualcomm | Same view as Nokia |

Summary

Potential proposals:

## NGAP indication for ‘2Rx non-REDCAP XR devices’ from gNB to Core Network

Based on the UE’s radio capability information, gNB send this indication to CN,

Upon the reception of this indication, the AMF may choose to reject the UE’s Registration Request in case the UE’s radio capability indication indicates 2Rx, but the subscription does not include the new SPID value.

Proposal 2: introduce a new indication indicating the UE is 2Rx in UE RADIO CAPABILITY INFO INDICATION message.

Q2: Please share your view on above proposal

| Company | Comment |
| --- | --- |
| Nokia | Agree. |
| ZTE | Agree |
| Deutsche Telekom | Agree |
| Ericsson | Agree |
| Apple | Agree. Please add Apple as a co-sourcing company. |
| Huawei | Agree. |
| Vodafone | Agree |
| Qualcomm | Agree. In addition to NGAP UE Radio Capability update to 5GC, we need to specify Xn, F1 enhancements for Served Cell Info and F1 Paging to include 2 Rx indication (there is similar discussion on going for RedCap as well) |

Summary

Potential proposals:

# TBD