3GPP TSG RAN3 Meeting#123                                                              R3-240904

- Athens, Greece, 26 February - 1 March, 2024

Agenda item:	23.2

Source:	Samsung, Ericsson

Title:	(TP to BL CR for TS 38.470) Support of LPHAP

Document for:	Others

# Introduction

This contribution provides the TP for BLCR to 38.470 on LPHAP.

# Text Proposal

Start of change

### 5.2.11	Positioning function

This function allows to transfer location management messages between gNB-CU and gNB-DU. With this function, gNB-CU request TRP information from gNB-DU, and gNB-DU response to gNB-CU with the TRP information. With this function, gNB-CU request positioning measurements from gNB-DU, and gNB-DU response to gNB-CU with the positioning measurements.

The function allows the gNB-CU to

-	transfer the positioning assistance data to gNB-DU. The gNB-DU is responsible for broadcasting the positioning assistance data according to the scheduling parameters available.

-	request the gNB-DU to configure SRS transmissions for UE.

-	request the gNB-DU to configure PRS transmissions.

-	request the gNB-DU to configure measurement gap or PRS processing window.

-	request the gNB-DU to activate or deactivate preconfigured measurement gap or PRS processing window.

-	request the gNB-DU to broadcast positioning system information.

-    request the gNB-DU to reserve or release the SRS resources for area-specific SRS transmissions.

End of change