3GPP TSG-RAN Meeting #123	R3-240320

Athens, Greece, 26th February – 1st March 2024

Agenda Item:	8.1

Source:	CATT

Title:	Discussion on service requirement of restricting satellite access RAT type

Document for:	Discussion

1. Introduction

In the recently LS R3-240045[1], SA2 requests RAN3 to investigate whether their specifications align with the SA1 requirements and the (updated) SA2 stage 2 specifications for designing restriction of satellite access RAT type.

The main point of the LS and CR is excerpt as below:

We checked our specifications and realised that some inter-system mobility scenarios had been omitted that meant that restrictions for satellite access would not work correctly. As a consequence, we have agreed the (attached) Rel 17 CR 5300 to TS 23.501 in order to support mobility restrictions from connected mode terrestrial NG-RAN to satellite access using EPC. For E-UTRA: - E-UTRA not allowed as primary access. - E-UTRA not allowed as secondary access. - E-UTRA in unlicensed bands not allowed as secondary access. - NB-IoT not allowed as primary access. - LTE-M not allowed as primary access. - WB-E-UTRAN (LEO) satellite access not allowed as primary access. - WB-E-UTRAN (MEO) satellite access not allowed as primary access. - WB-E-UTRAN (GEO) satellite access not allowed as primary access. - WB-E-UTRAN (OTHERSAT) satellite access not allowed as primary access. - NB-IoT (LEO) satellite access not allowed as primary access. - NB-IoT (MEO) satellite access not allowed as primary access. - NB-IoT (GEO) satellite access not allowed as primary access. - NB-IoT (OTHERSAT) satellite access not allowed as primary access. - LTE-M (LEO) satellite access not allowed as primary access. - LTE-M (MEO) satellite access not allowed as primary access. - LTE-M (GEO) satellite access not allowed as primary access. - LTE-M (OTHERSAT) satellite access not allowed as primary access. NOTE X: The use of WB-E-UTRAN satellite accesses, NB-IoT satellite accesses or LTE-M satellite accesses to connect to 5GC is not supported in this release of the specification.

In this paper, we will evaluate whether it is necessary to introduce new RAT Restriction for both NTN and IoT NTN.

2. Discussion

When we back to TN network, the RAT Restrictions is defined as BIT STRING {e-UTRA (0), nR (1), nR-unlicensed (2)}(SIZE(8, …)). ‘the RAT types “NB-IoT” and “LTE-M” are not included in the RAT Restrictions info in the NGAP even though SA2 defined them as RAT type in TS 23.501 clause 5.3.4.

For E-UTRA:

-	E-UTRA not allowed as primary access.

-	E-UTRA not allowed as secondary access.

-	E-UTRA in unlicensed bands not allowed as secondary access.

-	NB-IoT not allowed as primary access.

-	LTE-M not allowed as primary access.

Observation 1: For NR TN, the RAT type “NB-IoT” and “LTE-M” are not included in the RAT Restrictions info in NGAP even though SA2 defined them as RAT type.

When we support NR NTN in Rel-17 and Rel-18, the NTN RAT types “nR-LEO”, ”nR-MEO”, ”nR-GEO”, and “nR-OTHERSAT” were introduced in RAT Restriction Information IE and Extended RAT Restriction Information IE, as the tabular below:

| RAT Restrictions |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains RAT restriction related information as specified in TS 23.501 [9]. | - |  |
| --- | --- | --- | --- | --- | --- | --- |
| >PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >RAT Restriction Information | M |  | BIT STRING { e-UTRA (0), nR (1), nR-unlicensed (2), nR-LEO (3), nR-MEO (4), nR-GEO (5), nR-OTHERSAT (6)} (SIZE(8, …)) | Each position in the bitmap represents a RAT. If a bit is set to "1", the respective RAT is restricted for the UE. If a bit is set to "0", the respective RAT is not restricted for the UE. Bit 7 reserved for future use. | - |  |
| >Extended RAT Restriction Information | O |  | 9.3.1.126 | If this IE is included, the RAT Restriction Information IE is ignored. | YES | ignore |

9.3.1.126	Extended RAT Restriction Information

This IE provides RAT restrictions as specified in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Primary RAT Restriction | M |  | BIT STRING { e-UTRA (0), nR (1), nR-unlicensed (2), nR-LEO (3), nR-MEO (4), nR-GEO (5), nR-OTHERSAT (6)} (SIZE(8, …)) | Each position in the bitmap represents a Primary RAT. If a bit is set to "1", the respective RAT is restricted for the UE. If a bit is set to "0", the respective RAT is not restricted for the UE. Bit 7 reserved for future use. The Primary RAT is the RAT used in the access cell, or target cell. |
| Secondary RAT Restriction | M |  | BIT STRING { e-UTRA (0), nR (1), e-UTRA-unlicensed (2), nR-unlicensed (3)} (SIZE(8, …)) | Each position in the bitmap represents a Secondary RAT. If a bit is set to "1", the respective RAT is restricted for the UE. If a bit is set to "0", the respective RAT is not restricted for the UE. Bits 4-7 reserved for future use. A Secondary RAT is a RAT, distinct from the UE’s primary RAT, used in any cell serving the UE excluding the PCell. |

Observation 2: For NR NTN, the RAT types “nR-LEO”, ”nR-MEO”, ”nR-GEO”, and “nR-OTHERSAT” were introduced in RAT Restriction Information IE and Extended RAT Restriction Information IE.

Similarly, in IoT NTN, the new RAT types LEO, MEO, GEO, OTHERSAT are defined, aims to restrict the Handover of eMTC NTN, and we assumes the handover to NB-IoT NTN is not supported. The handover from EUTRAN TN to NR NTN is not considered in Rel-17 and Rel-18.

| Forbidden inter RATs | O |  | ENUMERATED(ALL, GERAN, UTRAN, CDMA2000, …, GERAN and UTRAN, CDMA2000 and UTRAN) | Inter-3GPP and 3GPP2 RAT access restrictions. “ALL” means that all RATs mentioned in the enumeration of this IE are restricted. |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR Restriction in EPS as Secondary RAT | O |  | ENUMERATED(NRrestrictedinEPSasSecondaryRAT, …) | Restriction to use NR when the NR is used as secondary RAT in EN-DC. | YES | ignore |
| Unlicensed Spectrum Restriction | O |  | ENUMERATED(UnlicensedRestricted, …) | Restriction to use unlicensed spectrum in the form of LAA or LWA/LWIP or NR-U as described in TS 23.401 [11]. | YES | ignore |
| NR Restriction in 5GS | O |  | ENUMERATED(NRrestrictedin5GS, …) | Restriction to use NR when the NR connects to 5GS. | YES | ignore |
| RAT Restrictions |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains RAT restriction related information as specified in TS 23.401 [11]. | YES | ignore |
| >PLMN Identity | M |  | 9.2.3.8 |  | - |  |
| >RAT Restriction Information | M |  | BIT STRING { LEO (0), MEO (1), GEO (2), OTHERSAT (3)} (SIZE(8, …)) | Each position in the bitmap represents a RAT. If a bit is set to "1", the respective RAT is restricted for the UE. If a bit is set to "0", the respective RAT is not restricted for the UE. Bits 4-7 reserved for future use. | - |  |

Observation 3: For IoT NTN, the new RAT types LEO, MEO, GEO, OTHERSAT are defined in the RAT Restriction Information.

For the new RAT restriction introduced in SA2 [1], RAN3 seems no need to have such fine grained division. First, the WB-E-UTRAN (LEO/MEO/GEO) satellite access is never discussed in RAN WGs, and the potential impact in RAN is not clear for us, it is not suitable to introduce it without further study. What’s more, following the legacy TN RAT restrictions, it is no need to divided e-UTRA into NB-IoT and eMTC.

Observation 4: WB-E-UTRAN (LEO/MEO/GEO) satellite access is never discussed and supported in RAN WGs.

More importance, the inter-system TN NTN HO still not discussed in RAN3, especially for NR TN to LTE NTN case. Supporting this scenario will trigger complexity discussion, e.g., UE capability, satellite capability, data transfer. If we only consider prevented the above scenario, it will never happen due to technical issue, so no need to indicate it explicated. After all, in 3GPP scope, we always focused on what we support, instead of listing all of the non-support.

Observation 5:Inter-system HO between NR TN and IoT NTN (NB-IoT NTN, EMTC NTN) is never discussed and supported in RAN WGs.

For the LTE TN to NR NTN case, we notice there is a relative discussion in RAN plenty meeting and will approval a new WI, but it only consider the IDLE mode mobility. The main information is excerpt as below, whether need any enhance from the perspective of RAT restriction can wait for further discussion.

- Mobility enhancements [RAN2, RAN4] (in RP-234053) - Specify support for IDLE mode mobility from E-UTRAN TN to NR NTN, where E-UTRAN TN provides satellite information in SIB. RP-234053 New WID: Non-Terrestrial Networks (NTN) for NR Phase 3 RAN chair: will remove objective 3. now but we will create a joint new LTE only WID in March 24 conclusion: An LTE WID for REL-19 for objective 3. of RP-234053 will be approved in March 24, so objective 3. will be removed from this REL-19 WID RP-23405

Observation 6: LTE TN to NR NTN HO is not supported in Rel-17 and Rel-18, RAN may further consider the idle mode mobility from E-UTRAN TN to NR NTN in Rel-19.

Base on the discussion above, we see it’s not necessary to introduce the new added RAT types in [1] to the RAT Restrictions Information in NGAP and S1AP.

Proposal 1: Not necessary to introduce the new added RAT types e.g. WB-EUTRAN (LEO/MEO/GEO) to the RAT Restrictions Information in NGAP and S1AP.

Correspondingly, we should provide our views and reply the LS to SA2, the draft LS is provided in [3].

Proposal 2: Reply LS to SA2 about our understanding, the draft LS is provided in R3-240376.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Observation 1: For NR TN, the RAT type “NB-IoT” and “LTE-M” are not included in the RAT Restrictions info in NGAP even though SA2 defined them as RAT type.

Observation 2: For NR NTN, the RAT types “nR-LEO”, ”nR-MEO”, ”nR-GEO”, and “nR-OTHERSAT” were introduced in RAT Restriction Information and Extended RAT Restriction Information.

Observation 3: For IoT NTN, the new RAT types LEO, MEO, GEO, OTHERSAT are defined in the RAT Restriction Information.

Observation 4: WB-E-UTRAN (LEO/MEO/GEO) satellite access is never discussed and supported in RAN WGs.

Observation 5:Inter-system HO between NR TN and IoT NTN (NB-IoT NTN, EMTC NTN) is never discussed and supported in RAN WGs.

Observation 6: LTE TN to NR NTN HO is not supported in Rel-17 and Rel-18, RAN may further consider the idle mode mobility from E-UTRAN TN to NR NTN in Rel-19.

Proposal 1: Not necessary to introduce the new added RAT types e.g. WB-EUTRAN (LEO/MEO/GEO) to the RAT Restrictions Information in NGAP and S1AP.

Proposal 2: Reply LS to SA2 about our understanding, the draft LS is provided in R3-240376.

4. Reference

[1]	R3-240045

[2]	RP-234053

[3]	R3-240376