3GPP TSG-RAN WG3 Meeting #124															R3-243209

Fukuoka, Japan, 20 – 24 May, 2024

Title: 	Discussion on LS on MDT measurements collection in MR-DC

Source: 	Huawei

Agenda item:	8.1

Document Type:	Discussion

1. Introduction

This document discusses the RAN3 impact on the MDT configuration in NR-DC regarding the incoming LS from SA5 in [1].

2. Background

In SONMDT Rel-18 WI phase, the signalling based immediate MDT in NR-DC was discussed. The main issue is whether it is beneficial for the OAM to indicate whether an MDT configuration should be triggered for the MN node only or for the SN node only or for both.

There was no consensus in RAN3 on the need of such an explicit indication from OAM to direct the gNB to forward the MDT-Configuration NR IE to SN in NR-DC case. A LS was sent to SA5 for confirmation on the need of the indication at RAN3 ##119bis-e meeting.

The incoming LS from SA5 in [1] is the reply LS for this remaining issue.

3. Discussion

According to TS 38.413, in the Trace Activation IE sent by AMF to NG-RAN node, there are a MDT Configuration-NR IE and a MDT Configuration-EUTRA IE.

9.3.1.167	MDT Configuration

This IE defines the MDT configuration parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MDT Configuration-NR | O |  | 9.3.1.169 |  |
| MDT Configuration-EUTRA | O |  | 9.3.1.170 |  |

For NGEN-DC and NE-DC, since the MN and SN work in different RATs, the OAM can use the above two IEs to separately set the MDT configurations. However, for NR-DC, RAN3 agreed that only one MDT Configuration-NR is needed for both MN and SN. It is not clear for the NG-RAN node about the intention of OAM on collecting MDT data from MN only, SN only or both nodes.

It was discussed and confirmed in previous discussions that sometimes OAM may prefer to collect the measurement results from a specific node only, e.g. MN, rather than from both nodes [1]. For example, in FR1 with FR2 NR-DC case, the Secondary NR cell path-loss model on FR2 may be different and different measurement threshold from MN side could be configured for SN.

Therefore, a single indicator “MN only” deems necessary. And in the reply LS from SA5 in [1] confirms the need.

Q: RAN3 discussed whether it is beneficial for the OAM to indicate whether an MDT configuration should be triggered for the MN node only or for the SN node only or for both.

A: SA5 think that an MDT configuration could be triggered for both MN and SN which is the default scenario, and for MN node only which facilitates flexible control of the MDT data collection.

- Proposal 1: For S-based MDT in NR-DC, an indicator meaning “MN only” is introduced in MDT configuration.

3. Conclusion

It is proposed to agree on the following proposal as per the incoming LS from SA5:

- Proposal 1: For S-based MDT in NR-DC, an indicator meaning “MN only” is introduced in MDT configuration.

The corresponding CR is provided in [2].

4. Reference

[1]	S5-241927 Reply LS on MDT measurements collection in MR-DC

[2]	R3-243210. Correction on MDT configuration in NR-DC