3GPP TSG-RAN3 Meeting #124	R3-243355

# Fukuoka, Japan, 20 - 24 May, 2024

Agenda item:	14.3

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS 38.300 and TS 38.413) Discussion on the support of Regenerative payload

Document for:	Discussion and Decision

# 1	Introduction

Last RAN Plenary agreed Rel-19 NR NTN WID to support regenerative payload with following objective:

Support of regenerative payload [RAN3, RAN2, RAN4]

-	Specify the support of gNB on board in TS 38.300

-	Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3]

Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

This contribution analyses the issues and the impact to RAN3. RAN3 also received a LS ([3]) from SA2 on security of IP transport over satellite transport links, this contribution also proposes a way forward.

# 2	Management of NG interface

A NTN system based on NGSO may have thousands of satellites or tens of thousands of satellites. For example, Starlink has launched ~5400 satellites, and plan to expand the number of satellites to 12000. Due to earth rotation, every satellite in a NGSO system can visit a specific geographical area but at different time. Similar to terrestrial network, an AMF in NTN serving UEs in specific areas. For example, an AMF deployed in USA may only serve the UEs that are located in the USA. For every satellite to serve USA, it needs to have setup NG with the AMF in the USA. Currently, there is no method to remove an established NG interface. This means an AMF need to maintain a very large number of NG interfaces (including IPSec, TNL connection), e.g. NG interface with every satellite in NGSO system.

On the other hand, at any specific timing duration (e.g. 1-hour), only a limited number of satellites pass USA and need to setup NG (or have “active” NG) with the AMF in USA.

It is very inefficient for AMF to manage a very large number of NG interfaces, while only a very small number of NG interfaces are “active” at any timing duration.

Observation 1-1: considering the potential very large number of satellites in NGSO, it is inefficient for an AMF to manage the NG with all satellites in a NGSO system.

It is similar situation to the gNB. There can be many (tens of AMFs or hundreds of AMFs) around the world, a NGSO satellite need to setup NG with AMFs around the world. At any specific timing duration (e.g. 1-hour), the gNB only need to have one or a few “active” NG interfaces with the AMFs in that specific geographical area. It is also not efficient for a NGSO satellite to maintain NG with an AMF when it does not serve the AMF’s serving area for a long duration (e.g. 12-hour).

Observation 1-2: considering the large number of AMFs around the world, it is inefficient for a gNB to manage the NG with all AMFs around the world.

To reduce the complexity in both gNB and AMF, there is no need to keep the NG when the NG is “not active” for a long period of time. Up to the implementation/configuration, the gNB may decide to remove the NG when it moves out of the serving area of the AMF. By removing the NG, both gNB and AMF does not need to maintain the NGAP application level data, and no need to maintain the NG TNL connection (i.e. IPSec tunnel, SCTP association)

Observation 1-3: It can reduce the complexity to both gNB and AMF if NG interface can be removed.

One may argue that the NG connection with the old AMF may be suspended when the satellite/gNB leaves, and be resumed when the satellite/gNB re-visit the area. However, there is no benefit for the NG suspend/resume. First, the argument for introducing the NG suspend/resume is to keep the NG application level configuration data when the gNB left. But it does not save the NG signaling (i.e. initiate a NG Setup procedure vs. initiate a NG Resume procedure). Second, the saved configuration data may be useless, especially in NSGO. This is due to two reasons: 1) The application level data (e.g. TACs) saved during suspend procedure is useless when the gNB resumes NG. For example, the last geo-area that satellites serves before it leaves USA is New York, but the first geo-area that the satellite serves for its next visit may be start from San Francisco. So the saved TACs in AMF during suspend is useless when the satellite start to serve USA from San Franciso in its next pass.  2) Due to the earth rotation, the geo-area that a NGSO satellite visit in each pass is different. This also causes the saved application level data useless.

Observation 1-4: there is no benefit to introduce NG suspend/resume procedure.

Based on above analysis, we propose to introduce a NG Removal procedure to reduce the complexity in both gNB and AMF.

Proposal 1: introduce a NG Removal procedure.

A draft NGAP TP can be found in Annex B – TP for TS 38.413

# 3	Feeder Link Switch Over

In Regenerative architecture, the TNL address of a satellite/gNB is anchored in the NTN GW, which ensures the DL NG-C/U IP packet with destination set to the gNB’s TNL address is routed to the correct NTN GW. The FLSO causes the changes of the gNB’s TNL address, i.e. from the old TNL address anchored in old NTN GW to the new TNL address anchored in the new NTN GW. The FLSO may also cause the change of the UE’s AMF. There are two scenarios:

- Scenario 1: FLSO without relocation of 5GC

After the FLSO, the gNB need to setup TNL association by using the TNL address. The NG between the gNB and the AMF is maintained.

- In case Soft FLSO is performed, the gNB can connect with the new NTN GW, while the connection with the old NTN GW is maintained during the transition period. After the gNB receives the new TNL address anchored in the new NTN GW, the gNB can reuse the existing multiple TLNA mechanism, i.e. the mechanism defined in TS 38.412 (copied as below)

** TS 38.412 **

When the configuration with multiple SCTP endpoints per NG-RAN node is supported and the NG-RAN node wants to add additional SCTP endpoints, the RAN configuration update procedure shall be the first NGAP procedure triggered on an additional TNLA of an already setup NG-C interface instance after the TNL association has become operational, and the AMF shall associate the TNLA to the NG-C interface instance using the included Global RAN node ID.

** TS 38.412 **

An example call flow is shown below:

Figure 1: Soft FLSO without changing 5GC

Step 2: The gNB setup the new TNL by using the new TNL address.

Step 3 and 4: The gNB send a RAN CONFIGURATION UPDATE message over the new TNL. The AMF can know the new TNL association is available in the gNB.

Step 5: The gNB needs to inform the UPF for the change of TNL address in the DL F-TEID for all connected UEs. This can be performed by the gNB initiating the PDU Session Resource Modify Indication procedure for each UE. During the soft switch period, it is possible that the DL NG-U packets for some UEs are still sent to old NTN GW while the DL NG-U packets for other UEs are sent to new NTN GW.

Step 7 and 8: After the TNL address of the DL F-TEID is updated, the gNB can initiate a RAN Configuration Update procedure to remove the old TNL address.

Proposal 2-1: the soft FLSO can be supported by reusing existing multiple SCTP association, i.e. gNB setup new SCTP association with AMF by using the new TNL address anchored in new NTN Gateway, in addition to the existing SCTP association established with AMF by using the old TNL address anchored in old NTN Gateway.

- In case Hard FLSO is performed, the gNB only connect with the new NTN GW after the FLSO. Since 5GC does not know the hard FLSO, the 5GC may continue sending the DL NG-C/U packet to the old NTN GW. The old NTN GW has no way to deliver it to the gNB or forward it to the new NTN GW. This causes packet loss. It may be better to inform the 5GC before the hard FLSO, so the 5GC can suspend the DL NG-C/U. To reduce the interruption, it is also necessary to reduce the time for the path switch procedure (i.e. from the old transport path via old NTN GW to the new transport path via the new NTN GW). It may be many UEs connected with the gNB before the FLSO, performing the per-UE PDU Session Resource Modification Indication procedure may cause signaling storm and be inefficient.

It is worthy to note that there is similar issue discussed and concluded for mobile IAB in Rel-18. After the mobile IAB performed partial migration and received a new TNL address, the mobile IAB need to inform the gNB-CU-UP to use the new TNL address for DL F1-U. RAN3 agreed a new non-UE associated F1AP IAB UP Configuration Update procedure to update the DL TNL address for F1-U tunnels of all connected UEs. It is beneficial to introduce a similar mechanism in NG, e.g. introduce a non-UE associated NGAP procedure to update the DL TNL address for NG-U tunnels of all connected UEs.  This procedure can also be used in Soft FLSO.

Proposal 2-2: introduce a new non-UE associated NGAP procedure to update the DL TNL address for NG-U tunnels of all connected UEs.

An example call flow is shown as below.

Figure 2: Hard FLSO without changing 5GC

Step 1: before the hard FLSO, the gNB informs the 5GC about the hard FLSO, so the 5GC can suspend DL NG-C/U transmission. It is possible to enhance the RAN Configuration Update procedure.

Step 2: The gNB disconnect from old NTN Gateway, and connect with new NTN Gateway. The gNB receives a new TNL address anchored in the new NTN Gateway.

Step 3: The gNB setup the new TNL by using the new TNL address.

Step 4 and 5: The gNB send a RAN CONFIGURATION UPDATE message over the new TNL. The AMF can know the new TNL association is available in the gNB.

Step 6: The gNB request the 5GC (e.g. UPF) to use the new TNL address in DL-F-TEID for all connected UEs.  It is possible to reuse the RAN Configuration Update procedure performed in Step 4 for this purpose.

Step 7 and 8: After the TNL address of the DL F-TEID is updated, the gNB can initiate a RAN Configuration Update procedure to remove the old TNL address.

- Scenario 2: FLSO with relocation of 5GC

This scenario may be considered as low priority scenario. For a stationary UE, its serving 5GC should remains unchanged. So the FLSO should not cause the relocation of 5GC.  In case the 5GC is changed, it may cause frequent 5GC relocation, e.g. when the next satellites move in.

In case the 5GC relocation during FLSO is required, Scenario 2 requires additional procedures to be performed to relocate the UE’s AMF after FLSO, in additional to the procedures performed in Scenario 1.

- In case Soft FLSO is performed, NG-HO need to be performed in order to change the UE’s AMF. This is similar to Transparent architecture as defined in TS 38.300:

For an RRC_CONNECTED UE, when the gNB is configured to ensure that the UE connects to an AMF that serves the country in which the UE is located, if the gNB detects that the UE is in a different country to that served by the serving AMF, then it should perform an NG handover to change to an appropriate AMF.

In order to perform the NG handover, the satellite need to host two logical gNBs, i.e. source gNB connected with the source AMF via old NTN GW, and target gNB connected with target AMF via new NTN GW. This is also similar to Rel-18 mobile IAB that a mobile IAB hosting two logical IAB-DUs in order to perform IAB-DU migration. By using two logical gNBs in the same satellite, existing NG-Handover procedure can be reused.

Figure 3: AMF relocation

So in case of soft FLSO with AMF relocation, existing NG-HO procedure can be performed by using two logical gNBs on the same satellite

- In case Hard FLSO is performed, it is still possible to reuse above procedure to perform NG-HO by using two logical gNBs on the same satellite. After the hard FLSO is completed, both source gNB and target gNB uses the new TNL address anchored in the new NTN GW for NG connection.

In a summary, the relocation of the AMF can be supported by NG-HO with two logical gNBs embarked on the same satellite.

Proposal 2-3: in case 5GC relocation is needed, the relocation of the AMF can be supported by NG-HO with two logical gNBs embarked on the same satellite.

Stage-2 TP for FLSO can be found in Annex A – TP for TS 38.300

Proposal 2-4: Agree the Stage-2 TP.

# 4	INACTIVE UE

In Regenerative architecture, a UE may transition to RRC INACTIVE via Satellite1/gNB1, then resume in another Satellite4/gNB4. The Xn may only exist between the neighboring gNBs/Satellites. There may be no Xn between gNB1 and gNB4. This can cause the failure of Retrieve UE context. Since the satellite’s trajectory is predictable. It can be predicted on which satellite will serve the UE’s geographical area in the future. One possible solution is to proactively push the UE context with updated AS security context to the next satellite via Xn interface at the time of satellite switch.

Proposal 3: RAN3 study how to support RRC INACTIVE UE in Regenerative architecture.

# 5	Discussion on SA2 LS R3-243023/S2-2405836

RAN3 received a SA2 LS ([3]) about the security of IP transport over satellite transport links. The SA2 LS is a reply LS to SA3:

SA3 is studying security and privacy aspects of 5G Satellite Access Phase 3 in Rel-19. 5G NTN regenerative payload on-board orbiting satellite communication is IP based. Feeder link and Inter-Satellite Link (ISL) are satellite transport links. The satellite transport links are not in scope of 3GPP. However, the IP transport allowing on-board 5G NTN payload to communicate with ground-based 3GPP network passes through these satellite transport links.

The regenerative NTN payload on-board orbiting satellite in a constellation operates in a dynamic environment due to satellite movement. SA3 is considering reusing network domain security as defined in TS 33.210 as a baseline to protect 3GPP system interfaces. However, fit for purpose effective security protection at the transport layer (e.g., DTLS) and at the network layer (e.g., IPsec) rely on underlying IP connectivity.

SA3 would like to seek feedback from SA2 on the architectural assumptions related to persistency, reliability, and availability of IP connectivity from satellite hosted 3GPP systems to ground-based 3GPP network in generic regenerative and UE-satellite-UE use cases.

SA2 replies:

SA2 would also like to point out that signalling transport and layer 1 specifications of N2 and S1 (including TSs 38.412 and 36.412) are in the scope of RAN3. SA2 would therefore like to give RAN3 the opportunity to provide feedback on SA3's LS.

From RAN3 perspective, the NGAP/XnAP uses the IP transport network. The Feeder Link or Inter-Satellite Link (ISL) is a segment of the IP transport network. In terrestrial network, the transport network connection between the NG-RAN node and the CN for the NG (or between the NG-RAN nodes for Xn) may also include a segment using wireless connection, e.g. a microwave backhaul. So there is no difference for the transport network including Feeder Link or ISL, vs. a group transport network including a wireless backhaul. So it is proposed to reply SA2/SA3 that the transport network including a feeder link or ISL is similar to a ground transport network including a wireless backhaul.

Proposal 4: reply SA2/SA3 that the transport network including a feeder link or ISL is similar to a ground transport network including a wireless backhaul.

# 6	Conclusion

In this contribution, we discussed the issues to support regenerative architecture. Our proposals are:

Observation 1-1: considering the potential very large number of satellites in NGSO, it is inefficient for an AMF to manage the NG with all satellites in a NGSO system.

Observation 1-2: considering the large number of AMFs around the world, it is inefficient for a gNB to manage the NG with all AMFs around the world.

Observation 1-3: It can reduce the complexity to both gNB and AMF if NG interface can be removed.

Observation 1-4: there is no benefit to introduce NG suspend/resume procedure.

Proposal 1: introduce a NG Removal procedure.

A draft NGAP CR can be found TP can be found in Annex B – TP for TS 38.413

Proposal 2-1: the soft FLSO can be supported by reusing existing multiple SCTP assocaition, i.e. gNB setup new SCTP association with AMF by using the new TNL address anchored in new NTN Gateway, in addition to the existing SCTP association established with AMF by using the old TNL address anchored in old NTN Gateway.

Proposal 2-2: introduce a new non-UE associated NGAP procedure to update the DL TNL address for NG-U tunnels of all connected UEs.

Proposal 2-3: the relocation of the AMF can be supported by NG-HO with two logical gNBs embarked on the same satellite.

Stage-2 TP for FLSO can be found in Annex A – TP for TS 38.300

Proposal 2-4: Agree the Stage-2 TP.

Proposal 3: RAN3 study how to support RRC INACTIVE UE in Regenerative architecture.

For the SA2 LS,

Proposal 4: reply SA2/SA3 that the transport network including a feeder link or ISL is similar to a ground transport network including a wireless backhaul.

# References

- TS 38.300

- TS 38.413

- R3-243023/S2-2405836, Reply LS on security of IP transport over satellite transport links (S3-240950 / S2-240390)

# Annex A – TP for TS 38.300

### 16.14.4	Switchover

#### 16.14.4.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for a specific NTN payload. In Transparent NTN payload, the feeder link switchover is a Transport Network Layer procedure. In Regenerative NTN payload, the feeder link switchover causes the change of the TNL address of the gNB, i.e. from the old TNL address anchored in the old NTN Gateway to the new TNL address anchored in the new NTN Gateway.

Service link switch refers to a change of the serving NTN payload.

Both hard and soft feeder link switchover are supported in NTN.

#### 16.14.4.2	Assumptions

A feeder link switch over may result in transferring the established connection for the affected UEs between two gNBs.

For soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switch over, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption may occur during the transition between the feeder links.

#### 16.14.4.3	Procedures

In Transparent NTN payload, the NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over between two gNBs is performed. The transfer of the affected UE(s)' context between the two gNBs at feeder link switch over is performed by means of either NG based or Xn based handover, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function.

In Regenerative NTN payload, the NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over is performed.

-	In case of soft feeder link switch without relocation of the AMF, the gNB can setup the additional new TNL association with the AMF by using the new TNL address. The gNB initiates the NGAP procedure over the new TNL association to inform the 5GC about the update of the gNB’s TNL address. After the 5GC has completed the update, the gNB initiates the RAN node configuration Update procedure to remove the TNL association using the old TNL address and disconnect from the old NTN Gateway.

-	In case of hard feeder link switch over without relocation of the AMF, the gNB may inform the AMF about the hard FLSO is to be performed. After the gNB receive the new TNL address anchored in the new NTN Gateway, the gNB setup the new TNL association with the AMF by using the new TNL address. The gNB initiates the NGAP procedure over the new TNL association to inform the 5GC about the update of the gNB’s TNL address. After the 5GC has completed the update, the gNB initiates the RAN node configuration Update procedure to remove the TNL association using the old TNL address.

-	The feeder link switch over may cause the relocation of the AMF. The Regenerative NTN payload concurrently supports two logical gNBs. The source logical gNB have NG connection with the source AMF. The target gNB have the NG connection with target AMF. The source logical gNB initiates the NG handover to target logical gNB for the connected UE. In case of soft FLSO, the re-location is performed during the transition between the feeder links. The source NG connection uses the TNL address anchored in the old NTN Gateway, and the target NG connection uses the TNL address anchored in the new NTN Gateway. In case of hard FLSO, the re-location is performed after FLSO. Both source NG connection and target NG connection use the TNL address anchored in the new NTN Gateway.

# Annex B – TP for TS 38.413

Start of Change

## 8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| Broadcast Session Setup | BROADCAST SESSION SETUP REQUEST | BROADCAST SESSION SETUP RESPONSE | BROADCAST SESSION SETUP FAILURE |
| Broadcast Session Modification | BROADCAST SESSION MODIFICATION REQUEST | BROADCAST SESSION MODIFICATION RESPONSE | BROADCAST SESSION MODIFICATION FAILURE |
| Broadcast Session Release | BROADCAST SESSION RELEASE REQUEST | BROADCAST SESSION RELEASE RESPONSE |  |
| Distribution Setup | DISTRIBUTION SETUP REQUEST | DISTRIBUTION SETUP RESPONSE | DISTRIBUTION SETUP FAILURE |
| Distribution Release | DISTRIBUTION RELEASE REQUEST | DISTRIBUTION RELEASE RESPONSE |  |
| Multicast Session Activation | MULTICAST SESSION ACTIVATION REQUEST | MULTICAST SESSION ACTIVATION RESPONSE | MULTICAST SESSION ACTIVATION FAILURE |
| Multicast Session Deactivation | MULTICAST SESSION DEACTIVATION REQUEST | MULTICAST SESSION DEACTIVATION RESPONSE |  |
| Multicast Session Update | MULTICAST SESSION UPDATE REQUEST | MULTICAST SESSION UPDATE RESPONSE | MULTICAST SESSION UPDATE FAILURE |
| Timing Synchronisation Status | TIMING SYNCHRONISATION STATUS REQUEST | TIMING SYNCHRONISATION STATUS RESPONSE | TIMING SYNCHRONISATION STATUS FAILURE |
| MT Communication Handling | MT COMMUNICATION HANDLING REQUEST | MT COMMUNICATION HANDLING RESPONSE | MT COMMUNICATION HANDLING FAILURE |
| Broadcast Session Transport | BROADCAST SESSION TRANSPORT REQUEST | BROADCAST SESSION TRANSPORT RESPONSE | BROADCAST SESSION TRANSPORT FAILURE |
| NG Removal | NG REMOVAL REQUEST | NG REMOVAL RESPONSE | NG REMOVAL FAILURE |

Table 8.1-2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Downlink RAN Configuration Transfer | DOWNLINK RAN CONFIGURATION TRANSFER |
| Downlink RAN Status Transfer | DOWNLINK RAN STATUS TRANSFER |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Error Indication | ERROR INDICATION |
| Uplink RAN Configuration Transfer | UPLINK RAN CONFIGURATION TRANSFER |
| Uplink RAN Status Transfer | UPLINK RAN STATUS TRANSFER |
| Handover Notification | HANDOVER NOTIFY |
| Initial UE Message | INITIAL UE MESSAGE |
| NAS Non Delivery Indication | NAS NON DELIVERY INDICATION |
| Paging | PAGING |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |
| Reroute NAS Request | REROUTE NAS REQUEST |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| AMF Status Indication | AMF STATUS INDICATION |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Downlink UE Associated NRPPa Transport | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT |
| Uplink UE Associated NRPPa Transport | UPLINK UE ASSOCIATED NRPPA TRANSPORT |
| Downlink Non UE Associated NRPPa Transport | DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Uplink Non UE Associated NRPPa Transport | UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| UE TNLA Binding Release | UE TNLA BINDING RELEASE REQUEST |
| UE Radio Capability Info Indication | UE RADIO CAPABILITY INFO INDICATION |
| RRC Inactive Transition Report | RRC INACTIVE TRANSITION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Uplink RIM Information Transfer | UPLINK RIM INFORMATION TRANSFER |
| Downlink RIM Information Transfer | DOWNLINK RIM INFORMATION TRANSFER |
| Retrieve UE Information | RETRIEVE UE INFORMATION |
| UE Information Transfer | UE INFORMATION TRANSFER |
| RAN CP Relocation Indication | RAN CP RELOCATION INDICATION |
| Connection Establishment Indication | CONNECTION ESTABLISHMENT INDICATION |
| AMF CP Relocation Indication | AMF CP RELOCATION INDICATION |
| Handover Success | HANDOVER SUCCESS |
| Uplink RAN Early Status Transfer | UPLINK RAN EARLY STATUS TRANSFER |
| Downlink RAN Early Status Transfer | DOWNLINK RAN EARLY STATUS TRANSFER |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Session Release Required | BROADCAST SESSION RELEASE REQUIRED |
| Timing Synchronisation Status Report | TIMING SYNCHRONISATION STATUS REPORT |
| RAN Paging Request | RAN PAGING REQUEST |

Next Change

### 8.7.X	NG Removal

#### 8.7.X.1	General

The purpose of the NG Removal procedure is to remove the NG interface instance between the NG-RAN node and the AMF in a controlled manner. If successful, this procedure erases any existing application level configuration data in the two nodes.

The procedure uses non UE-associated signaling.

#### 8.7.X.2	Successful Operation

Figure 8.7.x.2-1: NG Removal, successful operation

The NG-RAN node initiates the procedure by sending the NG REMOVAL REQUEST message to the AMF. Upon reception of the NG REMOVAL REQUEST message the AMF shall reply with the NG REMOVAL RESPONSE message. After receiving the NG REMOVAL RESPONSE message, the NG-RAN node shall initiate removal of the TNL association towards the AMF and may remove all resources associated with the NG interface. The AMF may then remove all resources associated with the NG interface.

#### 8.7.X.3	Unsuccessful Operation

Figure 8.7.x.3-1: NG Removal, unsuccessful operation

If the AMF cannot accept to remove the NG interface, it shall respond with an NG REMOVAL FAILURE message with an appropriate cause value.

#### 8.7.X.4	Abnormal Conditions

Void.

Next Change

#### 9.2.6.y1	NG REMOVAL REQUEST

This message is sent by a NG-RAN node to an AMF to initiate the removal of the NG interface.

Direction: NG-RAN node ® AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |

#### 9.2.6.y2	NG REMOVAL RESPONSE

This message is sent by the AMF to a NG-RAN node to acknowledge the initiation of removal of the NG interface.

Direction: AMF ® NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.6.y3	NG REMOVAL FAILURE

This message is sent by the AMF to indicate that removing the NG interface cannot be accepted.

Direction: AMF ® NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

Next Change

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

NGAP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM NGAP-CommonDataTypes

AMFConfigurationUpdate,

AMFConfigurationUpdateAcknowledge,

AMFConfigurationUpdateFailure,

AMFCPRelocationIndication,

AMFStatusIndication,

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

WriteReplaceWarningRequest,

WriteReplaceWarningResponse,

UplinkRIMInformationTransfer,

DownlinkRIMInformationTransfer,

NGRemovalRequest,

NGRemovalResponse,

NGRemovalFailure

FROM NGAP-PDU-Contents

id-AMFConfigurationUpdate,

id-AMFCPRelocationIndication,

id-AMFStatusIndication,

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

id-UplinkRIMInformationTransfer,

id-UplinkUEAssociatedNRPPaTransport,

id-WriteReplaceWarning,

id-NGRemoval

FROM NGAP-Constants;

-- **************************************************************

--

#### -- Interface Elementary Procedure Class

--

-- **************************************************************

NGAP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome							OPTIONAL,

&UnsuccessfulOutcome						OPTIONAL,

&procedureCode				ProcedureCode	UNIQUE,

&criticality				Criticality	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE			&InitiatingMessage

[SUCCESSFUL OUTCOME			&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE				&procedureCode

[CRITICALITY				&criticality]

}

-- **************************************************************

--

#### -- Interface PDU Definition

--

-- **************************************************************

NGAP-PDU ::= CHOICE {

initiatingMessage			InitiatingMessage,

successfulOutcome			SuccessfulOutcome,

unsuccessfulOutcome			UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	NGAP-ELEMENTARY-PROCEDURE.&procedureCode		({NGAP-ELEMENTARY-PROCEDURES}),

criticality		NGAP-ELEMENTARY-PROCEDURE.&criticality			({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			NGAP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	NGAP-ELEMENTARY-PROCEDURE.&procedureCode		({NGAP-ELEMENTARY-PROCEDURES}),

criticality		NGAP-ELEMENTARY-PROCEDURE.&criticality			({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			NGAP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	NGAP-ELEMENTARY-PROCEDURE.&procedureCode		({NGAP-ELEMENTARY-PROCEDURES}),

criticality		NGAP-ELEMENTARY-PROCEDURE.&criticality			({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			NGAP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

NGAP-ELEMENTARY-PROCEDURES NGAP-ELEMENTARY-PROCEDURE ::= {

NGAP-ELEMENTARY-PROCEDURES-CLASS-1			|

NGAP-ELEMENTARY-PROCEDURES-CLASS-2,

...

}

NGAP-ELEMENTARY-PROCEDURES-CLASS-1 NGAP-ELEMENTARY-PROCEDURE ::= {

aMFConfigurationUpdate			|

broadcastSessionModification	|

broadcastSessionRelease			|

broadcastSessionSetup			|

broadcastSessionTransport					|

distributionSetup				|

distributionRelease				|

handoverCancel					|

handoverPreparation				|

handoverResourceAllocation		|

initialContextSetup				|

mTCommunicationHandling			|

multicastSessionActivation					|

multicastSessionDeactivation				|

multicastSessionUpdate			|

nGReset							|

nGSetup							|

pathSwitchRequest				|

pDUSessionResourceModify		|

pDUSessionResourceModifyIndication			|

pDUSessionResourceRelease		|

pDUSessionResourceSetup			|

pWSCancel						|

rANConfigurationUpdate			|

timingSynchronisationStatus		|

uEContextModification			|

uEContextRelease				|

uEContextResume								|

uEContextSuspend							|

uERadioCapabilityCheck			|

uERadioCapabilityIDMapping		|

writeReplaceWarning							|

nGRemoval							,

...

}

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

downlinkRIMInformationTransfer NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DownlinkRIMInformationTransfer

PROCEDURE CODE			id-DownlinkRIMInformationTransfer

CRITICALITY				ignore

}

nGRemoval NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NGRemovalRequest

SUCCESSFUL OUTCOME		NGRemovalResponse

UNSUCCESSFUL OUTCOME	NGRemovalFailure

PROCEDURE CODE			id-NGRemoval

CRITICALITY				reject

}

END

-- ASN1STOP

Next Change

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

##### -- NG Setup Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- NG SETUP REQUEST

--

-- **************************************************************

NGSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGSetupRequestIEs} },

...

}

NGSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-GlobalRANNodeID				CRITICALITY reject	TYPE GlobalRANNodeID				PRESENCE mandatory	}|

{ ID id-RANNodeName					CRITICALITY ignore	TYPE RANNodeName					PRESENCE optional	}|

{ ID id-SupportedTAList				CRITICALITY reject	TYPE SupportedTAList				PRESENCE mandatory	}|

{ ID id-DefaultPagingDRX			CRITICALITY ignore	TYPE PagingDRX						PRESENCE mandatory	}|

{ ID id-UERetentionInformation		CRITICALITY ignore	TYPE UERetentionInformation			PRESENCE optional	}|

{ ID id-NB-IoT-DefaultPagingDRX		CRITICALITY ignore	TYPE NB-IoT-DefaultPagingDRX		PRESENCE optional	}|

{ ID id-Extended-RANNodeName		CRITICALITY ignore	TYPE Extended-RANNodeName			PRESENCE optional	},

...

}

-- **************************************************************

--

###### -- NG SETUP RESPONSE

--

-- **************************************************************

NGSetupResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGSetupResponseIEs} },

...

}

NGSetupResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMFName						CRITICALITY reject	TYPE AMFName					PRESENCE mandatory	}|

{ ID id-ServedGUAMIList				CRITICALITY reject	TYPE ServedGUAMIList			PRESENCE mandatory	}|

{ ID id-RelativeAMFCapacity			CRITICALITY ignore	TYPE RelativeAMFCapacity		PRESENCE mandatory	}|

{ ID id-PLMNSupportList				CRITICALITY reject	TYPE PLMNSupportList			PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		}|

{ ID id-UERetentionInformation		CRITICALITY ignore	TYPE UERetentionInformation		PRESENCE optional		}|

{ ID id-IAB-Supported				CRITICALITY ignore	TYPE IAB-Supported				PRESENCE optional		}|

{ ID id-Extended-AMFName			CRITICALITY ignore	TYPE Extended-AMFName			PRESENCE optional		}|

{ ID id-MobileIAB-Supported			CRITICALITY ignore	TYPE MobileIAB-Supported    	PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- NG SETUP FAILURE

--

-- **************************************************************

NGSetupFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGSetupFailureIEs} },

...

}

NGSetupFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional		}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- NG Removal Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- NG REMOVAL REQUEST

--

-- **************************************************************

NGRemovalSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGRemovalRequestIEs} },

...

}

NGRemovalRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-GlobalRANNodeID				CRITICALITY reject	TYPE GlobalRANNodeID				PRESENCE mandatory	},

...

}

-- **************************************************************

--

###### -- NG REMOVAL RESPONSE

--

-- **************************************************************

NGRemovalResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGRemovalResponseIEs} },

...

}

NGRemovaleIEs NGAP-PROTOCOL-IES ::= {

{ ID id-GlobalRANNodeID				CRITICALITY reject	TYPE GlobalRANNodeID				PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- NG SETUP FAILURE

--

-- **************************************************************

NGSetupFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGSetupFailureIEs} },

...

}

NGSetupFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

Next Change

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NGAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-AMFConfigurationUpdate					ProcedureCode ::= 0

id-AMFStatusIndication						ProcedureCode ::= 1

id-CellTrafficTrace							ProcedureCode ::= 2

id-DeactivateTrace							ProcedureCode ::= 3

id-DownlinkNASTransport						ProcedureCode ::= 4

id-DownlinkNonUEAssociatedNRPPaTransport	ProcedureCode ::= 5

id-DownlinkRANConfigurationTransfer			ProcedureCode ::= 6

id-DownlinkRANStatusTransfer				ProcedureCode ::= 7

id-DownlinkUEAssociatedNRPPaTransport		ProcedureCode ::= 8

id-ErrorIndication							ProcedureCode ::= 9

id-HandoverCancel							ProcedureCode ::= 10

id-HandoverNotification						ProcedureCode ::= 11

id-HandoverPreparation						ProcedureCode ::= 12

id-HandoverResourceAllocation				ProcedureCode ::= 13

id-InitialContextSetup						ProcedureCode ::= 14

id-InitialUEMessage							ProcedureCode ::= 15

id-LocationReportingControl					ProcedureCode ::= 16

id-LocationReportingFailureIndication		ProcedureCode ::= 17

id-LocationReport							ProcedureCode ::= 18

id-NASNonDeliveryIndication					ProcedureCode ::= 19

id-NGReset									ProcedureCode ::= 20

id-NGSetup									ProcedureCode ::= 21

id-OverloadStart							ProcedureCode ::= 22

id-OverloadStop								ProcedureCode ::= 23

id-Paging									ProcedureCode ::= 24

id-PathSwitchRequest						ProcedureCode ::= 25

id-PDUSessionResourceModify					ProcedureCode ::= 26

id-PDUSessionResourceModifyIndication		ProcedureCode ::= 27

id-PDUSessionResourceRelease				ProcedureCode ::= 28

id-PDUSessionResourceSetup					ProcedureCode ::= 29

id-PDUSessionResourceNotify					ProcedureCode ::= 30

id-PrivateMessage							ProcedureCode ::= 31

id-PWSCancel								ProcedureCode ::= 32

id-PWSFailureIndication						ProcedureCode ::= 33

id-PWSRestartIndication						ProcedureCode ::= 34

id-RANConfigurationUpdate					ProcedureCode ::= 35

id-RerouteNASRequest						ProcedureCode ::= 36

id-RRCInactiveTransitionReport				ProcedureCode ::= 37

id-TraceFailureIndication					ProcedureCode ::= 38

id-TraceStart								ProcedureCode ::= 39

id-UEContextModification					ProcedureCode ::= 40

id-UEContextRelease							ProcedureCode ::= 41

id-UEContextReleaseRequest					ProcedureCode ::= 42

id-UERadioCapabilityCheck					ProcedureCode ::= 43

id-UERadioCapabilityInfoIndication			ProcedureCode ::= 44

id-UETNLABindingRelease						ProcedureCode ::= 45

id-UplinkNASTransport						ProcedureCode ::= 46

id-UplinkNonUEAssociatedNRPPaTransport		ProcedureCode ::= 47

id-UplinkRANConfigurationTransfer			ProcedureCode ::= 48

id-UplinkRANStatusTransfer					ProcedureCode ::= 49

id-UplinkUEAssociatedNRPPaTransport			ProcedureCode ::= 50

id-WriteReplaceWarning						ProcedureCode ::= 51

id-SecondaryRATDataUsageReport				ProcedureCode ::= 52

id-UplinkRIMInformationTransfer				ProcedureCode ::= 53

id-DownlinkRIMInformationTransfer			ProcedureCode ::= 54

id-RetrieveUEInformation					ProcedureCode ::= 55

id-UEInformationTransfer					ProcedureCode ::= 56

id-RANCPRelocationIndication				ProcedureCode ::= 57

id-UEContextResume							ProcedureCode ::= 58

id-UEContextSuspend							ProcedureCode ::= 59

id-UERadioCapabilityIDMapping				ProcedureCode ::= 60

id-HandoverSuccess							ProcedureCode ::= 61

id-UplinkRANEarlyStatusTransfer				ProcedureCode ::= 62

id-DownlinkRANEarlyStatusTransfer			ProcedureCode ::= 63

id-AMFCPRelocationIndication				ProcedureCode ::= 64

id-ConnectionEstablishmentIndication		ProcedureCode ::= 65

id-BroadcastSessionModification				ProcedureCode ::= 66

id-BroadcastSessionRelease					ProcedureCode ::= 67

id-BroadcastSessionSetup					ProcedureCode ::= 68

id-DistributionSetup 						ProcedureCode ::= 69

id-DistributionRelease 						ProcedureCode ::= 70

id-MulticastSessionActivation 				ProcedureCode ::= 71

id-MulticastSessionDeactivation 			ProcedureCode ::= 72

id-MulticastSessionUpdate 					ProcedureCode ::= 73

id-MulticastGroupPaging						ProcedureCode ::= 74

id-BroadcastSessionReleaseRequired			ProcedureCode ::= 75

id-TimingSynchronisationStatus				ProcedureCode ::= 76

id-TimingSynchronisationStatusReport		ProcedureCode ::= 77

id-MTCommunicationHandling					ProcedureCode ::= 78

id-RANPagingRequest							ProcedureCode ::= 79

id-BroadcastSessionTransport				ProcedureCode ::= 80

id-NGRemoval								ProcedureCode ::= x

End of Change