3GPP TSG-RAN WG3 Meeting #124	R3-243391

Fukuoka, Japan, 20th – 24th May, 2024

Agenda Item:	12.2

Source: 	Lenovo

Title: 	Discussion on resource multiplexing for WAB node

Document for:	Discussion and decision

- Introduction

A new SID for additional topological enhancements [1] has been approved to be discussed in Rel-19, and following detail objectives are identified for Wireless Access Backhaul (WAB) fucus on RAN3 and RAN2.

The objectives of the Wireless Access Backhaul (WAB) study are as follows: - Study the support of WAB including [RAN3, RAN2]: - Study the architecture and protocol stack of supporting a gNB with MT function providing PDU session backhaul. - Study impact of WAB mobility within an existing RAN (e.g., inter-gNB neighbour relations). - Identify necessary inter-gNB- and gNB-to-CN signalling to address the support of WAB. - Study signalling enhancements on resource multiplexing for WAB. NOTE 1: No impact on the UE. NOTE 2: Coordination with other WGs (e.g. SA2) when needed. The WAB study does not preclude any backhaul scenario (e.g. NTN or TN).

In this contribution, scheduling and resource allocation/coordination for WAB-node migration is discussed, including resource type and resource coordination between access link and backhaul link or between two parent WAB-donor.

- Discussion

For IAB, following resource types had been defined to realize resource coordination and collision avoidance between the access and backhaul link for an IAB-node [2].

From an IAB-node MT point-of-view, as in Rel. 15, the following time-domain resources can be indicated for the parent link: - Downlink time resource; - Uplink time resource; - Flexible time resource. From an IAB-node DU point-of-view, the child link has the following types of time resources: - Downlink time resource; - Uplink time resource; - Flexible time resource; - Not available time resources (resources not to be used for communication on the DU child links). Each of the downlink, uplink and flexible time-resource types of the DU child link can belong to one of two categories: - Hard: The corresponding time resource is always available for the DU child link; - Soft: The availability of the corresponding time resource for the DU child link is explicitly and/or implicitly controlled by the parent node.

WAB has similarities with IAB. The resource configuration for IAB is therefore a candidate for WAB. However, due to differences in the architectures and protocol stacks of WAB and IAB, it may not be ideal to fully reuse the H/S/NA framework specified for IAB.

One main difference between IAB and WAB is that the WAB-node includes a full gNB (WAB-gNB) rather than the IAB-DU in the case of IAB. As a result, the interface between the WAB-gNB and the gNB serving the WAB-MT is Xn instead of F1. The F1 interface is a configuring interface, and the F1AP provides various IEs for configuring the IAB-DU by the IAB-donor-CU. However, the Xn interface has a different function, essentially connecting two peer gNBs and exchange information between the two peer gNBs. It is hence not natural to leave all the resource configurations of the WAB-gNB to be configured by the gNB serving the WAB-MT via the XnAP procedure.

Another issue to consider is that according to the current IAB specifications, H and NA resources have a forcing behavior at the IAB node, but the behavior of the IAB node for S resources is not fully determined by the IAB-donor or the parent node. Instead, the RAN1 specification (TS 38.213, section 14) provides a list of conditions for using S resources by the IAB-DU that includes, but is not constrained by, signaling from the parent node.

Therefore, we propose to take the H/S/NA framework as a starting point with modifications that avoid a full tedious configuration of one gNB (WAB-gNB) by another gNB (gNB serving the WAB-MT). A natural approach is to configure the WAB-MT resources by RRC and use L1/L2 signaling for indicating usage/availability of the resources for the WAB-MT. Then, the WAB-gNB can decide about using its own resources based on the configuration and L1/L2 indication as well as its multiplexing capability.

Proposal 1: Resource configuration based on the H/S/NA framework specified for IAB can be used as a starting point for WAB resource configurations. The resource configuration can be handled by RRC more appropriately than by XnAP.

As defined in the SID [1], the WAB network in R19 only supports one-hop backhauling, where the WAB-node is directly connected to the gNB serving the WAB-MT. In this WAB topology, the downlink transmission from WAB-gNB to the served UEs is scheduled by the WAB-gNB itself, and the uplink transmission from WAB-MT to the gNB serving the WAB-MT is scheduled by the gNB serving the WAB-MT.

In a centralized resource allocation to avoid inter-cell or inter-node resource interference, the gNB serving the WAB-MT should provide some semi-static reserved DL/UL resources for each served WAB-nodes, and how to schedule on these semi-static resources is left to the implementation of WAB-node’s scheduling algorithm. However, there is no F1 interface between gNB serving the WAB-MT and WAB-node, the legacy F1AP message used for resource configuration on the IAB-node cannot be reused. Then the resource configuration may need to be delivered via the XnAP sigalling between the gNB serving the WAB-MT and the WAB-gNB.

According to the IAB specifications, interpreting the resources available to the IAB-DU depends on the D/U/F configuration at the IAB-DU because the L1 availability indication indicates availability of soft DL symbols, soft UL symbols, and soft flexible symbols in a slot instead of indicating availability of each symbol separately. That, in part, reduces the overhead of the L1 availability indication signaling. A similar principle can be adopted for the L1/L2 resource usage indication for WAB. As a result, the gNB serving the WAB-MT may require knowledge of the D/U/F configuration at the WAB-gNB. For this purpose, the WAB-gNB can use Xn signaling to inform the gNB serving the WAB-MT of its own D/U/F resource configurations.

Proposal 2: The WAB-gNB can signal its D/U/F configuration to the gNB serving the WAB-MT over Xn.

As for the resource configuration on DU of the gNB serving the WAB-MT, legacy F1AP GNB-DU RESOURCE CONFIGURATION can be reused, which includes the resource configuration (D/U/F/NA/H/S) on the gNB-DU serving the WAB-MT, and it further includes the resource configuration and the multiplexing info of the WAB-gNB, which is helpful for the gNB-DU serving the WAB-MT to avoid scheduling the WAB-MT in the slot/symbol with hard resources in the collocated WAB-gNB.

Proposal 3: Legacy F1AP GNB-DU RESOURCE CONFIGURATION can be reused for resource configuration on WAB-donor-DU.

# Conclusion

This contribution aims to discuss enhancements for mobility of an IAB-node together with its served UEs. And following observations and proposals are concluded.

Proposal 1: Resource configuration based on the H/S/NA framework specified for IAB can be used as a starting point for WAB resource configurations. The resource configuration can be handled by RRC more appropriately than by XnAP.

Proposal 2: The WAB-gNB can signal its D/U/F configuration to the gNB serving the WAB-MT over Xn.

Proposal 3: Legacy F1AP GNB-DU RESOURCE CONFIGURATION can be reused for resource configuration on WAB-donor-DU.

# Reference

- RP-234041. New SID: Study on additional topological enhancements for NR.

- 3GPP TR 38.874. Study on Integrated Access and Backhaul (Release 16). V16.0.0.