3GPP TSG-RAN WG3 #124	R3-243506

Fukuoka, Japan, 20 – 24 May 2024

Agenda Item:	8.3

Source:	Ericsson, Deutsche Telekom, AT&T

Title:	Discussions on User Consent corrections

Document for:	Discussion and Decision

# 1	Introduction

During the RAN3#123 meeting in Athens, progress has been made on the user consent related discussions leaving only the question on the UE selection rules for MDT configurations that contain both MDT measurements subject to user consent and not subject to user consent. In order to resolve the issue an LS was sent to SA5 [1].

This contribution discusses the SA5’s response [2] and resolves the issue by incorporating the SA5 behavior into RAN3’s TS38.401 and TS37.320.

# 2	Discussion

## 2.1	Analysis of the reply LS from SA5

In their reply LS, SA5 points out that amendments to TS32.422 have been made in support to the request from RAN3 to adopt a new enhanced solution for UE selection rules related to MDT user consent. In the next section we will take these new descriptions into account to modify the CRs technically endorsed in RAN3.

The following text is extracted from the reply:

“In the <<original RAN3>> LS, RAN3 would like to check with SA5 the following approach and provide feedback:

- For MDT configurations that contain both MDT measurements subject to user consent and not subject to user consent, the RAN node may select UEs considering user consent and configure MDT measurements that are both subject to use consent and not subject to user consent or the RAN node may select UEs without considering user consent and configure MDT measurements not subject to user consent.

SA5 has discussed it and would like to provide the following considerations,

- If the MDT configuration contains both MDT measurements subject to user consent and not subject to user consent,

  - If the subscriber provided consent, the gNB shall select the corresponding UE and configure with all the MDT measurements in the MDT configuration.

  - If the subscriber did not provide consent, the gNB may select the UE and configure it with the MDT measurements not subject to user consent that are contained in the MDT configuration.

For more details, please see the agreed CR.”

The attached agreed 32.422CR [3] clarifies the user consent procedure. It specifies that the MDT activation shall be based on the OAM configured user-consent-request and clarifies the behavior when an MDT configuration contains both MDT measurements subject to user consent and not subject to user consent.

In particular, for the management-based MDT activation, if the requested MDT measurements are not subject to user consent, the RAN shall select the UE and start a trace session without considering user consent. If the requested measurements constitute a mixture of measurements subject to user consent and measurements not subject to user consent, the RAN shall:

- Configure and collect measurements subject to user consent, if the user gave consent

- Configure and collect only measurements not subject to user consent, if the user did not give consent

Relevant extract from 32.422CR [3]:

“If one or more MDT measurement name(s) is/are specified in MDT activation, the UDM shall:

- For user-consent-required MDT measurement name(s), start a Trace Session for the given subscriber if the user consent is available.

- For user-consent-required MDT measurement name(s), not start a Trace Session for the given subscriber if the user consent is not available.

- For user-consent-not-required MDT measurement name(s), start a Trace Session for the given subscriber without considering user consent.”

The same requirement is specified for the signalling-based MDT procedure.

In light of the above, the RAN stage 2 specifications need to be aligned to the SA5 and we therefore propose to update the current TS38.401 and TS37.320 specs to reflect the above specified behavior.

Proposal: it is proposed to update the Stage 2 descriptions on user consent for MDT in TS38.401 and TS37.320 in accordance to the changes applied by SA5 in their specifications and communicated in R3-243030

# 3	Conclusion

In this contribution User consent related corrections have been discussed.

The following proposals were derived:

Proposal: it is proposed to update the Stage 2 descriptions on user consent for MDT in TS38.401 and TS37.320 in accordance to the changes applied by SA5 in their specifications and communicated in R3-243030

CRs mirroring the proposal above are available in [4]and [5].

# References

- R3-241115, “Reply LS on user consent for trace reporting”, RAN3

- R3-243030/S5-242221, “Reply LS on the user consent for trace reporting”, SA5

- S5-242065, R18 CR 32.422 user consent alignment, Ericsson, AT&T, Verizon, Huawei, China Unicom, China Telecom, DTAG, CMCC

- R3-243507 Clarifications on User consent corrections for TS38.401

- R3-243508 Clarifications on User consent corrections for TS37.320