3GPP TSG-RAN WG3 Meeting #124	R3-243824

# Fukuoka, Japan, 20th - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1319 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on textual description of Early Status Transfer procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, CMCC, China Unicom, CATT, Samsung, Nokia, Ericsson, China Telecom, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current textual description of Early Status Transfer procedure is ambiguous. “For each DRB in the DRBs Subject To Early Status Transfer List IE for which the DISCARD DL COUNT Value IE is received in the EARLY STATUS TRANSFER message, the target NG-RAN node does not transmit forwarded downlink SDUs to the UE whose COUNT is less than the provided and discards them if transmission has not been attempted.” “For each DRB in the DRBs Subject To Early Status Transfer List IE, the source M-NG-RAN node shall forward to the target, the value of the received FIRST DL COUNT Value IE or DISCARD DL COUNT Value IE.” “For each DRB in the DRBs Subject To Early Status Transfer List IE, the M-NG-RAN node shall forward to the S-NG-RAN node, the value of the received FIRST DL COUNT Value IE or DISCARD DL COUNT Value IE.” “For each DRB in the DRBs Subject To Early Status Transfer List IE, the source S-NG-RAN node shall forward to the M-NG-RAN node and the M-NG-RAN node shall forward to the target S-NG-RAN node, during Conditional PSCell Change, the value of the received FIRST DL COUNT Value IE or DISCARD DL COUNT Value IE.” According to the IE table in 9.1.1.14, the DISCARD DL COUNT Value IE is included in the DRBs Subject To DL Discarding List rather than the DRBs Subject To Early Status Transfer List. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has no impact since it only updates the textual description of Early Status Transfer procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the “DRBs Subject To Early Status Transfer List IE” to “DRBs Subject To DL Discarding List IE”. - Add the new sentences to describe the handling of the received DISCARD DL COUNT Value IE. - Correct the redundant typo in the subtitle. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current textual description of early data forwarding and DL discarding is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: R3-243760 Rev1: Change the WID to “TEI18”; Change the Category as “F”; Add Ericsson, China Telecom and LG Electronics as the co-signers. |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

8.2.10	Early Status Transfer

8.2.10.1	General

The purpose of the Early Status Transfer procedure is to transfer the COUNT of the first downlink SDU that the source NG-RAN node forwards to the target NG-RAN node or the COUNT for discarding of already forwarded downlink SDUs for respective DRB during DAPS Handover or Conditional Handover.

For MR-DC with 5GC, the Early Status Transfer procedure is also used from the source S-NG-RAN node to the source M-NG-RAN node during a Conditional Handover as specified in TS 37.340 [8].

For Conditional PSCell Addition in MR-DC with NR SCG, the Early Status Transfer procedure is also used, from the M-NG-RAN node to the S-NG-RAN node as specified in TS 37.340 [8].

For Conditional PSCell Change in MR-DC with NR SCG, the Early Status Transfer procedure is also used from the source S-NG-RAN node to the M-NG-RAN node, and from the M-NG-RAN node to the target S-NG-RAN node as specified in TS 37.340 [8].

The procedure uses UE-associated signalling.

8.2.10.2	Successful Operation

Figure 8.2.10.2-1: Early Status Transfer during DAPS Handover or Conditional Handover, successful operation

Figure 8.2.10.2-2: Early Status Transfer during Conditional Handover in MR-DC operation, successful operation

Figure 8.2.10.2-3: Early Status Transfer during CPAC, successful operation

From source NG-RAN node to target NG-RAN node

The DRBs Subject To Early Status Transfer List IE included in the EARLY STATUS TRANSFER message contains the DRB ID(s) corresponding to the DRB(s) subject to be simultaneously served by the source and the target NG-RAN nodes during DAPS Handover or the DRB(s) transferred during Conditional Handover.

For each DRB in the DRBs Subject To Early Status Transfer List IE, the target NG-RAN node shall use the value of the FIRST DL COUNT Value IE as the COUNT of the first downlink SDU that the source NG-RAN node forwards to the target NG-RAN node.

For each DRB in the DRBs Subject To DL Discarding List IE for which the DISCARD DL COUNT Value IE is received in the EARLY STATUS TRANSFER message, the target NG-RAN node does not transmit forwarded downlink SDUs to the UE whose COUNT is less than the provided and discards them if transmission has not been attempted.

From source S-NG-RAN node to source M-NG-RAN node, for Conditional Handover

The DRBs Subject To Early Status Transfer List IE included in the EARLY STATUS TRANSFER message contains the DRB ID(s) corresponding to the DRB(s) transferred during Conditional Handover.

For each DRB in the DRBs Subject To Early Status Transfer List IE, the source M-NG-RAN node shall forward to the target, the value of the received FIRST DL COUNT Value IE; for each DRB in the DRBs Subject To DL Discarding List IE, the source M-NG-RAN node shall forward to the target, the value of the received DISCARD DL COUNT Value IE.

From M-NG-RAN node to S-NG-RAN node, for Conditional PSCell Addition

The DRBs Subject To Early Status Transfer List IE included in the EARLY STATUS TRANSFER message contains the DRB ID(s) corresponding to the DRB(s) transferred during Conditional PSCell Addition.

For each DRB in the DRBs Subject To Early Status Transfer List IE, the M-NG-RAN node shall forward to the S-NG-RAN node, the value of the received FIRST DL COUNT Value IE; for each DRB in the DRBs Subject To DL Discarding List IE, the M-NG-RAN node shall forward to the S-NG-RAN node, the value of the received DISCARD DL COUNT Value IE.

From source S-NG-RAN node to M-NG-RAN node, and from M-NG-RAN node to target S-NG-RAN node, for Conditional PSCell Change

The DRBs Subject To Early Status Transfer List IE included in the EARLY STATUS TRANSFER message contains the DRB ID(s) corresponding to the DRB(s) transferred during Conditional PSCell Change.

For each DRB in the DRBs Subject To Early Status Transfer List IE, the source S-NG-RAN node shall forward to the M-NG-RAN node and the M-NG-RAN node shall forward to the target S-NG-RAN node, during Conditional PSCell Change, the value of the received FIRST DL COUNT Value IE; for each DRB in the DRBs Subject To DL Discarding List IE, the source S-NG-RAN node shall forward to the M-NG-RAN node and the M-NG-RAN node shall forward to the target S-NG-RAN node, during Conditional PSCell Change, the value of the received DISCARD DL COUNT Value IE.

8.2.10.3	Unsuccessful Operation

Not applicable.

8.2.10.4	Abnormal Conditions

If the target NG-RAN node receives this message for a UE for which no prepared DAPS Handover or Conditional Handover exists at the target NG-RAN node, the target NG-RAN node shall ignore the message.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>