3GPP TSG-RAN3 Meeting # 124 	R3-243907

# Fukuoka, Japan,  20th - 24th May. 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0403 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on general F1AP principles for multicast reception in RRC_INACTIVE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Ericsson, CMCC, Huawei, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_enh-Core |  |  |  |  |  | Date: |  |  | 2024-05-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN3 agreed on the following in RAN3#121-bis meeting: “Both of the following two options are adopted to support PTM configuration provision during RRC Release procedure in split gNB scenario: Option 1: CU retrieves the PTM configuration from DU via CU initiated Multicast context setup/modification procedure. Option 2: During active MBS multicast sessions, the DU always ensures that the CU is provided with the latest PTM configuration via a new DU initiated Multicast context modification procedure.” Stage-3 support for the above agreements has been implemented, however, stage-2 specification is still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Basic principles on F1AP interactions for multicast reception in RRC_INACTIVE are captured in section 7.7.3. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release) because the changes affect only functions concerning MBS multicast reception in RRC_INACTIVE only. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 38.401 would lack specification of principles of F1AP interactions for multicast reception in RRC_INACTIVE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: update according to offline discussion, add co-source company. Rev 2: update impact analysis, add co-source company. |  |  |  |  |  |  |  |  |

// change starts.

### 7.7.3	Support of Multicast reception for UEs in RRC_INACTIVE

F1AP supports:

- enabling and disabling multicast reception for UEs in RRC_INACTIVE state for a specific multicast MBS session on cell level.

- retrieval of PTM configuration information from the the gNB-DU by means of the gNB-CU triggered Multicast Context Modification procedure.

- keeping the gNB-CU updated during active multicast MBS sessions with the latest PTM configuration by means of the gNB-DU initiated Multicast Context Notification procedure.

The gNB-CU decides whether multicast reception in RRC_INACTIVE state is applied.

// change ends.