3GPP TSG-RAN WG3 Meeting #125bis	R3-245864

# Hefei, China, 14-18 October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BL CR to 38.300) Introduction of NR Femto Architecture and Protocol Aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, TMO US, AT&T, Verizon Wireless, BT, Charter, Huawei, LG Electronics, Samsung, Lenovo, Baicells |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_WAB_5GFemto-Core |  |  |  |  |  | Date: |  |  | 2024-10-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce NR Femto and NR Femto GW in NG-RAN architecture description |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce logical architecture and protocol stacks for NR Femto and NR Femto GW |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No description of NR Femto and NR Femto GW in NG-RAN stage 2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.413 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R3-245864: endorsed as BL at RAN3 #125bis |  |  |  |  |  |  |  |  |

START OF CHANGES

# 3	Abbreviations and Definitions

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

UNCHANGED PART OMITTED

SCS	SubCarrier Spacing

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SDT	Small Data Transmission

SD-RSRP	Sidelink Discovery RSRP

SeGW	Security Gateway

SFI-RNTI	Slot Format Indication RNTI

SHR	Successful Handover Report

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

UNCHANGED PART OMITTED

NEXT CHANGE

# 4	Overall Architecture and Functional Split

UNCHANGED PART OMITTED

## 4.X	Support of NR Femtos

### 4.X.1	Architecture

Figure 4.X.1-1 shows a logical architecture for the NR Femto that has a set of NG interfaces to connect the NR Femto to the 5GC.

Figure 4.X.1-1: NR Femto Logical Architecture

NOTE: the SeGW is out of RAN scope.

The NR Femto may directly connect to the 5GC. The NG-RAN architecture may also deploy an NR Femto Gateway (NR Femto GW) to allow the concentration of the NG-C interface between the NR Femto and the 5GC. Based on implementation, the transport of NG-U between the NR Femto and the 5GC may be optionally concentrated in the NR Femto GW.

For NR Femto, the NG-C interface is defined as the interface:

-	Between the NR Femto GW and the Core Network;

-	Between the NR Femto and the NR Femto GW;

-	Between the NR Femto and the Core Network;

The NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto as an AMF. The NG interface between the NR Femto and the 5GC is the same regardless whether the NR Femto is connected to the 5GC via an NR Femto GW or not.

The functions supported by the NR Femto shall be the same as those supported by a gNB (with possible exceptions e.g. NNSF when the NR Femto connects via the NR Femto GW) and the procedures run between an NR Femto and the 5GC shall be the same as those between a gNB and the 5GC.

Xn-connectivity is supported between NR Femtos and between NR Femtos and gNBs, independent of whether any of the involved NR Femtos is connected to an NR Femto GW.

NEXT CHANGE

### 4.X.3	Interfaces

#### 4.X.3.1	Protocol Stack for NG User Plane

Editor’s note: Description FFS.

#### 4.X.3.2	Protocol Stacks for NG Control Plane

The two figures below show the NG-C protocol stack with and without the NR Femto GW.

When the NR Femto GW is not present (Fig. 4.x.3.2-1), all the NGAP procedures are terminated at the NR Femto and the AMF.

Any protocol function associated to a UE-associated procedure shall reside within the NR Femto and the AMF only.

Editor’s note: The sentence above will be captured in detail in the functional split section.

Figure 4.X.3.2-1: Control plane for NG Interface for NR Femto to AMF without the NR Femto GW

Editor’s note: It is FFS whether the figure above is needed.

Figure 4.X.3.2-2: Control plane for NG Interface for NR Femto to AMF with the NR Femto GW

END OF CHANGES