3GPP TSG-RAN WG3 Meeting #125-bis	R3-245849

Hefei , CN, 14 - 18 Oct, 2024

Title:	(BL pCR to TR 38.769) Study on solutions for Ambient IoT in NR

Agenda Item:	16.1

Source:	Huawei, CMCC

Document for:	pCR

# 1	Introduction

This contribution provides the baseline pCR to the TR 38.769 from RAN3 perspective, and captures the TPs [2] [3] and [4] agreed in RAN3#125bis meeting.

In RAN#105, TR 38.769 was updated to v1.0.0 [1], in this paper, some remaining “AIoT” in RAN3 introduced Figures are updated to “A-IoT” to align with the other parts of the TR, and also fixed few typos.

# 2	Reference

[1] 3GPP TR 38.769 Study on solutions for ambient IoT (Internet of Things) (Release 19) V1.0.0 (2024-09)

[2] R3-245846	(TP to TR 38.769) A-IoT RAN Architecture aspects

[3] R3-245847	(TP to TR 38.769) Protocol Stacks of Topology 2 solutions

[4] R3-245848	[TP for TR 38.769] CB:#AIoT2_RANCNinterface

# 3	Text Proposal to TR 38.769

------------Start of the First Change-------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 38.848: "Study on Ambient IoT (Internet of Things) in RAN".

[3]	RP-240826: " Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR".

[4]	3GPP TR 38.869: "Study on low-power Wake-up Signal and Receiver for NR".

[5]	3GPP TS 38.212: "NR; Multiplexing and channel coding".

[6]	EPC Radio-Frequency Identity Protocols Class-1 Generation-2 UHF RFID Protocol for Communications at 860 MHz – 960 MHz.

[7]	3GPP TR 23.700-13: "Study on Architecture support of Ambient power-enabled Internet of Things".

[8]	3GPP TR 33.713: "Study on security aspects of Ambient Internet of Things (AIoT) services in 5G".

[9]	3GPP TS 38.300: "NR and NG-RAN Overall description; Stage-2".

[10]	3GPP TR 22.369: "Service requirements for Ambient power-enabled IoT".

[11]	R1-2405855, "On external carrier wave for backscattering based Ambient IoT device", Huawei, HiSilicon, RAN1#118. Maastricht, Netherlands, August 2024.

# 3	Definitions of terms, symbols and abbreviations

This clause and its three (sub) clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

For the purposes of the study, RAN1 uses the following terms:

Device 1: ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device's UL transmission is backscattered on a carrier wave provided externally.

Device 2a: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is generated internally by the device.

D1T1: Deployment scenario 1 with connectivity topology 1, according to TR 38.848.

D2T2: Deployment scenario 2 with connectivity topology 2, according to TR 38.848.

Inventory: The service provided by the network to discover and acquire the identifier of A-IoT device(s).

Command: The service provided by the network to send the operation instruction to the A-IoT device (e.g. read, write, etc.).

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

A-IoT	Ambient IoT

A-IoT RAN	Ambient IoT Radio Access Network

BFSK	Binary frequency-shift keying

BPSK	Binary phase-shift keying

CFO	Carrier-frequency offset

CP	Cyclic prefix

CW	Carrier-wave

CW2D	Carrier-wave, or carrier-wave node, to device

D2R	Device to reader

DO-A	Device-originated autonomous

DO-DTT	Device-originated by device-terminated trigger

DT	Device-terminated

ED	Envelope detector

FEC	Forward error-correction code

FR	Frequency Range

IF	Intermediate frequency

IoT	Internet of Things

LPWA	Low-power, wide-area

LTE-MTC	Long Term Evolution – Machine Type Communication

MCS	Modulation and coding scheme

MSK	Minimum-shift keying

NB-IoT	Narrowband IoT

OOK	On-off keying

PDRCH	Physical device-to-reader channel

PIE	Pulse interval encoding

PRDCH	Physical reader-to-device channel

R2D	Reader to device

RF	Radio frequency

RFID	Radio frequency identification

SFO	Sampling-frequency offset

ZIF	Zero IF

------------Start of the Next Change-------------

## 6.4	RAN architecture aspects

This clause attempts to identify and describe architectural elements necessary to define a RAN architecture for support of Ambient IoT embedded in the overall 5G system architecture in support of topology 1 and topology 2 (as defined in TR 38.848 [2]).

This chapter also attempts to identify a functional split between RAN and CN.

The logical system architecture for A-IoT consists of the following architectural elements:

A-IoT device:	Equipment with characteristics outlined e.g., in TS 22.369 [10] and TR 38.848 [2].

A-IoT RAN:	Hosts certain functions for A-IoT as part of the functional split between RAN and CN.

Editor’s Note 4: Further details regarding A-IoT functions hosted in the A-IoT RAN and the respective functional split to be decided by RAN2, RAN3 and SA2.

A-IoT radio:	Radio interface between A-IoT device and A-IoT RAN node in topology 1 and between A-IoT device and A-IoT-enabled UE in topology 2.

Editor’s Note 5: Further details on A-IoT radio to be discussed by RAN1 and RAN2.

A-IoT CN:	Hosts certain functions for A-IoT as of the functional split between RAN and CN.

NOTE: the details of A-IoT CN are subject to SA2.

Editor’s Note 6: Further details regarding A-IoT functions hosted in the A-IoT CN and the respective functional split to be decided by RAN2, RAN3 and SA2.

XX interface:	Interface between the A-IoT RAN and the A-IoT CN on which certain A-IoT specific functions are performed.

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g., for topology 1 it may only represent a single interface instance, e.g., a new interface between A-IoT RAN and A-IoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between A-IoT CN and A-IoT RAN, or one instance for NG alone.

Common reader function:	A function that communicates with the A-IoT device by means of A-IoT radio.

Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2.

A-IoT RAN node function:	A function that contains e.g., the control of the A-IoT radio resources used towards the A-IoT device.

Editor’s Note 9: further details are FFS. Note that “control of A-IoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g., Inventory, Command) e.g., in case these functions have to be performed over a multitude of instances of the Common Reader Function are FFS.

### 6.4.1	Support of Topology 1

Figure 6.4.1-1 depicts a logical system architecture for topology 1, where the Common reader function and A-IoT RAN node function are deployed within an A-IoT RAN.

Figure 6.4.1-1 Logical system architecture for topology 1

For Toplogy 1, architecture and protocol aspects of split RAN architecture are not studied.

In Topology 1, the XX interface could be based on NG or a new interface carried over NG or a new interface.

Figure 6.4.1-2 shows the Protocol stack for Topology 1, assuming a SCTP-based transport:

Figure 6.4.1-2. Protocol Stack for Topology 1

NOTE:	The protocol stack in Figure 6.4.1-2 does not illustrate how A-IoT upper layer information, if any, is transported over XXAP. Details are subject to SA2 agreements.

For topology 1, the XXAP is terminated at an A-IoT RAN node.

The signalling transport for XXAP at the A-IoT RAN node is SCTP/IP. Other options of signalling transport for XXAP at the A-IoT RAN node (e.g., HTTP/2/TLS/TCP) were discussed, but will not be pursued.

Editor’s Note 3: The A-IoT CN may include AMF and A-IoT related functions which is up to SA2 decision.

In Topology 1, an A-IoT RAN node may serve one or more readers.

The A-IoT RAN node should enable the coordination of the usage of the A-IoT radio resources among readers.

Reader selection may need coordination between A-IoT RAN node and A-IoT CN.

### 6.4.2	Support of Topology 2

Figure 6.4.2-1 depicts a logical system architecture for topology 2, where the Common reader function is located at an A-IoT-enabled UE, and the A-IoT RAN node function is located at an A-IoT-enabled gNB.

The following definitions apply:

A-IoT-enabled gNB:	A gNB supporting A-IoT RAN node function in topology 2, which is able to communicate with the A-IoT-enabled UE via NR Uu interface.

A-IoT-enabled UE:	A UE supporting Common reader function, which is able to communicate with the A-IoTdevice via the A-IoT radio interface.

Figure 6.4.2-1 Logical system architecture for topology 2

Editor’s Note 1:	Figure 6.4.2-1 doesn’t illustrate the protocol between A-IoT enabled UE and A-IoT CN, if needed, the figure needs to be revised in case such is defined by SA2.

Editor’s Note 2:	In Topology 2, the XX interface could be based on NG or a new interface carried over NG or a new interface. XX signaling could be transported via XX-C or XX-U, which is FFS.

Editor’s Note 3:	The A-IoT CN could include AMF and A-IoT related functions, which is up to SA2 decision.

Editor’s Note 4:	The A-IoT enabled gNB performs radio resource management for A-IoT related radio resources, details are pending on RAN1 and RAN2 mechanisms.

Editor’s Note 5:	In Topology 2, it is FFS on reader selection, may need coordination between A-IoT RAN node and A-IoT CN.

In Topology 2, the RAN architecture should enable the coordination of the usage of the A-IoT radio resources among readers.

An A-IoT-enabled gNB could support both topology 1 and topology 2, this is an implementation matter.

#### 6.4.2.1	Solutions for Topology 2

##### 6.4.2.1.0	General

To support Topology 2, the following solutions are to be studied for conveying A-IoT upper layer information:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

NOTE:	The protocol stack for each solution option does not illustrate A-IoT CN internal architecture and how A-IoT upper layer information is transported, if any. Details are subject to SA2 agreements.

##### 6.4.2.1.1	Solution1: RRC based solution

Upon receiving XXAP: A-IoT related message from A-IoT CN, the A-IoT-enabled gNB transmits the related information towards the A-IoT-enabled UE via NR Uu RRC, and vice versa.

Figure 6.4.2.1.1-1: RRC based solution of Topology 2

## 6.5	Impacts on CN-RAN interface

Editor’s Note: Corresponds to the first RAN3 objective in the SID, to identify necessary impacts on signaling and procedures for CN-RAN interface.

### 6.5.1	Information exchanged between A-IoT CN and A-IoT RAN

#### 6.5.1.1	Inventory

Inventory can be sent by the A-IoT CN for a single device, or a group of devices, or all devices.

The Inventory Request from the A-IoT CN to the A-IoT RAN, may include the following:

(1)	A-IoT Device Identification (to find a single device, a group of devices, or all devices)

Note 1:	The definition of this identification is out of RAN3 scope.

Editor’s Note 1: It is FFS whether A-IoT RAN needs to interpret/store/process it.

(2)	Scope of inventory request (e.g., a certain area in which the inventory is to be triggered)

Multiple individual A-IoT Device IDs (one ID per device) can be provided to the A-IoT CN via a single Inventory Report.

Editor’s Note 2: It is up to SA2 whether device ID is sent transparent or not.

#### 6.5.1.2	Command

Command can be sent by the A-IoT CN for a single device.

Editor’s Note 1: it is FFS for command on a group of devices, or all devices.

Editor’s Note 2: it is FFS whether A-IoT RAN can remain agnostic of the type of request from the A-IoT CN (need to differentiate command and inventory).

#### 6.5.1.3	A-IoT radio resource allocation in case of NAS/UP based solutions

In NAS/UP based solutions, A-IoT radio resources can be requested in advance to the NAS/UP based communication with the A-IoT device or can be requested along with the NAS/UP based communication. There are different ways to trigger A-IoT radio resource allocation, e.g., upon CN request or or upon UE request, etc. Some aspects of A-IoT radio allocation may be preconfigured by OAM.

### 6.5.2	Signaling and Procedures for Topology 1

NOTE 1: XX communication depicted in the following chapters uses protocol elements (messages and information elements) detailed in section 6.5.1 and are not repeated unless additional description is necessary.

Editor’s Note 1: Reader selection is FFS.

#### 6.5.2.1	Candidate procedures for A-IoT Inventory for Topology 1

Figure 6.5.2.1-1: Message flow for A-IoT Inventory in Topology 1

1a.	The A-IoT CN sends an Inventory request message to the A-IoT RAN node, taking into account, among others, the A-IoT transaction scope.

1b.	The A-IoT RAN node allocates and coordinates the usage of A-IoT radio resources.

2.	The A-IoT RAN node sends an Inventory response message to the A-IoT CN.

NOTE 1: In step 2, the A-IoT RAN node may instead send an Inventory failure message to the A-IoT CN indicating that the inventory procedure could not be initiated towards the A-IoT device(s).

3.	The A-IoT RAN node performs the inventory procedure towards the A-IoT device(s) over the A-IoT radio interface.

4a/4b.	After receiving the inventory result from the A-IoT device(s), the A-IoT RAN node may send one or multiple Inventory reports towards the A-IoT CN including the received inventory result.

NOTE 2: Steps 4a/4b may happen in parallel with Step 3 for different A-IoT devices.

### 6.5.3	Signaling and Procedures for Topology 2

#### Editor’s Note: Reader selection is FFS.

#### 6.5.3.1	Candidate procedures for A-IoT Inventory for Topology 2

##### 6.5.3.1.1	RRC solution

Figure 6.5.3.1-1: Message flow for A-IoT Inventory in Topology 2 - RRC-based solution

1a.	The A-IoT CN sends an Inventory request message to the A-IoT enabled gNB

1b.	The A-IoT enabled gNB allocates and coordinates usage of A-IoT radio resources.

1c/2a	RRC communication with the A-IoT enabled UE takes place.

Editor’s Note 1:	RRC based communication is only depicted schematically, details in RAN2 FFS.

2.	The A-IoT-enabled gNB sends an Inventory response message to the A-IoT CN.

NOTE 1: In step 2, the A-IoT-enabled gNB may instead send an Inventory failure message to the A-IoT CN indicating that the inventory procedure could not be initiated towards the A-IoT device(s).

3.	The A-IoT-enabled gNB requests the A-IoT-enabled UE(s) to trigger inventory procedure towards the A-IoT device(s).

4a/4b.	After receiving inventory result reported from the A-IoT enabled UEs, the A-IoT-enabled gNB may send one or multiple Inventory reports towards the A-IoT CN including the received inventory result.

NOTE 2: Steps 4a/4b may happen in parallel with Step 3 for different A-IoT devices.

Editor’s Note 2:	Step 4a/4b between A-IoT-enable UE and A-IoT-enabled gNB can be refined by RAN2.

##### 6.5.3.1.2	NAS/UP solution

Editor’s Note 1: Future discussions on A-IoT Inventory will take place based on the following message flow, working on the content of the messages including ownership, associated functions, scope, etc.

Figure 6.5.3.1-2: Message flow for A-IoT Inventory in Topology 2 (if NAS/UP based solution is used)

Editor’s Note 2: how and where to depict signalling suitable for triggering A-IoT RAN node functions for A-IoT radio resource management needs further discussions for direct communication between A-IoT CN and A-IoT-enabled UE.

------------Start of the Next Change-------------

## 6.9	Locating ambient IoT devices

### 6.9.x	General

Editor’s note: Corresponds to the third RAN3 objective in the SID, to identify potential solutions for locating an Ambient IoT device with no specification impact, e.g., reusing existing user location report, or minimal specification impact to convey location information to core network.

A-IoT device location information may be used for the following purposes:

(a) improving the A-IoT operation itself, e.g., by A-IoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the A-IoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

For topology 1, A-IoT RAN node ID can be considered as a location of A-IoT device. For topology 2, UE ID of the A-IoT-enabled UE can be considered as a location of A-IoT device.

Editor’s Note 1:	How to know the “reader” location is FFS. Whether to use more than one “readers” within one A-IoT RAN for location purposes is FFS.

Editor’s Note 2:	Signalling details on how to provide the location information of the A-IoT device to the A-IoT CN needs further study.

Editor’s Note 3:	Introducing finer granularity for locating an Ambient device besides the “reader ID granularity” needs further study.

Editor’s Note 4:	Analysing the gap between the positioning requirements in the SID and the feasibility for A-IoT devices.

### 6.9.x	Proximity determination

Editor’s note: Proximity determination may be in a 6.9.x sub-clause, or another arrangement, depending on how the study proceeds.

Proximity determination is feasible with either of the two following solutions. Potential specification impact or not will not be determined in this study item.

Solution 1:	If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader.

Solution 2:	If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader based on measurements at the reader side.

Proximity determination based on device-side measurements is not considered.

------------Start of the Next Change-------------

# 8	Conclusions and recommendations

------------End of the Changes-------------