3GPP TSG-RAN WG3#125bis 	R3-245044

Hefei, China, 14-18 October 2024

Agenda item:	8.1. New Incoming LSs

Source:	Nokia, LG Electronics, NTT Docomo

Title:	Discussion on L3 L1/2 Triggered Mobility

Document for:	Discussion and Decision

# 1	Introduction

In RAN4#111, agreement was made according to which, LTM cell switch requirement is applicable without L1 measurement and report for FR1. This paper discusses the required support for disaggregated gNB architecture and impact to F1AP signalling.

# 2	Background

In RAN4# 111, agreement was made according to which, LTM cell switch requirement is applicable without L1 measurement and report for FR1 (R4-2410302). Further, an LS was sent to RAN2 informing this decision (R2-2406227).

Further, RAN2 discussed the RAN4 response and issued an LS to RAN3 requesting support over F1 for this functionality (R2-2407606)

Excerpt from R4-2410302 (RAN4 decision)

# 4. Topic #4: LTM – RAN2 LS Discuss the wording in the reply LS. Online agreement < Agreement>: - For FR1, the cell switch delay requirement is applicable for the case without L1 measurement and report, under certain conditions. - Further discuss the how the requirement applies together with the condition. < Agreement>: - In FR2: the RAN4 Rel-18 related LTM requirements are not applicable if L1 measurement of LTM candidate cell is not supported or configured.

Excerpt from R2-2406227 (RAN4 LS to RAN2)

45-1; Intra-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure. 45-1a; Inter-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure. Question 1: Are the above intra-frequency and inter-frequency L1 measurement and reporting features (45-1 and 45-1a) prerequisites to support intra-frequency and inter-frequency LTM, respectively? Question 2: The above features, 45-1 and 45-1a, from RAN1 and related RAN4 features (39-1, 39-2, 39-3-1, 39-3-2, 39-3-3, 39-3-4, 39-3-5, 39-3-6) are defined per BC for both intra-frequency and inter-frequency measurements. RAN2 would like check with RAN1/4 for which BC (e.g. BC of current serving cells, BC including current serving cells and cell to be measured or something else) these capabilities are to be considered for L1 intra-frequency and inter-frequency LTM measurements?. From RAN4’s perspective, the following consensus was reached: Reply to the Question 1: For whether intra-frequency and inter-frequency L1 measurements and reporting are prerequisites to support intra-frequency and inter-frequency LTM, RAN4 analysed RAN4 requirements and achieved following agreements - For FR1, RAN4 Rel-18 related LTM requirements are applicable to the case without L1 measurement and reporting of LTM candidate cells. - For FR2, RAN4 Rel-18 related LTM requirements are not applicable if L1 measurements for LTM candidate cells are not supported by the UE, or not configured by the network.

Excerpt from R2-2407606 (RAN2 LS to RAN3)

In the late phase of Rel-18 LTM definition, it was decided, that LTM can be triggered based on Layer 3 measurements by the UE. Consequently, Layer 3 measurements are delivered via RRC to the CU and the CU needs to request the DU to send MAC CE to trigger LTM execution. During RAN2 discussion, it was recognised, that there is no message over F1 which would be sent from CU to DU requesting to trigger MAC CE for the case that the LTM mobility is based on L3 measurements. It was also recognised that such a message would be needed from Release 18 onwards. 2. Actions: RAN2 kindly asks RAN3 to introduce F1 support for the case where the CU needs to request the DU to send MAC CE to trigger LTM execution based on UE Layer 3 measurements.

# 3	F1AP Implications

As discussed in the LS from RAN2 to RAN3, in case of disaggregated gNB architecture, the gNB-DU is unaware of L3 measurements. Therefore, in absence of L1 measurements from the UE, there are no means for the gNB-DU to trigger LTM to fulfil lthe requirements defined by RAN4. Therefore, signaling over F1AP is needed. We see impact on two aspects:

L3 based triggered LTM cell change

The gNB-DU requires a command, or at minimum assistance information from the gNB-CU over F1AP in order to trigger LTM in absence of L1 measurements from the UE.

Approach 1: The gNB-CU is is in charge of ordering the gNB-DU to issue the LTM command to the UE to a given cell without further evaluation at the gNB-DU. In this approach a single Cell ID is indicated.

  - The gNB-CU would at minimum, signal the Cell ID to which mobility is to take place. Likewise, given that in RAN1 the TCI State ID is mandatory in the MAC CE command, the gNB-DU will need to indicate one to the UE. This requirement is applicable regardless of whether LTM is RACH-less or RACH-based. Hence, the gNB-CU should also indicate an appropriate beam (TCI state(s)) or assistance information like SSB index which can used to derive TCI state(s) to the gNB-DU.

Approach 2: The gNB-CU provides assistance information to the gNB-DU, which it takes into consideration and ultimately determine the mobility. In this approach, multiple Cell IDs may be indicated with further information, allowing for the gNB-DU flexibility to chose the most appropriate cell. Further, this information could also complement cases where L1 measurements exist.

  - The gNB-CU would provide one or more Cell IDs, their related beam information, as well as other information such as RSRP or RSRQ measurements.

In our view, given we are considering changes for frozen Rel-18, approach 1 seems more appropriate.

Proposal 1: The gNB-CU indicates over F1 the Cell ID and target TCI State(s) or SSB index for LTM cell change to the gNB-DU.

L3 based early RACH-based TA acquisition

In the RAN2 LS, there is request only for change so handle triggering LTM in absence of L1 measurements. However, the early RACH-based TA acquisition procedure at the gNB-DU is based on L1 measurements where the gNB-DU selects a candidate cell and an associated SSB index and provide these parameters in a PDCCH order to the UE to trigger a RACH premable transmisison towards a candidate cell. Therefore, in our view, it is natural to also take into account L3 based early TA acquisition. In this case, the gNB-CU should indicate the gNB-DU toward which cell and beam (SSB index or TCI State ID) is to be aquired.

Proposal 2: The gNB-CU indicates over F1 the Cell ID(s) and SSB index for early TA acquisition to the gNB-DU.

F1AP Message

With respect to which message to use for issuing the LTM command or the early TA acquisition to the UE, it is possible to introduce a new message, or alternatively to reuse the UE Context Modification procedure.

Option 1 – Reuse UE Context Modification procedure: Reusing the existing modification procedure has the benefit of less specification changes, as well as having a response to the gNB-CU confirming that the requested action was triggered. However, at the expense of further signaling and overloading an already quite large procedure used for many general actions, which likely should not be carried out with the LTM actions simultaneously.

Option 2 – Introduce a new procedure: A new procedure would allow for a very clear indication of actions that are specific to LTM, and further for somewhat particular cases (scuh as LTM configured without L1 measurements). This would also allow to clearly separate these actions from other existing context modifying actions. However, at expense of further specification changes.

In our view, given that two separate LTM actions are needed (L3 based LTM, and L3 based early TA acquisition), introducing a new procedure would be preferable.

Proposal 3: Introduce a new Class 2 procedure for triggering L3 based LTM cell change and L3 based early TA acquisition.

# 4	Conclusions

Proposal 1: The gNB-CU indicates over F1 the Cell ID and target TCI State(s) or SSB index for LTM cell change to the gNB-DU.

Proposal 2: The gNB-CU indicates over F1 the Cell ID(s) and target TCI State(s) or SSB index for early TA acquisition to the gNB-DU.

Proposal 3: Introduce a new Class 2 procedure for triggering L3 based LTM cell change and L3 based early TA acquisition

# References

- R4-2410302

- R2-2406227

- R2-2407606